3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2203034

Electronic Meeting, 17 – 25 January 2022

Agenda Item:	8.13.3

Source: 	ZTE

Title: 	WF on Tx RF requirement for SAN type 1-O

Document for:	Approval

# 1.	Background

Contributions on system parameters for SAN type 1-O are summarized as following:

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200169 | CATT | Tx requirement; Classification; Coverage range; Number of; comments. FR1; FR2; conformance directions. Radiated transmit power; Directional; OTA peak directions set; OTA peak directions set; 5; The requirement in 38.104 can be used as the starting point.. OTA BS output power; TRP; See annex I; The TRP value can be determined once the NTN class in [2] is determined.. OTA output power dynamics; Directional; OTA peak directions set; OTA peak directions set; 1; This requirement can be reused from 38.104. OTA transmitter OFF power; Co-location; See clause 4.12; N/A; See clause 4.12; This requirement is not needed.. Directional; N/A; OTA peak directions set (Note 2); 1; This requirement is not needed.. OTA transient period; Co-location; See clause 4.12; N/A; See clause 4.12; This requirement is not needed.. Directional; N/A; OTA peak directions set (Note 2); 1; This requirement is not needed.. OTA modulation quality; Directional; OTA coverage range; OTA coverage range; 5; This requirement for QPSK and 16QAM can be reused from 38.104. OTA frequency error; Directional; OTA coverage range; OTA coverage range; 1; This requirement can be reused from 38.104. OTA time alignment error; Directional; OTA coverage range; OTA coverage range; 1; This requirement is not needed given CA and MIMO feature are not supported in Rel-17.. OTA occupied bandwidth; Directional; OTA coverage range; OTA coverage range; 1; This requirement can be reused from 38.104. OTA ACLR; TRP; N/A; N/A; See annex I; 38dB as discussed in [2]. OTA operating band unwanted emission; TRP; N/A; N/A; See annex I; FFS. OTA transmitter spurious emission; General requirement; TRP; N/A; N/A; See annex I; This requirement is needed. Compliance to ERC 74-01 Space stations’ limits, i.e. ITU Radio Regulations Annex 3.. Protection of the BS receiver of own or different BS; Co-location; See clause 4.12; N/A; See clause 4.12; This requirement can be reused from 38.104.. Additional spurious emissions; TRP; N/A; N/A; See annex I; This requirement needs to be studied as discussed for conducted part in [2]. Co-location with other base stations; Co-location; See clause 4.12; N/A; See clause 4.12; This requirement is not needed.. OTA transmitter intermodulation; Co-location; See clause 4.12; N/A; See clause 4.12; This requirement is not needed.. |
| R4-2201468 | ZTE | Proposal 1: for radiated transmit power and OTA BS output power of satellite gNB, it’s up to vendors’ declaration. Proposal 2: the scaling factor NTXU,counted = min(NTXU,active , 8×Ncells) is not applicable for satellite access node; Proposal 3: OTA transit OFF power and transition period requirement, OTA timing alignment requirement, Tx intermodulation, co-location spurious emission requirements, protection of own receiver requirement is not applicable for satellite access node. Proposal 4: for OTA OTA ACLR requirements and unwanted emission requirement, it should follow conducted requirements and scaling factor for OTA unwanted emission requirement is not applicable. Proposal 5: to support the NTN BS type 1-O with following OTA peak directions set and OTA coverage range declaration. |
| R4-2201816 | Huawei | Proposal 1: Reuse the Radiated transmit power (EIRP) accuracy requirement for Normal (±2.2 dB) and for Extreme (±2.7 dB) test conditions from NR BS type 1-O. Proposal 2: Approve the baseline structure of the SAN requirement for Radiated transmit power (EIRP) as in Table 1. Proposal 3: Reuse the OTA output power (TRP) accuracy (±2 dB) requirement for Normal test conditions from NR BS type 1-O. Proposal 4: Approve the baseline structure of the SAN requirement for OTA output power (TRP) as in Table 2. Proposal 5: reuse the conducted requirements for RE power control dynamic range and Total power dynamic range. Proposal 6: Approve the baseline structure of the SAN requirement for OTA Output power dynamics as in Table 3. Proposal 7: Not to define OTA TX ON/OFF requirement for NTN SAN type 1-O. Proposal 8: reuse conducted requirement for the OTA frequency error. Proposal 9: no OTA TAE to be defined for NTN SAN. Proposal 10: Include OTA EVM requirement for 64QAM in the NTN SAN specification (in addition to the EVM requirements for QPSK and 16QAM). Proposal 11: Approve the baseline structure of the SAN requirement for OTA EVM as in Table 4. Proposal 12: Not to define OTA TX IMD requirement for NTN SAN type 1-O. |
| R4-2201469 | ZTE | Proposal 1: to support the NTN BS type 1-O with following OSDD declaration and OTA REFSENS RoAoA declaration. Rx requirement; Classification; Applicability levels; Coverage range; Number of. FR1; FR1; conformance directions. OTA sensitivity; Directional; Minimum EIS; OSDD; 5. OTA reference sensitivity; Directional; OTA REFSENS; OTA REFSENS RoAoA; 5. OTA Dynamic range; Directional; OTA REFSENS; OTA REFSENS RoAoA; 1. OTA adjacent channel selectivity; Directional; minSENS; minSENS RoAoA; 1. OTA in-band blocking; Directional; OTA REFSENS and minSENS; OTA REFSENS RoAoA and minSENS RoAoA; 5. OTA out-of-band blocking; General requirement; Directional; minSENS; minSENS RoAoA; 1. OTA receiver spurious emissions; TRP; See clause 7.7; N/A; See annex I. OTA in-channel selectivity; Directional; minSENS; minSENS RoAoA; 1. NOTE 1: Directional requirement does not imply one compliance direction only. The directional requirement applies to a single direction at a time. NOTE 2: The compliance direction for co-location blocking is applicable for the wanted signal only but not the interfering signal.. |
| R4-2201817 | Huawei | Proposal 1: agree on the declared OTA sensitivity requirement (related text for the TP included in section 2.1.1). Proposal 2: for 1-O, define only OTA sensitivity requirement (and not OTA reference sensitivity level). Proposal 3: set of related manufacturer declarations to be reused from NR BS, as baseline (this work belongs to the Performance part of the WI). Proposal 4: agree on the requirement principles reuse from NR BS specification. Proposal 5: OTA RX IMD requirement is not needed for NTN SAN. Proposal 6: Not to define OTA ICS requirement for NTN SAN. Proposal 7: Approve the baseline structure of the SAN requirement for OTA sensitivity, as in Annex A. |
| R4-2201320 | Ericsson | Observation1: OTA radiated transmit power requirement shall be specified for NTN satellite access node type 1-H. Observation2: Satellite access node OTA Tx requirements could be specified following a similar approach than for TN OTA Tx requirements. Proposal: Specify satellite access node OTA x requirements for type 1-H and 1-O following a similar approach as for TN OTA Tx requirements, as proposed in Table 2. Requirement. Radiated transmit power; This requirement is based on manufacturer declaration of an EIRP level for each supported beam.. OTA base station output power; This requirement checks the accuracy (+/-2dB) of the declared TRP output power. OTA output power dynamics. OTA RE power control dynamic range; Same limits than 1-H. OTA total power dynamic range; Same limits than 1-H. OTA transmit ON/OFF power; NA, only for TDD operations. OTA transmitted signal quality. OTA frequency error; Same limits than 1-H. OTA modulation quality; Same limits than 1-H. OTA time alignment error; Same limits than 1-H. OTA occupied bandwidth; Same limits than 1-H. OTA ACLR; Same limits than 1-H, the X scaling should not be relevant here.. OTA out-of-band emission; Same limits than 1-H, the X scaling should not be relevant here.. OTA transmitter spurious emission; Same limits than 1-H. OTA transmitter intermodulation; Not needed for 1-H and 1-O. |
| R4-2201818 | Huawei | Proposal 1: for SAN spec, consider manufacture declaration on the class of the SAN (to choose from GEO SAN, LEO600 SAN, or LEO1200 SAN). Proposal 2: set of related manufacturer declarations to be reused from NR BS, as baseline (this work belongs to the Performance part of the WI), with the structure provided in Table 1 to be approved as the starting point. Proposal 3: AAS-specific definitions to be reused from related NR specification (e.g. TAB connectors), as baseline for NTN specification drafting. Proposal 4: introduce SAN abbreviation for the Satellite Access Node. Proposal 5: define two SAN types: SAN type 1-H: Satellite Access Node operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB SAN type 1-O: Satellite Access Node operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB Proposal 6: for the TS 38.108 Core requirements drafting purposes, consider both Normal as well as Extreme testing conditions. Proposal 7: Recommend to TSG RAN to split the TS 38.181 specification into two parts, for conducted conformance testing (TS 38.181-1), and for radiated conformance testing (TS 38.181-2). Proposal 8: Reuse the output power accuracy requirement for Normal and for Extreme test conditions from NR BS. Proposal 9: Approve the baseline structure of the SAN type 1-H output power requirement from Table 2. Proposal 10: Approve the baseline structure of the SAN type 1-H requirement for RE power control dynamic range as in Table 3. Proposal 11: Approve the baseline structure of the SAN type 1-H requirement for Total power dynamic range as in Table 4. Proposal 12: Approve the baseline structure of the SAN type 1-H requirement for frequency error requirement as in Table 5. Proposal 13: Include EVM requirement for 64QAM in the NTN SAN specification (in addition to the EVM requirements for QPSK and 16QAM) Proposal 14: Approve the baseline structure of the SAN type 1-H requirement for EVM requirement as in Table 6. Proposal 15: no MIMO TAE to be defined for NTN SAN. Proposal 16: Approve the baseline structure of the SAN type 1-H requirement for Protection of the own BS receiver (Tx spur) requirement as in Table 7. Proposal 17: re-consider the intra system transmitter intermodulation requirement for NTN SAN. |
| R4-2201321 | Ericsson | Observation1: OTA sensitivity requirement shall be specified for NTN satellite access node type 1-H. Observation2: Satellite access node OTA Rx requirements could be specified following a similar approach than for TN OTA Rx requirements. Proposal: Specify satellite access node OTA Rx requirements for type 1-H and 1-O following a similar approach as for TN OTA Rx requirements, as proposed in Table 2. Requirement. OTA sensitivity; Based on manufacturer declaration and the declared directions.. OTA reference sensitivity level; Same limits than 1-H but adjusted with NTN ΔOTAREFSENS. OTA dynamic range; Same limits than 1-H but adjusted with NTN ΔOTAREFSENS. OTA ACS; Similar to 1-H but based on NTN EISminSENS adjusted with NTN ΔminSENS. OTA blocking; Similar to 1-H but adjusted with NTN ΔOTAREFSENS for intererfer as well. Based on NTN EISminSENS adjusted with NTN ΔminSENS for the interferer. OTA out-of-band blocking; Similar to 1-H but based on NTN EISminSENS. OTA receiver spurious emission; Same limits than 1-H. OTA receiver intermodulation; Not applicable for NTN 1-H and 1-O. OTA in-channel selectivity; Similar to 1-H adjusted with NTN ΔminSENS for both wanted signal and interefer values. |

- Way forward

# Agreement:

X scaling factors is not needed for OTA out-of-band emission and OTA transmitter spurious emission since MIMO is not supported on SAN.

| Requirement | Proposal |
| --- | --- |
| Radiated transmit power | The requirement in 38.104 can be used as the starting point, e.g. it’s up to manufacturer declaration |
| OTA base station output power | This requirement checks the accuracy (+/-2dB) of the declared TRP output power |
| OTA output power dynamics |  |
| OTA RE power control dynamic range | Same limits than 1-H |
| OTA total power dynamic range | Same limits than 1-H |
| OTA transmit ON/OFF power | NA, only for TDD operations |
| OTA transmitted signal quality |  |
| OTA frequency error | Same limits than 1-H |
| OTA modulation quality | Same limits than 1-H |
| OTA occupied bandwidth | Same limits than 1-H |
| OTA ACLR | Same limits than 1-H |
| OTA out-of-band emission | Same limits than 1-H |
| OTA transmitter spurious emission | Same limits than 1-H |
| OTA transmitter intermodulation | FFS Depends on conclusion of issue 1-5-2 |