3GPP TSG-RAN WG4 Meeting # 101bis-e 													R4-2203115

Electronic Meeting, 17 – 25 January, 2022

Agenda item:			6.16.4

Source:	Moderator (Nokia)

Title:	Email discussion summary for [101bis-e][309] NR_exto71GHz_BSRF

Document for:	Information

# Introduction

This email discussion summary covers BS RF requirements for extending NR operation to 71 GHz. Based on the input contributions the discussion is split into two major topics, Tx requirements and Rx requirements, within which individual requirements are discussed in various sub-topics. Generally, proposals and requirements having most dependency have been grouped together.

The template has been adapted to include comment section separately for each issue to facilitate discussion.

# Topic #1: Tx requirements

This topic covers Tx requirements.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200136 | CATT | Proposal 1: For MIMO TAE，to specify [10ns] for 906kHz SCS TAE and a value in range of [10ns-35ns] for 480kHz SCS TAE. Proposal 2: To specifyΔfOBUE = 3500 MHz for FDL,high – FDL,low > 4000 MHz for 52.6-71GHz. Proposal 3: To specify the following Fstep,X for spurious emissions. Operating band; Fstep,1; Fstep,2; Fstep,3; Fstep,4; Fstep,5; Fstep,6. (GHz); (GHz); (GHz); (GHz); (GHz); (GHz). n263; 18; 43; 53.5; 74.5; 85; 127. |
| R4-2200411 | Nokia, Nokia Shanghai Bell | Proposal 1: The EVM window length for NR operation in 52.6 – 71 GHz range should be defined as 50% of the normal CP length and 85.9% of the extended CP length. Proposal 2: The MIMO time alignment error requirements for NR operation in 52.6 – 71 GHz range should be defined as 40 ns for 480 kHz SCS and 20ns for 960 kHz SCS. Proposal 3: The intra-band contiguous CA time alignment error requirements for NR operation in 52.6 – 71 GHz range should be specified as 80 ns for 480 kHz SCS and 40 ns for 960 kHz SCS. Proposal 4: The intra-band non-contiguous CA time alignment error requirements for NR operation in 52.6 – 71 GHz range should be specified as 160 ns for 480 kHz SCS and 80 ns for 960 kHz SCS. Proposal 5: The ACLR requirement for NR operation in 52.6 – 71 GHz range should be specified as 24 dB. Proposal 6: The OBUE limits for NR operation in 52.6 – 71 GHz range should be defined as: Table 1: OBUE limits (Category A) applicable in the frequency range 52.6 – 71 GHz Frequency offset of measurement filter -3B point, f; Frequency offset of measurement filter centre frequency, f_offset; Limit; Measurement bandwidth. 0 MHz f < 0.1*BWcontiguous; 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz; Min(-5 dBm, Max(Prated,t,TRP – 31 dB, -12 dBm)); 1 MHz. 0.1*BWcontiguous f < fmax; 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax; Min(-13 dBm, Max(Prated,t,TRP – 39 dB, -20 dBm)); 1 MHz. NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap.. Table 2: OBUE limits (Category B) applicable in the frequency range 52.6 – 71 GHz Frequency offset of measurement filter -3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Limit; Measurement bandwidth. 0 MHz f < 0.1*BWcontiguous; 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz; Min(-5 dBm, Max(Prated,t,TRP – 31 dB, -12 dBm)); 1 MHz. 0.1*BWcontiguous f < fB; 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz; Min(-13 dBm, Max(Prated,t,TRP – 39 dB, -20 dBm)); 1 MHz. fB f < fmax; fB +5 MHz f_offset < f_ offsetmax; Min(-5 dBm, Max(Prated,t,TRP – 29 dB, -10 dBm)); 10 MHz. NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. NOTE 2: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz.. Proposal 7: ΔfOBUE shall be defined as 3500 MHz for NR operation in 52.6 – 71 GHz range. Proposal 8: The Tx spurious emission limits for NR operation in 52.6 – 71 GHz range should be defined as: Moderator comment: tables not copy pasted here, both cat A and cat B emission limits unchanged, only new step frequencies specified as below Table 5: Step frequencies for defining the BS radiated Tx spurious emission limits in FR2 (Category B) Operating band; Fstep,1 (GHz); Fstep,2 (GHz); Fstep,3 (GHz) (Note 2); Fstep,4 (GHz) (Note 2); Fstep,5 (GHz); Fstep,6 (GHz). n257; 18; 23.5; 25; 31; 32.5; 41.5. n258; 18; 21; 22.75; 29; 30.75; 40.5. n259; 23.5; 35.5; 38; 45; 47.5; 59.5. [Licensed band]; [46]; [61]; [62.5]; [74.5]; [76]; [91]. Unlicensed band; 18; 43; 53.5; 74.5; 85; 127. NOTE 1: Fstep,X are based on ERC Recommendation 74-01 [19], Annex 2. NOTE 2: Fstep,3 and Fstep,4 are aligned with the values for ΔfOBUE in Table 9.7.1-1.. |
| R4-2200414 | Nokia, Nokia Shanghai Bell | Moderator: This is draft CR, included in section 1.2.5 |
| R4-2200943 | Ericsson | Proposal 1: Taking to account both co-existence studies in TR 38.803, existing emission masks and feasibility analysis of power amplifiers, the BS ACLR shall be set to 21 dB. Proposal 2: For ACLR, add new table entries for new carrier bandwidths, ACLR limit and FR applicability note in TS 38.104, Table 9.7.3.3-1. Proposal 3: For unwanted emission set fOBUE to 3000 MHz. Proposal 4: For OBUE define requirement limits for Category A according to Table 2.2.3-8. Table 2.2.3-8: Category A OBUE limits applicable in the frequency range 52.6 – 71 GHz Frequency offset of measurement filter -3 dB point, f (MHz); Frequency offset of measurement filter centre frequency, f_offset (MHz); Limit (dBm); Measurement bandwidth (MHz). 0 f < 0.1*BWcontiguous; 0.5 f_offset < 0.1* BWcontiguous +0.5; Min(-5, Max(Prated,t,TRP – 31, -12)); 1. 0.1*BWcontiguous f < fmax; 0.1* BWcontiguous +0.5 f_offset < f_ offsetmax; Min(-13, Max(Prated,t,TRP – 39, -20)); 1. Proposal 5: For OBUE define requirement limits for Category B according to Table 2.2.3-9. Table 2.2.3-9: Category B OBUE limits applicable in the frequency range 52.6 – 71 GHz Frequency offset of measurement filter -3 dB point, f (MHz); Frequency offset of measurement filter centre frequency, f_offset (MHz); Limit (dBm); Measurement bandwidth (MHz). 0 f < 0.1*BWcontiguous; 0.5 f_offset < 0.1* BWcontiguous +0.5; Min(-5, Max(Prated,t,TRP – 31, -12)); 1. 0.1*BWcontiguous f < fB; 0.1* BWcontiguous +0.5 f_offset < fB +0.5; Min(-13, Max(Prated,t,TRP – 39, -20)); 1. fB f < fmax; fB +5 f_offset < f_ offsetmax; Min(-5, Max(Prated,t,TRP – 29, -10)); 10. Proposal 6: For spurious emission, re-use FR2 Category A requirement limits. Proposal 7: For spurious emission, adopt Fstep,X values as listed for band n263 in Table 2.2.4-4 for Category B requirement limits. Table 2.2.4-4: Step frequencies for defining the BS radiated transmitter spurious emission limits in FR2 (Category B) Operating band; Fstep,1 (GHz); Fstep,2 (GHz); Fstep,3 (GHz); Fstep,4 (GHz); Fstep,5 (GHz); Fstep,6 (GHz). n257; 18; 23.5; 25; 31; 32.5; 41.5. n258; 18; 21; 22.75; 29; 30.75; 40.5. n259; 23.5; 35.5; 38; 45; 47.5; 59.5. n26x (licensed operation); 46; 61; 63; 74; 76; 91. n263; 18; 43; 54; 74; 84; 127. Observation 1: TAE < 33 ns for SCS = 960 kHz, based on theoretical analysis. Observation 2: Simulations show good performance for TAE = 65 ns @ SCS = 480 kHz and TAE = 32.5 ns @ SCS = 960 kHz. Proposal 8: MIMO TAE = 32.5 ns for SCS = 960 kHz and TAE = 65 ns for SCS = 480 kHz. Proposal 9: For CA set TAE = 65 ns for SCS = 480 kHz and TAE = 32.5 ns for SCS = 960 kHz. Proposal 10: A reference architecture for Non-contiguous Intra Band Carrier aggregation, is a UE with separate receivers for each component carrier Proposal 11: TAE = 260 ns for non-contiguous carrier aggregation. Proposal 12: TAE = 3 µs for inter-band CA. Proposal 13: For EVM, add new tables with information only agreed values and remaining FFS 480 kHz and 960 kHz as described in Table 2.6-3 and Table 2.6-4. Table 2.6-3: EVM window length for normal CP, FR2-2, 480 kHz SCS Channel bandwidth (MHz); FFT size; CP length in FFT samples; EVM window length W; Ratio of W to total CP length (%). 400; 4096; 72; FFS; FFS. 800; 4096; 144; FFS; FFS. 1600; 4096; 288; FFS; FFS. Table 2.6-4: EVM window length for normal CP, FR2-2, 960 kHz SCS Channel bandwidth (MHz); FFT size; CP length in FFT samples; EVM window length W; Ratio of W to total CP length (%). 400; 4096; 36; FFS; FFS. 800; 4096; 72; FFS; FFS. 1600; 4096; 144; FFS; FFS. 2000; 4096; 180; FFS; FFS. |
| R4-2200845 | Ericsson | Moderator: This is draft CR, included in section 1.2.5 |
| R4-2201456 | ZTE Corporation | Proposal 1: propose MIMO TAE requirement as 16.25ns for 960kHz and 32.5ns for 480kHz; Proposal 2: postpone CA TAE requirement to Rel-18 timeframe; Proposal 3: to reuse the existing requirement in TR 38.803 for 52.6-71GHz with round up to nearest integer. Proposal 4: fine to reuse the 50% EVM window length for FR2-2. |
| R4-2201821 | Huawei | Proposal 1: Set the MIMO TAE requirement as Option 1 or Option 2 below, with slight preference to Option 1. Option 1: For SCS = 480 kHz: MIMO TAE = 65 ns For SCS = 960 kHz: MIMO TAE = 32.5 ns Option 2: For SCS = 480 kHz: MIMO TAE = 32.5 ns For SCS = 960 kHz: MIMO TAE = 32.5 ns Proposal 2: adopt 65 ns TAE for intra-band contiguous CA. Proposal 3: if the non-contiguous CA is confirmed to be in Rel-17 scope, reuse 260ns TAE requirement for intra-band non-contiguous CA. Proposal 4: reuse 3us TAE requirement for inter-band CA. Proposal: Define ΔfOBUE values as [3.5] GHz. |

## Open issues summary and comment collection

Please note it is possible and often necessary to select multiple options to create coherent agreements/requirements.

### Sub-topic 1-1 TAE

Background: Following was agreed in previous meeting:

- Agree same TAE applies for 120 kHz SCS in FR2-2 as in FR2-1. Agree that only highest supported SCS is tested.

- For MIMO case: “Scale the value under 120kHz for larger SCS cases” as starting point.

- For CA cases: FFS.

Issue 1-1-1:  Interband CA TAE

- Proposals

  - Option 1: reuse 3us TAE requirement for inter-band CA (Huawei, R4-2201821)

  - Option 2: TBA

- Recommended WF

  - Option 1

Issue 1-1-2:  Intraband contiguous CA TAE

- Proposals

  - Option 1: 65ns  (Huawei, R4-2201821)

  - Option 2: Postpone to Rel-18 (ZTE, R4-2201456)

  - Option 3: 65 ns for 480 kHz SCS, 32.5 ns for 960 kHz SCS (Ericsson, R4-2200943)

  - Option 4: 80 ns for 480 kHz SCS, 40 ns for 960 kHz SCS (Nokia, R4-2200411)

- Recommended WF

  - TBA

Issue 1-1-3:  Intraband non-contiguous CA TAE

- Proposals

  - Option 1: Postpone to Rel-18 (ZTE, R4-2201456)

  - Option 2: 260 ns (Huawei R4-2201821, Ericsson, R4-2200943)

    - Ericsson proposal based on reference architecture with separate receiver for each component carrier

  - Option 3: 160 ns for 480 kHz SCS, 90 ns for 960 kHz SCS (Nokia, R4-2200411)

- Recommended WF

  - TBA

Issue 1-1-4:  MIMO TAE

- Proposals for

  - Option 1: 32.5 ns for 480 kHz CSC, 16.25 ns for 960 kHz SCS (ZTE, R4-2201456)

  - Option 2: 65 ns for 480 kHz SCS, 32.5 ns for 960 kHz SCS (Ericsson, R4-2200943, Huawei, R4-2201821)

  - Option 3: 40 ns for 480 kHz SCS, 20 ns for 960 kHz SCS (Nokia, R4-2200411)

  - Option 4: [10-35ns] for 480 kHz SCS, [10 ns] for 960 kHz SCS (CATT, R4-2200136)

  - Option 5: 32.5 ns for both 480 and 960 kHz SCS (Huawei, R4-2201821)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Nokia | Issue 1-1-1: Interband CA TAE Support option 1. Issue 1-1-2: Intraband contiguous CA TAE Propose option 4; ok with option 1 or 3; disagree with option 2 as this will also postpone UE MRTD. Issue 1-1-3: Intraband non-contiguous CA TAE Propose option 3; ok with option 2; disagree with option 1 as this will also postpone UE MRTD. Issue 1-1-4: MIMO TAE Propose option 3; ok with option 2 or 5; option 1 or 4 are too stringent as minimum requirements, implementation can aim for smaller TAE when needed. |
| Ericsson | Issue 1-1-1: Interband CA TAE Option1: reuse 3us TAE requirement for inter-band CA Issue 1-1-2: Intraband contiguous CA TAE Option 3: 65 ns for 480 kHz SCS, 32.5 ns for 960 kHz SCS. We prefer to make room for 40 ns radio channel dispersion. This was also Ericsson input to TS 38.808. (Option 4: with 40 ns for SCS = 960 kHz is 40/73 = 55% of CP and will not fit 40 ns of radio channel dispersion, same comment to option 5). Issue 1-1-3: Intraband non-contiguous CA TAE Option 2: 260 ns, based on reference architecture with separate receiver for each component carrier. This is a fair assumption, since the FR2-2 spans from 52.6 to 71 GHz. Issue 1-1-4: MIMO TAE Option 2: 65 ns for 480 kHz SCS, 32.5 ns for 960 kHz SCS, based in our simulation input. |
| Qualcomm | Issue 1-1-1: Interband CA TAE We are ok with option 1. Issue 1-1-2: Intraband contiguous CA TAE We are fine with options 3 or 4. Issue 1-1-3: Intraband non-contiguous CA TAE We are fine with options 2 or 3. Issue 1-1-4: MIMO TAE We are fine with option 2. |
| Huawei | Issue 1-1-1: Interband CA TAE Option 1. Issue 1-1-2: Intraband contiguous CA TAE We were initially proposing option 1. With 3 companies proposing values, we suggest to drop option 2. To make progress, maybe we can limit the TAE values to a range of values (e.g. 32.5 – 80 ns) during this meeting, and leave the final decision till next meeting. Issue 1-1-3: Intraband non-contiguous CA TAE Option 2. While non-cont CA is still not confirmed for Rel-17, we would be also fine to drop this requirement (UE MRTD shall be ok to reuse the intraband cont CA) Issue 1-1-4: MIMO TAE Option 2. Options 1 and 4 are not considered to account for the real implementations with all the impairments consideration. Simulation results showing the thp degradation is not the only factor to consider here. |
| CATT | Issue 1-1-1: Interband CA TAE Ok with the option 1. Issue 1-1-2: Intraband contiguous CA TAE Option 3 if no problem is found. Issue 1-1-3: Intraband non-contiguous CA TAE For FR2, UE separate receiver for NC CA may not be always valid, further confirmation from UE vendors may be needed. So we think scaling according to SCS may be the proper way. Issue 1-1-4: MIMO TAE Option 4 is from our side, we’re also ok with option 1. Not sure if other options can guarantee the system performance. We understand the intention of considering implementation capability, but if the implementation capability can’t meet the system performance, then either improve the implementation or to declare not support this feature. Relaxing the requirement to a value which can’t guarantee the system performance is not a correct way. |
| Samsung | Issue 1-1-1: Fine with option 1. Issue 1-1-2: Fine to define this requirement since the intra-band CA is agreed to be defined according to system parameter discussion. Issue 1-1-3: Option 2 is preferred if TAE for non-contiguous intra-band CA is agreed to be defined in BS spec. Issue 1-1-4: Option 2 is preferred. |
| NEC | Issue 1-1-1: Interband CA TAE We support option 1. Issue 1-1-2: Intraband contiguous CA TAE We are fine with options 3 or 4. Issue 1-1-3: Intraband non-contiguous CA TAE We support options 2. Issue 1-1-4: MIMO TAE We support option 2. |
| ZTE | Issue 1-1-1: Interband CA TAE We are ok with option 1, it was agreed that we don’t have band combination within FR2-2 and FR2-1 and FR2-2 at least, in other words, this requirement is not useful in Rel-17 at least. Issue 1-1-2: Intraband contiguous CA TAE We need more discussions on that, large than MIMO TAE requirement should be reasonable due to increasing carrier numbers; LTE CA introduced in Rel-10, 5 carrier are assumed if I remember correctly, then 65*4 =260ns defined. Here for FR2, we need to have more discussion on its potential RF architecture together. Issue 1-1-3: Intraband non-contiguous CA TAE To avoid the discussion due to CBM or IBM discussion from UE side, our suggestion is to postpone the discussion until there are clear market demand. Issue 1-1-4: MIMO TAE Option 1 and option 3 should are fine for us based on the simulation results provided so far. |

### Sub-topic 1-2 EVM

Some EVM related proposal were made related to EVM table format. Moderator recommends comments in this area to be provided directly to the draft CR.

Issue 1-2: EVM window length

Background: Agreed in previous meeting

- [50%] EVM window length is starting point for normal CP length. This is to be confirmed once spectral utilization and out-of-band emissions are agreed.

- Add new column to Table 9.6.1.2-1 to differentiate EVM capability within FR2 range.

- Proposals

  - Option 1: The EVM window length for NR operation in 52.6 – 71 GHz range should be defined as 50% of the normal CP length and 85.9% of the extended CP length. (Nokia, R4-2200411)

  - Option 2: reuse the 50% EVM window length for FR2-2. (ZTE, R4-2201456)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Nokia | Propose option 1; option 2 is incomplete as EVM window length for the extended CP length also need to be defined. |
| Ericsson | At this point in time, we prefer to re-use 50% EVM window length for 120 kHz SCS (re-use from FR2-1). For 480 kHz and 960 kHz we need to further study the detail of EVM window length. Let’s first collect parameters such as FFT length, SU, etc. Then we can decide the window length properly. We have proposals for that in our contribution R4-2200843, but it seems to be lost here. Therefore, we can’t support any of given options. We can suggest to consider Option 3: For 120 kHz agree to use 50% window length. In addition, extended CP is not supported for FR2-2, which means that option 1 is not really relevant here. According to TS 38.211 running CR endorsed after RAN1#107-e only extended CP is for 60 kHz SCS, see below the copied table for convenience. Table 4.2-1: Supported transmission numerologies. Cyclic prefix. 0; 15; Normal. 1; 30; Normal. 2; 60; Normal, Extended. 3; 120; Normal. 4; 240; Normal. 5; 480; Normal. 6; 960; Normal. |
| Nokia | We have confirmed with our RAN1 delegates that extended CP is not considered for >60 kHz SCS in FR2-2, so we support option 2. |
| Qualcomm | We are fine with option 2. |
| CATT | Ok with option 2. And it can be revisited when SU is agreed. |
| NEC | We are fine with option 2. |
| ZTE | Agree with CATT |
| Ericsson | We can agree to 50% window for 120 kHz SCS. For 480 and 960 kHz we still need to consider test time aspects previously mentioned by test equipment vendors and SU discussion. We suggest to come back next meeting. |

### Sub-topic 1-3 Emissions: OBUE

Background: Following was agreed in previous meeting:

- [Re-use FR2-1 OBUE], deltafOBUE needs more discussion but tentatively in range [3 – 4 GHz]. Re-use of FR2-1 includes using TRP as metric.

- No separate requirements for licensed and unlicensed at this point of time.

- For ACLR wait for coex study conclusion in thread 130.

- Make decision on deltafOBUE on Jan 2022 RAN4 meeting.

- Align the value for deltafOBUE and deltafOOB

Issue 1-3-1: deltafOBUE

Proposals

  - Option 1: ΔfOBUE = 3500 MHz for FDL,high – FDL,low > 4000 MHz for 52.6-71GHz (CATT R4-2200136, Nokia R4-2200411, Huawei R4-2201821)

  - Option 2: ΔfOBUE = 3000 MHz (Ericsson, R4-2200943)

- Recommended WF

  - Option 1

Issue 1-3-2: OBUE limits

Proposals

  - Option 1: Tables 1 and 2 below (Nokia R4-2200413, Ericsson R4-2200943)

Table 1: OBUE limits (Category A) applicable in the frequency range 52.6 – 71 GHz

| Frequency offset of measurement filter -3B point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-5 dBm, Max(Prated,t,TRP – 31 dB, -12 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fmax | 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax | Min(-13 dBm, Max(Prated,t,TRP – 39 dB, -20 dBm)) | 1 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. |  |  |  |

Table 2: OBUE limits (Category B) applicable in the frequency range 52.6 – 71 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-5 dBm, Max(Prated,t,TRP – 31 dB, -12 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fB | 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz | Min(-13 dBm, Max(Prated,t,TRP – 39 dB, -20 dBm)) | 1 MHz |
| fB f < fmax | fB +5 MHz f_offset < f_ offsetmax | Min(-5 dBm, Max(Prated,t,TRP – 29 dB, -10 dBm)) | 10 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. NOTE 2: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz. |  |  |  |

  - Option 2: TBA

- Recommended WF

  - Option 1

| Company | Comments |
| --- | --- |
| Nokia | Issue 1-3-1: deltafOBUE Propose option 1, which is more aligned with ETSI EN 303 722 and 2 GHz maximum channel bandwidth compared to option 2. Issue 1-3-2: OBUE limits Propose option 1. |
| Ericsson | Issue 1-3-1: deltafOBUE Based on required exclusion to handle IM3 we suggest setting the exclusion to 3 GHz. We think 3.5 GHz is a bit of excessive. We don’t see the technical justification to add another 500 MHz to get 3.5 GHz exclusion. Considering adjacent services that’s a pretty wide exclusion zone. Based on filter technologies and considerations to adjacent services 3 GHz is sufficient. We prefer option 2 Issue 1-3-2: OBUE limits The extension of FR2 mask presented above supports the ETSI BRAN c2 mask well. We support option 1 |
| Huawei | Issue 1-3-1: deltafOBUE Option 1 was based on ETSI guidance. Probably we can limit the range to 3-3.5 GHz and have more time to analyze the filter performance. Issue 1-3-2: OBUE limits More time to check Option 1 needed. |
| CATT | Issue 1-3-1: deltafOBUE Issue 1-3-2: OBUE limits Both ok with the recommended WF. |
| NEC | Issue 1-3-1: Option 1. It is preferable to make the requirement aligned with ETSI EN 303 722 as much as possible. |
| ZTE | Issue 1-3-2: OBUE limits Both ok with the recommended WF. |
| Ericsson | We are ok to compromise to go for option 1. |

### Sub-topic 1-4 Emissions: Spurious emissions

Background: the following was agreed in previous meeting:

- For licensed operation and unlicensed operation, RAN4 should re-use the FR2 approach and use FR2 spurious emission requirements for NR in 52.6 to 71 GHz and make adaptations with respect Fstep,X taking to account larger carrier bandwidths.

The following band definitions were agreed at TSG RAN4#98-e:

- Licensed band definition: [66 – 71] GHz.

- Unlicensed band definition: 57 – 71 GHz

Issue 1-4-1: Spurious emission limits

- Proposals

  - Option 1: Re-use both cat A and cat B emission limits, with new step frequencies defined for cat B. (Nokia R4-2200411)

  - Option 2: TBA

- Recommended WF

  - TBA

Issue 1-4-2: Step frequencies

- Proposals

  - Option 1: (Nokia R4-2200411, CATT R4-2200136), CATT proposal was only for unlicensed band

Step frequencies for defining the BS radiated Tx spurious emission limits in FR2 (Category B)

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) (Note 2) | Fstep,4 (GHz) (Note 2) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n259 | 23.5 | 35.5 | 38 | 45 | 47.5 | 59.5 |
| [Licensed band] | [46] | [61] | [62.5] | [74.5] | [76] | [91] |
| Unlicensed band | 18 | 43 | 53.5 | 74.5 | 85 | 127 |
| NOTE 1: Fstep,X are based on ERC Recommendation 74-01 [19], Annex 2. NOTE 2: Fstep,3 and Fstep,4 are aligned with the values for ΔfOBUE in Table 9.7.1-1. |  |  |  |  |  |  |

  - Option 2: (Ericsson R4-2200943)

Step frequencies for defining the BS radiated transmitter spurious emission limits in FR2 (Category B)

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) | Fstep,4 (GHz) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n259 | 23.5 | 35.5 | 38 | 45 | 47.5 | 59.5 |
| n26x (licensed operation) | 46 | 61 | 63 | 74 | 76 | 91 |
| n263 | 18 | 43 | 54 | 74 | 84 | 127 |

- Recommended WF

  - Option 1 in case 3500 MHz ΔfOBUE agreed, option 2 in case 3000 MHz ΔfOBUE agreed.

  - Only n263 (i.e. unlicensed band) to be included in specifications, values for licensed band are for information

| Company | Comments |
| --- | --- |
| Nokia | Issue 1-4-1: Spurious emission limits Propose option 1. Issue 1-4-2: Step frequencies Propose option 1, which is more aligned with ETSI EN 303 722 and 2 GHz maximum channel bandwidth compared to option 2. |
| Ericsson | Issue 1-4-1: Spurious emission limits We support re-use of FR2 spurious emission level for Cat A and Cat B. The Cat B levels requires some modifications with respect to the exclusion zone. We prefer option 1. Issue 1-4-2: Step frequencies Based on the fact that we support 3 GHz exclusion zone we prefer option 2. |
| Huawei | Issue 1-4-1: Spurious emission limits Option 1 looks ok. Issue 1-4-2: Step frequencies Fix deltaOBUE first. |
| CATT | Issue 1-4-1: Spurious emission limits Option 1 Issue 1-4-2: Step frequencies Option 1 |
| ZTE | Issue 1-4-1: Spurious emission limits Option 1, both licensed band and unlicensed band should be defined. |
| Ericsson | Since we are alone on the 3 GHz side, we can offer a compromise to support 3.5 GHz to make progress on emissions. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2200414 Draft CR to clauses 9.1 – 9.5 | ericsson: Ericsson: Maybe better to split table 9.4.3.3-1 in two separate tables for FR2-1 and FR2-2 to be more precis on applicability. The applicability is anyway described in section 4/5. See proposal in draft spec text in R4-2200843. |
|  | Nokia response to Ericsson: Ok to have separate tables for FR2-1 and FR2-2. |
|  | CATT: The same comment as Ericsson. And some alignment in high level is needed for the CRs. |
| R4-2200845 Draft CR to TS 38.104: Addition of requirements for NR extension up to 71 GHz in subclause 9.6 to 9.8 | Nokia: Format looks ok. |
|  | Huawei: - Table 9.6.2.3-1: it looks odd to mix FR2 and FR2-1. Better to replace FR2 with “FR2-1, FR2-2” - More time needed to check OBUE. |
|  | CATT: - Table 9.6.2.3-1, maybe a note that 256QAM is only applicable for FR2-1 is clearer? - Table 9.6.3.3-4, the change should consider the impact to other specs such as 38.141-2. Table 9.7.3.3-1, “FR2-1” and “FR2-2” may not be necessarily needed because the frequency range is clear enough. The same comments to Table 9.7.3.3-3, Table 9.7.3.3-4, Table 9.7.4.3.2-1, Table 9.7.4.3.2-2, Table 9.7.4.3.2-4, etc |
|  | Ericsson: I agree with comments from Huawei, lets use FR2-1 and FR2-2 in applicability tables. For OBUE I think we can add tables but add [] around values to get structure in place. To CATT, I think we don’t need both a note and applicability column in table. The update to conformance specs we can handle later in conformance part. We have some legacy issues to resolve in the section about TAE. A table would significantly improve readability of the specs. Please check uploaded revision. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 TAE | For issue 1-1-1 for interband CA, all companies agree to use 3us TAE. For issue 1-1-2 for intraband contiguous CA, 6 companies would be ok with option 3 (65 ns for 480 kHz SCS, 32.5 ns for 960 kHz SCS) and 4 companies would be ok with option 4 (80 ns for 480 kHz SCS, 40 ns for 960 kHz SCS) For issue 1-1-3 for intraband non-contiguous CA option 2 (260 ns) got most support with 6 companies being ok with it For issue 1-1-4 for MIMO option 2 (65 ns for 480 kHz SCS, 32.5 ns for 960 kHz SCS) got most support with 6 companies being ok with it Tentative agreements: TAE is set as - Interband CA: 3us - Contiguous intra-band CA: 65 ns for 480 kHz SCS, 32.5 ns for 960 kHz SCS - Non-contiguous intraband CA: 260 ns - MIMO: 65 ns for 480 kHz SCS, 32.5 ns for 960 kHz SCS Candidate options: Recommendations for 2nd round: Capture agreements in WF and/or draftCR. |
| Sub-topic #1-2 EVM | For EVM all commenting companies agree to re-use 50% (of the normal CP length) EVM window length for FR-2-2. Tentative agreements: - re-use 50% of the normal CP EVM window length for FR-2-2. Candidate options: Recommendations for 2nd round: Capture agreements in WF and/or draftCR. |
| Sub-topic #1-3 Emissions: OBUE | For issue 1-3-1 with deltafOBUE only one company does not support 3.5 GHz. The other 5 companies are ok with 3.5 GHz. For issue 1-3-2 for OBUE limits all companies having a preference agree option 1: Tables 1 and 2 below (Nokia R4-2200413, Ericsson R4-2200943) Table 1: OBUE limits (Category A) applicable in the frequency range 52.6 – 71 GHz Frequency offset of measurement filter -3B point, f; Frequency offset of measurement filter centre frequency, f_offset; Limit; Measurement bandwidth. 0 MHz f < 0.1*BWcontiguous; 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz; Min(-5 dBm, Max(Prated,t,TRP – 31 dB, -12 dBm)); 1 MHz. 0.1*BWcontiguous f < fmax; 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax; Min(-13 dBm, Max(Prated,t,TRP – 39 dB, -20 dBm)); 1 MHz. NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap.. Table 2: OBUE limits (Category B) applicable in the frequency range 52.6 – 71 GHz Frequency offset of measurement filter -3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Limit; Measurement bandwidth. 0 MHz f < 0.1*BWcontiguous; 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz; Min(-5 dBm, Max(Prated,t,TRP – 31 dB, -12 dBm)); 1 MHz. 0.1*BWcontiguous f < fB; 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz; Min(-13 dBm, Max(Prated,t,TRP – 39 dB, -20 dBm)); 1 MHz. fB f < fmax; fB +5 MHz f_offset < f_ offsetmax; Min(-5 dBm, Max(Prated,t,TRP – 29 dB, -10 dBm)); 10 MHz. NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. NOTE 2: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz.. Tentative agreements: - ΔfOBUE = 3500 MHz for FDL,high – FDL,low > 4000 MHz for 52.6-71GHz - OBUE limits as shown in Table 1 and Table 2 above Candidate options: Recommendations for 2nd round: Capture agreements in WF and/or draftCR. |
| Sub-topic #1-4 Emissions: Spurious emissions | For spurious emissions all commenting companies are aligned to re-use both cat A and cat B emission limits, with new step frequencies defined for cat B. For the detailed step frequencies, they should be aligned with deltafOBUE and therefore based on sub-topic 1-3 option 1 is to be followed. Tentative agreements: - Re-use cat A and cat B emission limits, with new step frequencies defined for cat B. - Step frequencies are set as in R4-2200136, i.e. only unlicensed band to be included. Candidate options: Recommendations for 2nd round: Capture agreements in WF and/or draftCR. |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2200414 | To be revised |
| R4-2200845 | To be revised |

## Discussion on 2nd round (if applicable)

To take place directly in WF and draftCRs. It is possible to give comments to draftCR also below.

| CR/TP number | Comments collection |
| --- | --- |
| Revision of R4-2200414 Draft CR to clauses 9.1 – 9.5 |  |
| Revision of R4-2200845 Draft CR to TS 38.104: Addition of requirements for NR extension up to 71 GHz in subclause 9.6 to 9.8 |  |
|  | Nokia: comments and suggestions made directly on draft WF and CR. |

Comments copied from Tx WF:

| Company | Comments |
| --- | --- |
| ZTE | Both ZTE and CATT simulation results has shown that more stringent requirement are needed for MIMO TAE requirement, and Nokia's proposal R4-2200411 is also close to ours. As compromise, maybe we are fine with OPTION 3 instead of OPTION 2. In addition, for intra-band contiguous CA or non-contiguous CA case, as I mentioned before it's better to discuss together with UE CBM or IBM together to avoid revising requirement later on due to big interruption on UE Rx beam sweeping. This requirement has been discussed a lot in UE RRM session in the past, we want to avoid that happen again in Rel-17 or in future if identified by UE RRM colleagues. In addition, for intra-band non-contiguous CA, as mentioned in Ericsson contribution on RF architecture, 260ns is based on assumption of 5CC aggregated with individual RF chain inherited from LTE CA in Rel-10 I think, the maximum TAE between 1st CC and 5th CC should be 65x4=260ns (eCA increase CCs numbers up to [32] in [Rel-14/15])，for FR2-2, I am not sure whether this architecture is still valid or common used, In addition, the maximum aggregate CC numbers should also been further discussed with market demand. |
| Ericsson | === MIMO === For MIMO, receiver we can look at time domain and check first principles and intuition, ie that a channel inside CP or a channel with very little power outside CP will perform well. The CP for SCS = 480 kHz is 146 ns and the CP for SCS = 960 kHz is 73 ns. The TDLA-5 last tap is at 48 ns delay and -30 dBc down. The TDLA-5 second to last tap is at 26.5 ns delay. For SCS=480 kHz a TAE=65 ns will fit the entire TDLA-5 delay train inside CP = 146 ns No degradation is expected from a receiver estimating channel across all CP. For SCS = 960 kHz a TAE = 32.5 ns will push last tap of TDLA-5 out, but last tap is 30 dB down => negligible impact. This is what we see in our simulation, ie TAE = 65 ns and TAE = 32.5 ns works for TDLA-5, when UE estimates channel across whole CP. === for intra band contiguous === This is colocated and proposed TAE = 65 ns and 32.5 ns fit within CP and leave margin for channel dispersion. Intra band contiguous should fit in common wide receiver FFT, so CP fraction is main design parameter. CBM discussion in RRM is related to interband CA. === for intra band non-contiguous === The issue is that TAE = 260 ns works for FR2-1 up to 51.6 GHz, there is noting major going on in FR2-2 52.6-71 GHz preventing 260 ns TAE from working. The issue is if RAN4 shall assume single wide FFT also for intra-band non-contiguous cases (fraction of CP for TAE), but that is not feasible across whole 52.6-71 GHz band |
| CATT | For the NC CA, we have concerns of the separated receiver assumption for UE side. Separated receiver assumption is not always valid for FR2 UE. We also would like to know the system impact for this requirement. For MIMO TAE, our channel model is assumed to TDLA with 10ns delay spread and 5Hz Doppler. From our simulation, the for 35ns TAE, throughput for 960kHz @ 35dB SNR is close to 0. We would like to keep it FFS in this meeting. |

# Topic #2: Rx requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200137 | CATT | Observation 1: FRC and SNR value for each new FRC is to be confirmed once spectral utilization is agreed. Proposal 1: To specify the following Fstep,X for receiver spurious emissions. Operating band; Fstep,1; Fstep,2; Fstep,3; Fstep,4; Fstep,5; Fstep,6. (GHz); (GHz); (GHz); (GHz); (GHz); (GHz). n263; 18; 43; 53.5; 74.5; 85; 127. Proposal 2: To use the following equations to calculate wanted signal mean power and interfering signal mean power respectively for OTA in-channel selectivity requirement for FR2-2. EISREFSENS_wanted = EISREFSENS_50M +10*log10(Bwwanted/ BW50M)+( SNRwanted – SNR50M)+3 EISREFSENS_interfer = EISREFSENS_50M +10*log10(Bwinterfer/ BW50M)+ ICS-0.9 Observation 2: ICS in equation is achieved by IoT simulation instead of the difference between the wanted signal and the interfering signal. Proposal 3: To define the G-FR2-A1-8 for 480kHz SCS, and G-FR2-A1-9 for 960kHz SCS for OTA in-channel selectivity requirement for FR2-2. |
| R4-2200151 | CATT | Moderator: This is draft CR, included in section 2.2.8 |
| R4-2200412 | Nokia, Nokia Shanghai Bell | Proposal 1: Some FR2 parameters (like modulation and coding rate) can be reused for NR operation in 52.6 – 71 GHz range, while other parameters (like allocated resource blocks) and thus they should be finalized when the parameters they depend on (like maximum SU for each SCS and channel bandwidth combination) are finalized. Proposal 2: The ACS requirement for NR operation in 52.6 – 71 GHz range should be specified as 22 dB. Proposal 3: The in-band blocking level (33 dB higher than the OTA REFSENS level) of BS type 2-O should be used for NR operation in 52.6 – 71 GHz range. Proposal 4: The Rx spurious emission limits for NR operation in 52.6 – 71 GHz range should be defined as: Table 1: Radiated Rx spurious emission limits for BS type 2-O Spurious frequency range (Note 4); Limit (Note 5); Measurement Bandwidth; Note. 30 MHz 1 GHz; -36 dBm; 100 kHz; Note 1. 1 GHz 18 GHz; -30 dBm; 1 MHz; Note 1. 18 GHz Fstep,1; -20 dBm; 10 MHz; Note 2. Fstep,1 Fstep,2; -15 dBm; 10 MHz; Note 2. Fstep,2 Fstep,3; -10 dBm; 10 MHz; Note 2. Fstep,4 Fstep,5; -10 dBm; 10 MHz; Note 2. Fstep,5 Fstep,6; -15 dBm; 10 MHz; Note 2. Fstep,6 2nd harmonic of the upper frequency edge of the UL operating band; -20 dBm; 10 MHz; Note 2, Note 3. NOTE 1: Bandwidth as in ITU-R SM.329 [2], s4.1. NOTE 2: Limit and bandwidth as in ERC Recommendation 74-01 [19], Annex 2. NOTE 3: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 4: The step frequencies Fstep,X are defined in table 10.7.3-2. NOTE 5: Additional limits may apply regionally.. Table 2: Step frequencies for defining the radiated Rx spurious emission limits for BS type 2-O Operating band; Fstep,1 (GHz); Fstep,2 (GHz); Fstep,3 (GHz); Fstep,4 (GHz); Fstep,5 (GHz); Fstep,6 (GHz). n257; 18; 23.5; 25; 31; 32.5; 41.5. n258; 18; 21; 22.75; 29; 30.75; 40.5. n259; 23.5; 35.5; 38; 45; 47.5; 59.5. n260; 25; 34; 35.5; 41.5; 43; 52. n261; 18; 25.5; 26.0; 29.85; 30.35; 38.35. n262; 37.2; 45.2; 45.7; 49.7; 50.2; 58.2. Licensed band; 46; 61; 62.5; 74.5; 76; 91. Unlicensed band; 18; 43; 53.5; 74.5; 85; 127. Proposal 5: The interferer levels for general receiver intermodulation (25 dB higher than the OTA REFSENS level) of BS type 2-O should be used for NR operation in 52.6 – 71 GHz range. Proposal 6: Specify the BS ICS requirement as 10 dB for NR operation in 52.6 – 71 GHz range and calculate the wanted signal and interfering signal power levels from the OTA REFSENS level accordingly. Proposal 7: fOOB shall be defined as 3500 MHz for NR operation in 52.6 – 71 GHz range. |
| R4-2200844 | Ericsson | Proposal 1: For OTA sensitivity specify OTA reference sensitivity level as described in Table 2.1-1. Table 2.1-1: Reference sensitivity requirement BS channel bandwidth (MHz); Sub-carrier (kHz); FRC bandwidth (MHz); Reference measurement channel; OTA reference sensitivity level, EISREFSENS (dBm). 50, 100, 200; 60; 50; G-FR2-A1-1; EISREFSENS_50M + FR2_REFSENS. 50; 120; 50; G-FR2-A1-2; EISREFSENS_50M + FR2_REFSENS. 100, 200, 400; 120; 100; G-FR2-A1-3; EISREFSENS_50M + 3 + FR2_REFSENS. 400, 800, 1600; 480; 400; G-FR2-A1-6; EISREFSENS_50M + 9 + FR2_REFSENS. 400, 800, 1600, 2000; 960; 400; G-FR2-A1-7; EISREFSENS_50M + 9 + FR2_REFSENS. Proposal 2: For ACS applicable for FR2-2 define the interferer signal mean power to EISREFSENS_50M + [25.7] + ΔFR2_REFSENS. Proposal 3: For ACS define interfering signal type as 100 MHz DFT-s-OFDM NR signal, 120 kHz SCS, 32 RBs. Proposal 4: For in-band blocking add new row in Table 10.5.2.3-0 in TS 38.104 for FR2-2 specific fOOB. Proposal 5: For in-band blocking set interfering signal power to EISREFSENS_50M + [33] + ΔFR2_REFSENS. Proposal 6: For receiver spurious emission set Fstep,x to values listed in Table 2.4-2 for band n263. Table 2.4-2: Step frequencies for defining the spurious emission limits Frequency range (MHz); Operating band; Fstep,1 (GHz); Fstep,2 (GHz); Fstep,3 (GHz); Fstep,4 (GHz); Fstep,5 (GHz); Fstep,6 (GHz). 26500 to 29500; n257; 18; 23.5; 25; 31; 32.5; 41.5. 24250 to 27500; n258; 18; 21; 22.75; 29; 30.75; 40.5. 39500 to 43500; n259; 23.5; 35.5; 38; 45; 47.5; 59.5. 37000 to 40000; n260; 25; 34; 35.5; 41.5; 43; 52. 27500 to 28350; n261; 18; 25.5; 26.0; 29.85; 30.35; 38.35. 47200 to 48200; n262; 37.2; 45.2; 45.7; 49.7; 50.2; 58.2. 66000 to 71000; n26x (licensed operation); 46; 61; 63; 74; 76; 91. 57000 to 71000; n263; 18; 43; 54; 74; 84; 127. Proposal 7: For receiver intermodulation set the interfering signal level to EISREFSENS_50M + [25] + ΔFR2_REFSENS. Proposal 8: For in-channel selectivity define wanted signal power level and interfering signal power level as described in Table 2.6-1. Table 2.6-1: Relation between wanted signal power and interfering signal power BS channel bandwidth (MHz); Wanted signal power (dBm); Interfering signal power (dBm). 50; EISREFSENS_50M + ΔFR2_REFSENS; EISREFSENS_50M + 10 + ΔFR2_REFSENS. 100; EISREFSENS_50M + 3 + ΔFR2_REFSENS; EISREFSENS_50M + 13 + ΔFR2_REFSENS. 400; EISREFSENS_50M + 9 + ΔFR2_REFSENS; EISREFSENS_50M + 19 + ΔFR2_REFSENS. |
| R4-2201457 | ZTE Corporation | Proposal 1: to reuse the existing ACS requirement in TR 38.803 for 52.6-71GHz with round up to nearest integer and frequency offset updates; Proposal 2: propose to follow the existing FR2 requirement for 52.6-71GHz IBB requirement; Proposal 3: to reuse 14dBc in-channel selectivity for 52.6-71GHz BS ICS requirement; |
| R4-2201822 | Huawei | Proposal: Based on the above background on the FR2-1 RX IMD derivation, the reuse of the legacy 8 dB offset of the RX IMD below the blocking level is considered as acceptable approach. Proposal: reuse the FR2-1 approach with the 10 dB offset among the wanted and the interferer signals. Moderator: This is for ICS. Proposal: Define ΔfOOB values as [3.5] GHz. Observation: in order to derive the above step frequencies, the ΔfOBUE / ΔfOOB value needs to be agreed first. Afterwards, the process is straight-forward and follows 74-01 guidance. |
| R4-2201823 | Huawei | Moderator: This is draft CR, included in section 2.2.8 |

## Open issues summary

Please note it is possible and often necessary to select multiple options to create coherent agreements/requirements.

### Sub-topic 2-1 EIS

Background: Agreed WF from R4-2115644: For OTA reference sensitivity use FR2 EIS_REFSENS_50M sensitivity declaration as the starting point, i.e., BS sensitivity for FR2-2 is declared based on 50 MHz reference and allowed power level to be declared for each class is according to TS 38.104, clause 10.3.3.

Agreed in previous meeting:

  - Existing G-FR2-A1-2 can be re-used for 100MHz/120kHz for ICS for 52.6-71GHz.

  - For OTA reference sensitivity define a new dedicated FRC (G-FR2-A1-6) for 480 kHz SCS and 400 MHz carrier bandwidth

  - For OTA reference sensitivity define a new dedicated FRC (G-FR2-A1-7) for 960 kHz SCS and 400 MHz carrier bandwidth.

Issue 2-1: EIS

- Proposals

  - Option 1: For OTA sensitivity specify OTA reference sensitivity level as described in Table 2.1-1. (Ericsson, R4-2200844)

Table 2.1-1: Reference sensitivity requirement

| BS channel bandwidth (MHz) | Sub-carrier (kHz) | FRC bandwidth (MHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- | --- |
| 50, 100, 200 | 60 | 50 | G-FR2-A1-1 | EISREFSENS_50M + FR2_REFSENS |
| 50 | 120 | 50 | G-FR2-A1-2 | EISREFSENS_50M + FR2_REFSENS |
| 100, 200, 400 | 120 | 100 | G-FR2-A1-3 | EISREFSENS_50M + 3 + FR2_REFSENS |
| 400, 800, 1600 | 480 | 400 | G-FR2-A1-6 | EISREFSENS_50M + 9 + FR2_REFSENS |
| 400, 800, 1600, 2000 | 960 | 400 | G-FR2-A1-7 | EISREFSENS_50M + 9 + FR2_REFSENS |

  - Option 2: TBA

- Recommended WF

  - Option 1

| Company | Comments |
| --- | --- |
| Nokia | Ok with option 1 with ‘9’ in square brackets pending finalization of new FRCs and simulation results. |
| Ericsson | We prefer option 1. |
| Qualcomm | Ok with option 1. |
| Huawei | Same as Nokia. |
| CATT | We reached the same value with option 1 in our contribution and support Nokia’s comments that 9 can be in brackets for further check. |
| ZTE | For FR2-2 FRC based simulation, it could be done, however SNR might be still -1dB since it’s up to declaration. It’s not big issues. Here we are discussing the REFESNS range |

### Sub-topic 2-2 FRC

Background: Agreements in previous meeting

  - Existing G-FR2-A1-2 can be re-used for 100MHz/120kHz for ICS for 52.6-71GHz.

  - For OTA reference sensitivity define a new dedicated FRC (G-FR2-A1-6) for 480 kHz SCS and 400 MHz carrier bandwidth

  - For OTA reference sensitivity define a new dedicated FRC (G-FR2-A1-7) for 960 kHz SCS and 400 MHz carrier bandwidth.

Issue 2-2: FRC

- Proposals

  - Option 1: Proposal 1: Some FR2 parameters (like modulation and coding rate) can be reused for NR operation in 52.6 – 71 GHz range, while other parameters (like allocated resource blocks) and thus they should be finalized when the parameters they depend on (like maximum SU for each SCS and channel bandwidth combination) are finalized. (Nokia, R4-2200412)

Observation from CATT (R4-2200137): FRC and SNR value for each new FRC is to be confirmed once spectral utilization is agreed.

- Recommended WF

Moderator’s proposal: postpone further FRC discussion on EIS FRCs until SU is finalized.

| Company | Comments |
| --- | --- |
| Nokia | Agree with moderator’s proposal. |
| Ericsson | Option 1: Sounds like a reasonable way forward. |
| Qualcomm | Ok with option 1. |
| Huawei | Agree with moderator’s proposal. |
| CATT | Support the recommended WF. |
| ZTE | Support the recommended WF. |

### Sub-topic 2-3 ACS

Background: agreements in previous meeting

- Use 100 MHz DFT-s-OFDM interferer, SU details FFS.

- Add new rows for FR2-2 in relevant ACS tables.

Issue 2-3: ACS

Background:

- Proposals

  - For ACS define interfering signal type as 100 MHz DFT-s-OFDM NR signal, 120 kHz SCS, 32 RBs. (Ericsson, R4-2200844)

- Recommended WF

Moderator’s proposal: Confirm the PRB value once SU is agreed.

| Company | Comments |
| --- | --- |
| Nokia | Agree with moderator’s proposal. |
| Ericsson | We are ok with given proposal and WF. |
| Qualcomm | Ok with option 1. |
| Huawei | Agree with moderator’s proposal. |
| CATT | Support the recommended WF. |
| ZTE | Support the recommended WF. |

### Sub-topic 2-4 In-band blocking

Background: Agreements in previous meeting

- Use 100 MHz DFT-s-OFDM interferer, SU details FFS.

- Add new rows for FR2-2 in relevant IBB tables.

Issue 2-4: In-band blocking

- Proposals

  - Option 1: The in-band blocking level (33 dB higher than the OTA REFSENS level) of BS type 2-O should be used for NR operation in 52.6 – 71 GHz range. (Nokia, R4-2200412, Ericsson R4-2200844, ZTE R4-2201457)

  - Option 2: TBA

- Recommended WF

Option 1

| Company | Comments |
| --- | --- |
| Nokia | Propose option 1. |
| Ericsson | We are ok with option 1. |
| Qualcomm | Ok with option 1. |
| CATT | Support the recommended WF. |
| ZTE | Fine with recommended WF. |

### Sub-topic 2-5: spurious emissions step frequencies and ΔfOOB

Background: Agreements in previous meeting

- Use FR2 approach with necessary adaptations on step size for spurious emissions, i.e., align emission levels with TX requirements

- Use TRP as emissions metric

- Step size Fstep,x needs further discussion

- When fOBUE is determined align fOOB with fOBUE.

Issue 2-5: Spurious emissions steps and ΔfOOB

- Proposals

  - Option 1: 3.5 GHz (Nokia, R4-2200412, CATT, R4-2200137, Huawei R4-2201822)

- Table 2: Step frequencies for defining the radiated Rx spurious emission limits for BS type 2-O

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) | Fstep,4 (GHz) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n259 | 23.5 | 35.5 | 38 | 45 | 47.5 | 59.5 |
| n260 | 25 | 34 | 35.5 | 41.5 | 43 | 52 |
| n261 | 18 | 25.5 | 26.0 | 29.85 | 30.35 | 38.35 |
| n262 | 37.2 | 45.2 | 45.7 | 49.7 | 50.2 | 58.2 |
| Licensed band | 46 | 61 | 62.5 | 74.5 | 76 | 91 |
| Unlicensed band | 18 | 43 | 53.5 | 74.5 | 85 | 127 |

  - Option 2: 3 GHz (Ericsson, R4-2200844)

- Table 2.4-2: Step frequencies for defining the spurious emission limits

| Frequency range (MHz) | Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) | Fstep,4 (GHz) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 26500 to 29500 | n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| 24250 to 27500 | n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| 39500 to 43500 | n259 | 23.5 | 35.5 | 38 | 45 | 47.5 | 59.5 |
| 37000 to 40000 | n260 | 25 | 34 | 35.5 | 41.5 | 43 | 52 |
| 27500 to 28350 | n261 | 18 | 25.5 | 26.0 | 29.85 | 30.35 | 38.35 |
| 47200 to 48200 | n262 | 37.2 | 45.2 | 45.7 | 49.7 | 50.2 | 58.2 |
| 66000 to 71000 | n26x (licensed operation) | 46 | 61 | 63 | 74 | 76 | 91 |
| 57000 to 71000 | n263 | 18 | 43 | 54 | 74 | 84 | 127 |

- Recommended WF

Discuss in Tx side, align decision for Rx following the agreement from previous meeting

| Company | Comments |
| --- | --- |
| Nokia | Propose option 1, which is more aligned with ETSI EN 303 722 and 2 GHz maximum channel bandwidth compared to option 2. |
| Ericsson | Based on the fact that we prefer 3 GHz exclusion we prefer option 2. |
| Qualcomm | Ok with option 1. |
| Huawei | Align with the Tx side. |
| CATT | Option 1. |
| Ericsson | Since we are alone proponent of option 2 we can compromise on supporting option 1. |

### Sub-topic 2-6 Rx IMD

Background: Agreements in previous meeting:

- Use 100 MHz DFT-s-OFDM signal with [8] dB offset.

- SU agreement is needed before frequency offset and RB number details can be agreed.

Issue 2-6: Rx IMD

Background:

- Proposals

  - Option 1: The interferer levels for general receiver intermodulation (25 dB higher than the OTA REFSENS level) of BS type 2-O should be used for NR operation in 52.6 – 71 GHz range. (Nokia R4-2200412, Ericsson R4-2200844)

  - Option 2: the legacy 8 dB offset of the RX IMD below the blocking level is considered as acceptable approach. (Huawei R4-2201822)

Moderator: option 2 is the same as option 1 with the assumption that in-band blocking level is 33 dB higher than OTA REFSENS level)

- Recommended WF

TBA

| Company | Comments |
| --- | --- |
| Nokia | Propose option 1, this can be agreed with IBB together which means option 2 is the same. |
| Ericsson | We prefer option 1. |
| Qualcomm | Ok with option 1. |
| CATT | Option 1. |
| ZTE | okay with option 1 |

### Sub-topic 2-7 In-channel selectivity

Background: Agreements in previous meeting:

- Specify the BS ICS requirement as [10] dB for NR operation in 52.6 – 71 GHz range.

- For in-channel selectivity scale wanted power and interfering power levels for 100 MHz and 400 MHz carrier bandwidths.

Issue 2-7-1: In-channel selectivity levels

- Proposals

  - Option 1: use the following equations to calculate wanted signal mean power and interfering signal mean power respectively for OTA in-channel selectivity requirement for FR2-2.

EISREFSENS_wanted = EISREFSENS_50M +10*log10(Bwwanted/ BW50M)+( SNRwanted – SNR50M)+3

EISREFSENS_interfer = EISREFSENS_50M +10*log10(Bwinterfer/ BW50M)+ ICS-0.9 (CATT R4-2200137)

  - Option 2: Specify the BS ICS requirement as 10 dB for NR operation in 52.6 – 71 GHz range and calculate the wanted signal and interfering signal power levels from the OTA REFSENS level accordingly. (Nokia R4-2200412)

  - Option 3: For in-channel selectivity define wanted signal power level and interfering signal power level as described in Table 2.6-1. (Ericsson, R4-2200844)

Table 2.6-1: Relation between wanted signal power and interfering signal power

| BS channel bandwidth (MHz) | Wanted signal power (dBm) | Interfering signal power (dBm) |
| --- | --- | --- |
| 50 | EISREFSENS_50M + ΔFR2_REFSENS | EISREFSENS_50M + 10 + ΔFR2_REFSENS |
| 100 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS |
| 400 | EISREFSENS_50M + 9 + ΔFR2_REFSENS | EISREFSENS_50M + 19 + ΔFR2_REFSENS |

  - Option 4: Reuse the FR2-1 approach with the 10 dB offset among the wanted and the interferer signals. (Huawei R4-2201822, ZTE R4-2201457)

- Recommended WF

  - TBA

Issue 2-7-2: FRC for in-channel selectivity

- Proposals

  - Option 1: define the G-FR2-A1-8 for 480kHz SCS, and G-FR2-A1-9 for 960kHz SCS for OTA in-channel selectivity requirement for FR2-2 (CATT R4-2200137)

  - Option 2: TBA

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Nokia | Issue 2-7-1: In-channel selectivity levels Propose option 2; ok with equations in option 1; option 3 or 4 means 14dB ICS which simulation results in Nokia and ZTE papers show this is unnecessary stringent with IoT less than 10dB. Issue 2-7-2: FRC for in-channel selectivity Should be finalized after SU is finalized. |
| Ericsson | Issue 2-7-1: In-channel selectivity levels Following the approach used for FR2 we suggest limits as described in table above. Scaling is required o capture larger CBW. 10 dB difference as before is assumed. We prefer option 3. Issue 2-7-2: FRC for in-channel selectivity We don’t really follow the reasoning for using IoT assumptions for FR2-2 resulting in Option 1. We prefer some more time to discuss the need for two new FRC dedicated for FR2-2. |
| Nokia | Response to Ericsson: Allowed receiver desensitization is 3dB for ICS requirements, so the ICS shall attenuate the interfering signal at least to the noise floor level of the BS, and thus IoT simulation results were used to decide the ICS requirements in the NR WI (e.g., at least 10dB ICS is required to attenuate the interfering signal to the noise floor level if the IoT is 10dB). |
| Qualcomm | Issue 2-7-1: Ok with option 1 and ICS levels based on the methodology defined there. Issue 2-7-2: TBD. It is not clear how the proposed Reference measurement channel were derived. |
| Huawei | Issue 2-7-1: In-channel selectivity levels Option 3 and 4 seems to be the same. More analysis needed to compare with option 2. |
| CATT | Issue 2-7-1: In-channel selectivity levels We may need to align the concept first. If the ICS is the same meaning as clarified in 38.817-02. With 14 dB ICS (38.817-02 concept), the power level difference is 10 dB. So we may prefer the relaxed one which is option 2 if simulation shows no problem to have more flexibility of the ADC capability. Issue 2-7-2: FRC for in-channel selectivity Option 1 means that only one FRC for one SCS is sufficient, and agree that SU should be decided first. |
| ZTE | Issue 2-7-1: In-channel selectivity levels Option 3 and option 4 is the same, if necessary ,we could further discuss in 2nd round. Issue 2-7-2: FRC for in-channel selectivity This could be further discussed after SU is confirmed |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2200151 Draft CR for TS 38.104 on introduction of BS RF Rx requirements for 57-71GHz in section 10.1 – 10.5 | Nokia: For ΔfOOB, the frequency subrange should be indicated in Table 10.5.2.3-0. |
|  | Ericsson: With the 3 GHz exclusion zone this CR looks good for us. |
|  | CATT: Thanks for the comments. We can revise for Nokia’s comment. For the exclusion zone, Rx should be aligned with Tx, we can wait the decision. |
| R4-2201823 Draft CR to TS 38.104: implementation of FR2-2 requirements: FRC annex | Nokia: Format looks ok. |
|  | ericsson: Do we really need the statement regarding applicability in the Annex. The applicability is handled per requirement. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1: EIS | Out of five companies all support option 1, with 3 of the 5 companies requesting 9 to be put to brackets Tentative agreements: - Option 1, with the addition of putting 9 to brackets for G-FR2-A1-6 and G-FR2-A1-7 Candidate options: Recommendations for 2nd round: Capture agreements in WF and/or draftCR. |
| Sub-topic#2-2: FRC | For FRC the general consensus included both in the only proposal and moderators suggestion for WF is to postpone FRC discussion in EIS FRC until spectrum utilization is agreed. Tentative agreements: - postpone further FRC discussion on EIS FRCs until SU is finalized Candidate options: Recommendations for 2nd round: Capture agreements in WF and/or draftCR. |
| Sub-topic#2-3: ACS | For ACS 5 companies prefer to confirm the PRB value for ACS interferer signal after SU is agreed, one company is ready to agree 32 RB now. Tentative agreements: - Confirm ACS interferer PRB value once SU is agreed. Candidate options: Recommendations for 2nd round: Capture agreements in WF and/or draftCR. |
| Sub-topic#2-4: In-band blocking | For in-band blocking all commenting companies agree on option 1: in-band blocking level (33 dB higher than the OTA REFSENS level) of BS type 2-O should be used for NR operation in 52.6 – 71 GHz range Tentative agreements: - in-band blocking level (33 dB higher than the OTA REFSENS level) of BS type 2-O should be used for NR operation in 52.6 – 71 GHz range Candidate options: Recommendations for 2nd round: Capture agreements in WF and/or draftCR. |
| Sub-topic#2-5: spurious emissions | For spurious emissions 5 companies commented: 3 companies prefer 3.5 GHz deltafOOB, 1 company prefers 3 GHz, and 1 company wants to align with Tx side. Alignment with Tx was agreed already in previous meeting. Tentative agreements: - 3.5 GHz deltafOOB Candidate options: Recommendations for 2nd round: Capture agreements in WF and/or draftCR taking into account that the agreement is aligned with Tx side. |
| Sub-topic#2-6: Rx IMD | For Rx IMD all commenting companies agree to set interferer level 25 dB above OTA REFSENS Tentative agreements: - The interferer levels for general receiver intermodulation (25 dB higher than the OTA REFSENS level) of BS type 2-O should be used for NR operation in 52.6 – 71 GHz range. Candidate options: Recommendations for 2nd round: Capture agreements in WF and/or draftCR. |
| Sub-topic#2-7: In-channel selectivity | For ICS opinions are still divided and there is no clear majority as companies do not even state preference for the proposal they submitted to the meeting. If input contributions are counted, then there is 3 vs. 2 majority to define 10 dB power difference. Further discussion in WF is needed. For FRC for ICS further discussion is needed especially when SU is agreed. Tentative agreements: - FRC for ICS to be confirmed when SU is known. Candidate options: - 10 dB ICS requirement - 10 dB power difference between signals (=~14 dB ICS requirement) - Formulas from option 1 are used the calculate power difference once ICS requirement is agreed. Recommendations for 2nd round: Continue discussion in WF and capture agreements there. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2200151 | To be revised |
| R4-2201823 | To be revised |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

To take place directly in WF and draftCR. It is possible to provide comments to draftCRs also below.

| CR/TP number | Comments collection |
| --- | --- |
| Revision of R4-2200151 Draft CR for TS 38.104 on introduction of BS RF Rx requirements for 57-71GHz in section 10.1 – 10.5 | Nokia: comments and suggestions made directly on draft WF and CR. |
| Revision of R4-2201823 Draft CR to TS 38.104: implementation of FR2-2 requirements: FRC annex | Nokia: comments and suggestions made directly on draft WF and CR. |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on BS RF Tx requirements | Nokia, Nokia Shanghai Bell |  |
| WF on BS RF Rx requirements | Ericsson |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2200136 | Discussion on BS TX RF requirements for 52 6-71GHz | CATT | Noted |  |
| R4-2200411 | Proposals on BS transmitter requirements for extending current NR operation to 71 GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2200414 | Draft CR to TR 38.104: Clauses 9.1 to 9.5 | Nokia, Nokia Shanghai Bell | revised |  |
| R4-2200843 | On BS RF transmitter requirements for the frequency range 52 to 71 GHz | Ericsson | Noted |  |
| R4-2200845 | Draft CR to TS 38.104: Addition of requirements for NR extension up to 71 GHz in subclause 9.6 to 9.8 | Ericsson | Revised |  |
| R4-2201456 | Further discussion on BS Tx requirements for 52.6-71GHz | ZTE Corporation | Noted |  |
| R4-2201821 | Discussion on the remaining BS RF requirements for FR2-2: Tx requirements | Huawei | Noted |  |
| R4-2200137 | Discussion on BS RX RF requirements for 52 6-71GHz | CATT | Noted |  |
| R4-2200151 | Draft CR for TS 38.104 on introduction of BS RF Rx requirements for 57-71GHz in section 10.1 – 10.5 | CATT | revised |  |
| R4-2200412 | Proposals on BS receiver requirements for extending current NR operation to 71 GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2200844 | On BS RF receiver requirements for the frequency range 52 to 71 GHz | Ericsson | Noted |  |
| R4-2201457 | Further discussion on BS Rx requirements for 52.6-71GHz | ZTE Corporation | Noted |  |
| R4-2201822 | Discussion on the remaining BS RF requirements for FR2-2: Rx requirements | Huawei | Noted |  |
| R4-2201823 | Draft CR to TS 38.104: implementation of FR2-2 requirements: FRC annex | Huawei | revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2203012 | Draft CR to TR 38.104: Clauses 9.1 to 9.5 | Nokia, Nokia Shanghai Bell | Endorsed | Content agreerable, but still some FFS inside, waiting for agreements from main session. Therefore, update still needed in future meeting. |
| R4-2203013 | Draft CR to TS 38.104: Addition of requirements for NR extension up to 71 GHz in subclause 9.6 to 9.8 | Ericsson | Endorsed | Content agreerable, but still some FFS inside, waiting for agreements from main session. Therefore, update still needed in future meeting. |
| R4-2203015 | Draft CR for TS 38.104 on introduction of BS RF Rx requirements for 57-71GHz in section 10.1 – 10.5 | CATT | Endorsed | Content agreerable, but still some FFS inside, waiting for agreements from main session. Therefore, update still needed in future meeting. |
| R4-2203014 | Draft CR to TS 38.104: implementation of FR2-2 requirements: FRC annex | Huawei | Endorsed | Content agreerable, but still some FFS inside, waiting for agreements from main session. Therefore, update still needed in future meeting. |
| R4-2203016 | WF on BS RF Tx requirements | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2203017 | WF on BS RF Rx requirements | Ericsson | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Nokia | Man Hung Ng | man_hung.ng@nokia.com |
| Ericsson | Torbjörn Elfström | torbjorn.elfstrom@ericsson.com |
| Qualcomm | Mustafa Emara | memara@qti.qualcomm.com |
| Huawei | Michal Szydelko | Michal.szydelko@huawei.com |
| CATT | Huiping Shan | shanhuiping@catt.cn |
| NEC | Tetsu Ikeda | tetsu.ikeda@nec.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)