3GPP TSG-RAN WG4 Meeting #101bis-e                         	        R4-2200573

# Electronic meeting, 17th January – 25th January, 2022

Agenda item:	6.1.3.3

Source:	Huawei, HiSilicon

Title:	Further results on FR1 MIMO OTA Channel model validation

Document for:	Approval

# Introduction

For the purpose of lab alignment and requirements development, channel model validation has been discussed for several meetings for NR UE FR1 MIMO OTA. Reference validation targets were agreed in last RAN4 #100e and 101e meetings [2][3]. To proceed with the Channel validation work, we submitted results of Doppler/Temporal correlation and Spatial correlation in RAN4 #101e meeting [5]. This document presents the validation results of remaining items i.e. Power Delay Profile (PDP) and Cross-polarization.

Table 1: Volunteer FR1 MIMO OTA Labs

|  | Lab volunteer | City (Lab address) |
| --- | --- | --- |
| 1 | CAICT | Beijing |
| 2 | Huawei | Shanghai |
| 3 | Xiaomi | Beijing |
| 4 | MediaTek | Beijing |
| 5 | Apple | Cupertino |
| 6 | BUPT & CMCC Joint Lab | Beijing |
| 7 | SGS | New Taipei City |

# Discussion

Table 1: General parameters

| Channel model | FR1 CDL-C UMa |
| --- | --- |
| Polarization | X2V |
| Measurements | Power Delay Profile (PDP) Cross-polarization |
| Frequencies | 2450MHz, 3600MHz |

  - Power Delay Profile (PDP)

The delay resolution is determined by the sweep bandwidth of VNA in PDP validation, e.g., 200MHz bandwidth resulting in 5ns resolution in delay domain. Besides, the bandwidth of channel emulator should also be taken into consideration given that the test bandwidth is no more than 40MHz. If the operation bandwidth of a channel emulator in FR1 is less than 200MHz, the available delay resolution is more than 5ns, e.g., 160MHz bandwidth resulting in 6.25ns resolution. As the clusters whose delay within 10ns are not easy to be recognized, we combine them and renormalize their power. The delay for the strongest cluster is chosen for the combined, as shown in below table.

- 2.45GHz

2.45GHz Beam1 reference [3]

| Index | Delay(ns) | Power(dB) | Cluster combined& renormalized |
| --- | --- | --- | --- |
| 1 | 0 | -31.4 | -34.3 |
| 2 | 76.6 | -28.1 | NA |
| 3 | 79.4 | -20.7 | NA |
| 4 | 81 | -20.5 | -19.5 |
| 5 | 85 | -25.9 | NA |
| 6 | 232.4 | 0 | 0.0 |
| 7 | 235.4 | -2.8 | NA |
| 8 | 239.4 | -3.9 | NA |
| 9 | 289.6 | -30.8 | -33.0 |
| 10 | 299.8 | -38.7 | NA |
| 11 | 448.4 | -32.9 | -35.8 |
| 12 | 477.5 | -31.1 | -34.0 |

2.45GHz Beam2 reference [3]

| Index | Delay(ns) | Power(dB) | Cluster combined& renormalized |
| --- | --- | --- | --- |
| 1 | 0 | -24.8 | -27.9 |
| 2 | 76.6 | 0 | 0.0 |
| 3 | 79.4 | -11.3 | NA |
| 4 | 81 | -2.7 | NA |
| 5 | 85 | -3.8 | NA |
| 6 | 232.4 | -23.2 | NA |
| 7 | 235.4 | -16.9 | -18.4 |
| 8 | 239.4 | -23.8 | NA |
| 9 | 289.6 | -25 | -27.8 |
| 10 | 299.8 | -37.4 | NA |
| 11 | 448.4 | -24.8 | -27.9 |
| 12 | 477.5 | -24.9 | -28.0 |
| 13 | 1679.1 | -38.1 | NA(<-40) |

Measurement results from Huawei’s Lab:

- 3.6GHz

3.6GHz Beam1 reference [3]

| Index | Delay(ns) | Power(dB) | Cluster combined& renormalized |
| --- | --- | --- | --- |
| 1 | 0 | -31.5 | -34.2 |
| 2 | 76.6 | -28.2 | NA |
| 3 | 79.4 | -20.8 | NA |
| 4 | 81 | -20.3 | -19.3 |
| 5 | 85 | -25.7 | NA |
| 6 | 232.4 | 0 | 0.0 |
| 7 | 235.4 | -3.4 | NA |
| 8 | 239.4 | -3.8 | NA |
| 9 | 289.6 | -32 | -34.7 |
| 10 | 448.4 | -33.1 | -35.8 |
| 11 | 477.5 | -32 | -34.7 |

3.6GHz Beam2 reference [3]

| Index | delay(ns) | power(dB) | Cluster combined& renormalized |
| --- | --- | --- | --- |
| 1 | 0 | -24.8 | -27.8 |
| 2 | 76.6 | 0 | 0.0 |
| 3 | 79.4 | -11.4 | NA |
| 4 | 81 | -3.2 | NA |
| 5 | 85 | -3.6 | NA |
| 6 | 232.4 | -23.3 | -18.3 |
| 7 | 235.4 | -16.9 | NA |
| 8 | 239.4 | -23.5 | NA |
| 9 | 289.6 | -26.1 | -28.9 |
| 10 | 299.8 | -38.9 | NA |
| 11 | 448.4 | -25.1 | -28.1 |
| 12 | 477.5 | -25.8 | -28.8 |

Measurement results from Huawei’s lab:

  - Cross-polarization

Measurement results of cross-polarization are depicted in following table. During the test we noticed that for the two alternative measurement methods of H component [6], cited as below, significant different results were observed, with magnitude comparable to pass/failed limit e.g. 1dB [7]

- Alt1: Use a horizontally polarized (vertically oriented) magnetic loop dipole

- Alt2: a horizontally polarized sleeve dipole measured in four orthogonal horizontal positions and summed to measure the H component

Measurement results from Huawei’s lab: (with loop)

| Freq | Beam | Reference | Measured |
| --- | --- | --- | --- |
| UMa C, fc ≤ 2.5 GHz | Beam 1 | Input 1: V/H = -0.5 dB Input 2: V/H = 0.6 dB Input 1+ 2: V/H = 0 dB | Input 1: V/H = -0.98 dB Input 2: V/H = 0.63 dB Input 1+ 2: V/H = -0.21 dB |
|  | Beam 2 | Input 1: V/H = -0.6 dB Input 2: V/H = 0.7 dB Input 1+ 2: V/H = 0 dB | Input 1: V/H = -1.1dB Input 2: V/H = -0.07 dB Input 1+ 2: V/H = -0.60 dB |
| UMa C, fc > 2.5 GHz | Beam 1 | Input 1: V/H = -0.6 dB Input 2: V/H = 0.7 dB Input 1+ 2: V/H = 0 dB | Input 1: V/H = -0.45 Input 2: V/H = 1.32 dB Input 1+ 2: V/H = 0.39 dB |
|  | Beam 2 | Input 1: V/H = -0.7 dB Input 2: V/H = 0.8 dB Input 1+ 2: V/H = 0 dB | Input 1: V/H = 0.25 Input 2: V/H = 1.61 Input 1+ 2: V/H = 0.91 dB |

# Conclusions

For FR1 MIMO OTA Channel validation, in addition to Doppler/Temporal correlation and Spatial correlation data submitted in RAN4 #101e meeting [5], this document presents the validation results of remaining items i.e. Power Delay Profile (PDP) and Cross-polarization.

Observation 1: For FR1 MIMO OTA Channel validation, measurement results from Huawei’s lab meet the Pass/Fail limits proposed in [7] for all required metrics, i.e. Power Delay Profile (PDP), Doppler/Temporal data, Spatial correlation and Cross-polarization.

# Reference

- R4-2120684 WF on NR MIMO OTA vivo, CAICT

- R4-2115759 Reference Channel Emulation Curves for Validation Purposes, RAN4 100-e, Keysight Technologies UK Ltd, Spirent Communications, CMCC, CAICT

- R4-2119379 Beam Specific Reference Channel Emulation Curves for Validation Purposes for FR1 CDL-C Uma, RAN4#101-e, Keysight Technologies, Spirent Communications, CMCC, CAICT

- R4-2120683 Updated time plan for FR1 lab alignment and requirement, RAN4#101-e, CAICT, OPPO

- R4-2119541 Initial results on NR UE FR1 MIMO OTA channel model validation, RAN WG4 Meeting #101-e, Huawei, HiSilicon

- 3GPP TS 38.151 V0.6.0

- R4-2119093 Channel Model Spatial Validation Pass/Fail limits, RAN4#101-e, Spirent