3GPP TSG-RAN WG4 Meeting # 101-Bis-e												 R4-220xxxx

Electronic Meeting, January 17 – 25, 2022

Agenda item:			7.1

Source:	Moderator (Intel Corporation)

Title:	Email discussion summary for [101-bis-e][327] FR2_enhTestMethods

Document for:	Information

# Introduction

This document covers discussions of the Enhanced Test Methods in FR2 study item.

# Topic #1: Maintenance on objectives 1~6  (AI 7.1.1)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200452 TP to TR 38.884 on release independence applicability of test method enhancements | Apple | Text proposal to TR 38.884 to include a fourth applicability category for the applicability of release independence to FR2 test methodology enhancements. The text below is added to the General clause: "When the enhanced test methodology applies to a UE feature supported only from a specific release, the test method becomes applicable only from that release onwards." |

## Open issues summary

### Sub-topic 1-1: Text proposals for TR 38.884

Issue 1-1: TP on applicability of release independence

TP R4-2200452 introduces a fourth applicability category for the applicability of release independence to FR2 test methodology enhancements.

- Recommended WF

  - Moderator suggests companies provide any feedback on TP R4-2200452 directly into Section 1.3.2 CRs/TPs comments collection.

## Companies views’ collection for 1st round

### Open issues

Sub-topic 1-1: TP on applicability of release independence

Provide feedback for TP R4-2200452 to upcoming section 1.3.2

### CRs/TPs comments collection

Moderator suggests companies to comment directly for the CR below. in 1.3.2 CRs/TPs comment collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2200452 | MediaTek: Just a clarification question by example. Assume the “Rel-17 enhanced test methodology” requires “Rel-16 UE beam lock feature”, does it mean the “Rel-17 enhanced test methodology” only be applicable for “Rel-16 and beyond UE”, but NOT applicable for “Rel-15 UE”? If yes, in this case, can we further add “Rel-15 UE beam lock function” to make it applicable? |
|  | Apple: to respond to MediaTek, our understanding is that UE beam lock was introduced in Rel-15 and was one of the first aspects captured into TR38.810. Does this help to clarify? |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1: TP for TR 38.884 | Sub-topic 1-1: TP on applicability of release independence There was a question for clarification about UE beam lock feature, but no views against text proposal R4-2200452. Recommended WF: Confirm whether the TP is agreeable |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round

TBA

## Companies views’ collection for 2nd round

### Open issues

Confirm whether TP is agreeable.

## Summary for 2nd round

### Open issues

|  | Status summary |
| --- | --- |
| Issue 1-1 | TBA |

# Topic #2: OTA test methods for UE RF, RRM and demodulation for 52.6~71GHz  (AI 7.1.2)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200907 On permitted test methods for demodulation in FR2-2 | Apple | Observation 1: Atmospheric attenuation has no impact on path loss at the range scales of an FR2 testing chamber Observation 2: For 71 GHz and reference range length of 0.725m the path loss is 66.7 dB Observation 3: For 71 GHz the cable loss per meter is 10.3 dB Observation 4: The values of probe antenna gain and backoff from P1dB need to be further checked with test equipment vendors to verify their applicability to band n263 Observation 5: In general, a simple extension of the permitted methods up to FR2-2 does not appear to yield a system capable of achieving a sufficient SNR for demodulation testing. Proposal 1: Further discussion on how to achieve testable SNR ranges for band n263 is needed. |
| R4-2201873 TP to TR 38.884 on extension of NR test methods to FR2-2 | Intel Corporation | Text proposal to TR 38.884 on the extension of test methods to FR2-2 covers the following: 1) UE RF permitted test methods (general aspects) 2) UE RF testing methodology enhancements (sub-clause headings) 3) RRM testing methodology enhancements 4) UE Demodulation testing methodology enhancements 5) Propagation conditions |
| R4-2201874 NR FR2-2 OTA test methods for RRM | Intel Corporation | Proposal 1: All test methods and measurement setup for FR2 RRM methodology defined in TR 38.810 [3] Clause 6 are applicable for FR2-2, except for Noc derivation (TR 38.810 - Clause 6.2.1.4.3) and Maximum SNR derivation (TR 38.810 - Annex B.2) Proposal 2: For FR2-2 RRM testing the Noc level is derived similar to the Noc level for UE demodulation test methods Proposal 3: Perform an informative assessment of testable RRM DL SNR range for FR2-2 for maximum frequency (~71GHz) using TR38.810 methodology as starting point. |
| R4-2201875 NR FR2-2 OTA test methods for UE demodulation | Intel Corporation | Proposal 1: For analysis of FR2-2 test methodology definition for multi-path fading channels consider TDL-A channel model with RMS delay spread is in range of 5-20ns and with 3km/h UE mobility. Proposal 2: For multi-path fading channel modelling use Fsample = 2000MHz for channel bandwidth up to 2000MHz as long the value is feasible from TE implementation perspective. Proposal 3: Define Noc levels for FR2-2 UE demodulation testing only based on the following definition: - Noc (PC_X, Band_Y) = RESFENSPCX, BandY -10log10(SCSREFSENS x PRBREFSENS x 12) - SNRREFSENS + ∆thermal Proposal 4: Consider parameters from Table 2 for Noc level definition. Proposal 5: Consider DNF, DFF, and IFF test methods and 100, 400, 1600, and 2000 MHz CBWs for maximum testable SNR derivation. Proposal 6: Ask inputs from TE vendors on the following test system parameters to derive max testable SNR: - TE amplifier 1dB compression, dBm; - Backoff from 1dB compression, dB; - Cable loss, dB; - Free space path loss, dB; - TE DL absolute power setting uncertainty, dB; - Probe antenna gain, dB; - Beam peak search procedure/measurement error, dB. |
| R4-2201921 On Vehicular UEs | Keysight Technologies UK Ltd | Proposal 1: Avoid developing a standardized ground plane for FR2 vehicular UEs as part of this SI Proposal 2: Consider the optional ground plane designed and manufactured by the OEM an integral part of the FR2 vehicular UE DUT submitted for conformance testing. Proposal 3: Pending feedback from vehicular UE OEMs, consider battery and AC/DC powered operation acceptable for FR2 vehicular UEs and request manufacturers to provide proper guidance on cable routings. |
| R4-2201927 UE types and permitted methods for FR2-2 | Intel Corporation | UE types Observation 1: Measurement uncertainty analysis will focus on handheld UEs in Rel-17 and further discussion on its array size is needed before starting the analysis. PC3 antenna array Observation 2: For handheld UE in FR2-1, core requirements assumed a baseline 4x1 array, while the testability/MU analyses used an 8x2 array. For FR2-2, array size discussions for core requirements are currently ongoing and may result in a larger sized array. Proposal 1: RAN4 should discuss feasible worst-case array sizes for both an 8-element and 16-element array assumption. Proposal 2: If no conclusion is found for the array size of PC3 in this meeting, derive MU based on two worst-case sizes. Applicability of permitted test methods Observation 3: Besides updating frequency-dependent parameters and relevant assumptions, the permitted RF test methods (clause 5.2) can be extended to FR2-2. Proposal 3: Unless otherwise stated, FR2-2 will follow the baseline UE RF methodology detailed in TR 38.810. MU assessment will be revised to reflect proper frequency-dependent parameters and worst-case array size. Applicability of test methodology enhancements Observation 4: Some methodology enhancement techniques for test time reduction (Objective 5) can be extended to FR2-2. Objective 3 should be postponed until these core requirements are discussed and defined. Objectives 1 and 4 need more discussion. Proposal 4: Applicability of methodology enhancements of three methods in Objective 5 can be extended to FR2-2. Objective 3 discussions should be postponed until core requirements are discussed. Lastly, we should further discuss the remaining objectives. |
| R4-2201990 Issues with MIMO EVM Measurement Using the Pseudo-Inverse | Lenovo, Motorola Mobility | Proposal 1: If H ̃ has full rank, then G_ZF=H ̃^(-1). If H ̃ has does not have full rank, the two layers cannot be separated and the EVM requirement is failed. A text proposal for Section 5.2.3.1.1.2 of TR38.884-120 implementing Proposal 1 for Method 1 MIMO equalization is given in the Appendix below. Proposal 2: Agree to the text proposal for Section 5.2.3.1.1.2 of TR38.884-120 in the Appendix. |

## Open issues summary

### Sub-topic 2-1: UE types

Issue 2-1a: Vehicular UE – ground plane

In RAN4 #101e, a vehicular UE was defined to be the combination of OBU/TCU + antenna + optional ground plane [R4-2120767]. Discussions for this UE type included UE architecture and ground plane details, and companies were encouraged to share their views in this meeting.

- Proposal 1: Avoid developing a standardized ground plane for FR2 vehicular UEs as part of this SI (Keysight, R4-2201921)

- Proposal 2: Consider the optional ground plane designed and manufactured by the OEM an integral part of the FR2 vehicular UE DUT submitted for conformance testing. (Keysight, R4-2201921)

- Recommended WF

  - Moderator suggests companies share their views on whether Proposal 1 and Proposal 2 are agreeable. Additional feedback on ground plane is welcomed.

Issue 2-1b: Vehicular UE – power supply and routings

- Proposal 3: Pending feedback from vehicular UE OEMs, consider battery and AC/DC powered operation acceptable for FR2 vehicular UEs and request manufacturers to provide proper guidance on cable routings. (Keysight, R4-2201921)

- Recommended WF

  - Moderator suggests companies provide their views on Proposal 3 and give guidance on cable routings.

Issue 2-1c: Handheld UE – worst-case antenna array size

FR2-2 array size discussions for PC3 core requirements are currently ongoing and may result in a larger sized array than the baseline 4x1 used in FR2-1. Currently, the array sizes being considered include: 4, 8 and 16 – elements. To make progress in our discussions, we should address what would be feasible worst-case array sizes for an 8-element array and a 16-element array.

- Proposal 1: RAN4 should discuss feasible worst-case array sizes for both an 8-element and 16-element array assumption. (Intel, R4-2201927)

- Recommended WF

  - Companies share their views on feasible worst-case sizes for a handheld UE with an 8-element and 16-element antenna array assumption.

Issue 2-1d: Handheld UE – MU assessment

Given the study item’s timeline, if no consensus on the array size assumption of PC3 is reached during this meeting, we may proceed with MU derivations based on two separate worst-case sizes:

- 8x2

- Larger than 16-elements worst-case (suitable for either an 8-element or larger core array assumption)

- Proposal 2: If no conclusion is found for the array size of PC3 in this meeting, derive MU based on two worst-case sizes. (Intel, R4-2201927)

- Recommended WF

  - Companies provide their views on Proposal 2 and other options to make progress during this meeting.

### Sub-topic 2-2: Test methodology for UE RF

Issue 2-2a: MIMO EVM Measurement

In R4-2201990 an issue of EVM measurement for two-layer Tx is identified. Basically, using pseudo-inverse matrix in zero-forcing equalization does not allow to separate MIMO layers and hence correctly measure EVM. The following observations and proposals were made:

- Observation 1:	If a square matrix H has full rank, then H+=H-1, and the pseudo-inverse is not needed. (Lenovo, Motorola Mobility)

- Observation 2:	If a square matrix H does not have full rank, then HHH does not have full rank and the inverse HHH-1does not exist.  As a result, if H ̃ does not have full rank, then the pseudo-inverse cannot be defined or computed as

H+=HHH-1HH

So, if the pseudo-inverse H+were to be used, it would be necessary to define it in some other manner such as in terms of the singular value decomposition of H. (Lenovo, Motorola Mobility)

- Observation 3:  If the square matrix H  does not have full rank, then it is not possible to separate the MIMO layers using the pseudo-inverse or any other linear receiver and the EVM requirement will be failed. (Lenovo, Motorola Mobility)

- Observation 4: 	If H does not have full rank and GZF=H+is used, the resulting EVM will be no less than 71%. (Lenovo, Motorola Mobility)

- Proposal 1:  If H has full rank, then GZF=H-1. If H has does not have full rank, the two layers cannot be separated and the EVM requirement is failed. (Lenovo, Motorola Mobility)

- Proposal 2:	Agree to the text proposal for Section 5.2.3.1.1.2 of TR38.884-120 in the Appendix. (Lenovo, Motorola Mobility)

- Recommended WF

  - Companies are encouraged to provide their view on necessity of EVM measurement methodology update and suggested text proposal for TR38.884.

Issue 2-2b: Permitted test methods

With the relevant frequency-dependent parameters updated, the UE RF testing methodology and permitted test methods in TR 38.810 can be extended to FR2-2. As the editor’s note in clause 5 of TR 38.884 states, the baseline UE RF methodology includes testing and calibration of permitted methods along with a preliminary assessment of measurement uncertainty (detailed in Annex B).

- Proposal: Unless otherwise stated, FR2-2 will follow the baseline UE RF methodology detailed in TR 38.810. MU assessment will be revised to reflect proper frequency-dependent parameters and worst-case array size. (Intel, R4-2201927)

- Recommended WF

  - Moderator suggests companies share their views on whether the above Proposal is agreeable, and if additional aspects need to be considered to extend applicability of permitted test methods to FR2-2.

Issue 2-2c: Enhanced test methodology

The test methodology enhancements captured in Objectives 1 through 5 of this study also need to be evaluated. We previously agreed that the enhancements content found in TR 38.884 would be used as baseline to start the FR2-2 applicability assessment. Furthermore, test time reduction methods have been discussed and agreed to be extended to FR2-2 [R4-2115767].

- Proposal 1: At least, RSRPB based Rx beam peak search, Single link polarization measurement and Fast Spherical Coverage Method can be applied to 52.6-71GHz directly. (vivo)

Agreement: Proposal 1 is agreed.

- Proposal: Applicability of methodology enhancements of three methods in Objective 5 can be extended to FR2-2. Objective 3 discussions should be postponed until core requirements are discussed. Lastly, we should further discuss the remaining objectives.

- Recommended WF

  - Moderator suggests companies share their view on the Proposal above, particularly postponing Objective 3 discussions. Additionally, companies can provide their input on whether an enhancement can be extended to FR2-2 or needs to be assessed and further discussed (use Objectives below for inputs).

    - Objective 1 – methodology for high DL power and low UL power test cases

    - Objective 2 – solutions to minimize the impact of polarization basis mismatch

    - Objective 4 –extreme temperature conditions for all applicable FR2 UE RF test cases

    - Objective 5 – test time reduction

### Sub-topic 2-3: Test methodology for RRM

Issue 2-3a: FR2-1 RRM test methodology reuse

- Proposal 1: All test methods and measurement setup for FR2 RRM methodology defined in TR 38.810 [3] Clause 6 are applicable for FR2-2, except for Noc derivation (TR 38.810 - Clause 6.2.1.4.3) and Maximum SNR derivation (TR 38.810 - Annex B.2) (Intel)

- Recommended WF

  - Companies are encouraged to provide their view on the above proposal.

Issue 2-3b: Noc level derivation

In RAN4 #101e Noc level derivation for UE demodulation test methods were discussed.

- Proposal 1: For FR2-2 RRM testing the Noc level is derived similar to the Noc level for UE demodulation test methods (Intel)

- Recommended WF

  - Companies are encouraged to share their views on whether Noc level derivation for RRM testing can be reused from demodulation testing methodology for FR2-2.

Issue 2-3c: Informative assessment of the testable RRM DL SNR range

In RAN4 #101e it was agreed to provide informative assessment on max testable DL SNR for FR2-2 for UE demodulation test methods.

- Proposal 1: Perform an informative assessment of testable RRM DL SNR range for FR2-2 for maximum frequency (~71GHz) using TR38.810 methodology as starting point (Intel)

- Recommended WF

  - Companies are encouraged to provide their view on the above proposal.

### Sub-topic 2-4: Test methodology for UE demodulation and CSI

Issue 2-4a: Multi-path fading channel model

Captured in R4-2120767

Agreement: Define methodology for multi-path fading and static propagation conditions modelling for FR2-2 - Reuse FR2-1 static propagation conditions methodology for FR2-2 - Channel model parameters i.e. delay spread and Doppler spread need to be defined firstly.

- Proposal 1: For analysis of FR2-2 test methodology definition for multi-path fading channels consider TDL-A channel model with RMS delay spread is in range of 5-20ns and with 3km/h UE mobility (Intel)

- Recommended WF

  - Moderator suggests discussing the proposed channel model for further FR2-2 analysis.

Issue 2-4b: Path delay grid

Captured in R4-2120767

Agreement: For multi-path fading channel modelling, further discuss Fsample value with candidate options as following - Option 1: 2000MHz - Option 2: 800MHz - Option 3: 400MHz - Other options not precluded

- Proposal 1: For multi-path fading channel modelling use Fsample = 2000MHz for channel bandwidth up to 2000MHz as long the value is feasible from TE implementation perspective. (Intel)

- Recommended WF

  - Companies are encouraged to provide their view on the proposed 2000MHz sampling frequency and 2000MHz channel bandwidth for multi-path fading channel modelling.

Issue 2-4c: Noc level derivation

Captured in R4-2120767

Agreement: Define Noc levels for FR2-2 UE demodulation testing based on FR2-1 methodology: - Noc (PC_X, Band_Y) = RESFENSPCX, BandY -10log10(SCSREFSENS x PRBREFSENS x 12) - SNRREFSENS + ∆thermal - FFS: Noc(PC_X, Band_Y) = -155 dBm/Hz + REFSENSPC_X, Band_Y, 100MHz – REFSENSPC3, n260, 100MHz - Note: Further confirmation of used parameters is needed based on core requirements definition.

- Proposal 1: Define Noc levels for FR2-2 UE demodulation testing only based on the following definition:

Noc (PC_X, Band_Y) = RESFENSPCX, BandY -10log10(SCSREFSENS x PRBREFSENS x 12) - SNRREFSENS + ∆thermal (Intel)

- Proposal 2: Consider parameters from Table 2 for Noc level definition. (Intel)

| Parameter | Clarification/Value |
| --- | --- |
| REFSENSPCX, BandY | The REFSENS value in dBm specified for Power Class X UE in band Y |
| CBW for REFSENS | 100 MHz |
| SCSREFSENS | 120 kHz |
| PRBREFSENS | NRB associated with subcarrier spacing SCSREFSENS for channel bandwidth used for REFSENS calcualtion |
| SNRREFSENS | -1 dB |
| ∆thermal | An amount of dB that the wanted noise is set above UE thermal noise, giving a rise in total noise of ∆BB. ∆thermal = 6dB, giving a rise in total noise of 1dB |

- Recommended WF

  - Companies are encouraged to provide their view whether proposed and only proposed equation can be used for Noc level derivation for FR2-2.

  - Companies are encouraged to provide their view on the proposed parameters.

Issue 2-4d: Informative assessment of the testable Demodulation DL SNR range

Captured in R4-2120767

Agreement: RAN4 to perform an informative assessment of testable DL SNR range for FR2-2 for maximum frequency (~71GHz) using TR38.810 methodology as starting point. - Derivation of max testable SNR for other portions of FR2-2 range may be further performed - Further refinement on the test methodology from TR 38.810 not precluded

- General methodology

  - Observation 1: Atmospheric attenuation has no impact on path loss at the range scales of an FR2 testing chamber (Apple)

  - Observation 2:	 In general, a simple extension of the permitted methods up to FR2-2 does not appear to yield a system capable of achieving a sufficient SNR for demodulation testing. (Apple)

  - Proposal 1: Further discussion on how to achieve testable SNR ranges for band n263 is needed. (Apple)

- Test methods and CBWs to be considered

  - Proposal 1: Consider DNF, DFF, and IFF test methods and 100, 400, 800, 1600, and 2000 MHz CBWs for maximum testable SNR derivation (Intel)

- Required parameters for assessment

  - Observation 1: For 71 GHz and reference range length of 0.725m the path loss is 66.7 dB (Apple)

  - Observation 2:	 For 71 GHz the cable loss per meter is 10.3 dB (Apple)

  - Observation 3:	 The values of probe antenna gain and backoff from P1dB need to be further checked with test equipment vendors to verify their applicability to band n263 (Apple)

  - Proposal 1: Ask inputs from TE vendors on the following test system parameters to derive max testable SNR: (Intel)

    - TE amplifier 1dB compression, dBm;

    - backoff from 1dB compression, dB;

    - Cable loss, dB;

    - Free space path loss, dB;

    - TE DL absolute power setting uncertainty, dB;

    - Probe antenna gain, dB;

    - Beam peak search procedure/measurement error, dB.

- Recommended WF

  - Companies are encouraged to discuss observations made for SNR calculation methodology for FR2-2 and provide their views on the proposed test methods and CBWs to be considered.

  - TE vendors are encouraged to provide their view on the required test system parameters.

### Sub-topic 2-5: Text proposals for TR 38.884

Issue 2-5: TP on test methods extension to FR2-2

TP R4-2201873 on the extension of test methods to FR2-2 covers the following:

- UE RF permitted test methods (general aspects)

- UE RF testing methodology enhancements (sub-clause headings)

- RRM testing methodology enhancements

- UE Demodulation testing methodology enhancements

- Propagation conditions

- Recommended WF

  - Moderator suggests companies provide any feedback on TP R4-2201873 directly into Section 2.3.2 CRs/TPs comments collection.

## Companies views’ collection for 1st round

### Open issues

Sub-topic 2-1: UE types

Issue 2-1a: Vehicular UE – ground plane

Issue 2-1b: Vehicular UE – power supply and routings

Issue 2-1c: Handheld UE - worst-case antenna array assumption

Issue 2-1d: Handheld UE – MU assessment

| Company | Comments |
| --- | --- |
| Rohde & Schwarz | Issue 2-1a: Vehicular UE – ground plane Although the standardized ground plane would be desirable to ensure repeatability of results among test systems and UE’s, we understand and share the concerns that drive into Proposals 1 and 2. We agree to them, although we think it’s important to ensure the overall setup including the optional ground plane is limited in size such it can be contained and tested within one of the available quiet zone sizes (i.e. 20, 30, 40 and 55cm). Issue 2-1b: Vehicular UE – power supply and routings We agree to proposal 3. |
| LGE | We agree the proposals 1 and 2. Ground plane shape and size can have significant impact on number of the OTA performance parameters and impact may be smaller or larger in different test cases. The impact depends a lot on the solution (DUT) and therefore the OEM has the best knowledge on how to take this into account for conformance testing. We also agree that available quiet zone size should be used. We also agree the proposal 3. Guidance on cable routings is dependent on the solution and OEM has the best understanding on this area on their solution. We also agree the Keysight observation that interference due to cable currents is easier to manage at higher frequencies than in OTA testing below 6GHz and ground plane can be used to mitigate impacts from cables needed to power the DUT during OTA testing. |
| MediaTek | Issue 2-1a: Vehicular UE – ground plane We share similar view that to have clear “applicable quiet zone“ would be the key. Issue 2-1b: Vehicular UE – power supply and routings The proposal is made sense for us. Issue 2-1c: Handheld UE - worst-case antenna array assumption Maybe wait for core requirement discussion about array element Issue 2-1d: Handheld UE – MU assessment Maybe wait for core requirement discussion about array element |
| Samsung | Issue 2-1c: Handheld UE - worst-case antenna array assumption Prefer waiting for core requirements assumption, or reusing 8x2 given no more than 16 elements under discussion in core session. Issue 2-1d: Handheld UE – MU assessment As worst case array size, we think only one worst case is enough |
| vivo | Issue 2-1a: Vehicular UE – ground plane Support proposal 1 and proposal 2. Issue 2-1c: Handheld UE - worst-case antenna array assumption Also prefer waiting the outcome from main session on FR2-2 antenna assumption. And avoid repeating the history of Rel-15 that defining 2x8 as the worst-case assumption of actually 1x4 UE antenna. More discussion is needed. Issue 2-1d: Handheld UE – MU assessment Share similar view with Samsung, the MU of each system should be single value. In addition, it should be noted that RAN4 should avoid defining two set of sampling grids for two worst-case sizes. |
| Apple | Issue 2-1c: Handheld UE - worst-case antenna array assumption Our preference is to let RAN4 RF to conclude the antena array assumption used in the requirement definition before we jump into MU assessments. However, in an effort to make progress, we are also fine to consider informational example MU assessments performed according to the full set of proposed antenna array assumptions (case 1: based on 4 elements used for requirement derivation / 2x8 for MU assessment; case 2: based on 8 elements for requirment / TBD for MU; case 3: based on 16 eleemnts for requirement / TBD for MU) Issue 2-1d: Handheld UE – MU assessment Same comment as above |
| Qualcomm | Issue 2-1c: For a test method discussion, there is limited motivation to limit number of elements. There are already proposals for 16e designs in the core discussion (2x8), so that would automatically qualify for a ‘worst-case antenna array size’. It can be increased further for a degree of future-proofing. Issue 2-2a: Perhaps Motorola could clarify why we need to establish procedure when H is not full rank. In our understanding, |
| Intel | Issue 2-1a: Vehicular UE – ground plane Both proposals are reasonable. We also echo the importance of the setup and QZ sizes. Issue 2-1c and 2-1d: Handheld UE – worst-case antenna array assumption and MU assessment Similar view as Apple. While we prefer to wait for the conclusion of the core requirement discussion, for the sake of progress, we can discuss the option of assessing MU for different array sizes (based on the assumptions being considered in core discussions). |

Sub-topic 2-2: Test methodology for UE RF

Issue 2-2a: MIMO EVM Measurement

Issue 2-2b: Permitted test methods

Issue 2-2c: Enhanced test methodology

| Company | Comments |
| --- | --- |
| Rohde & Schwarz | Issue 2-2a: From our point of view, we don’t agree with changing to the inverse matrix as proposed by Lenovo. In the test the UEs are configured for using uplink MIMO and thus the matrix will always have a full rank. So while in this case it would be possible to use the inverse matrix, we also need to cover the 2x1 case in FR2, the polarization misalignment (which was the original issue for this enhancement). In this case, using the inverse matrix does not work and the pseudo inverse must be used. So we shall keep the pseuso-inverse also for the 2x2. We agree with the statement that in case the UE does not use MIMO transmission in the UL MIMO TC, the TC should fail. Issue 2-2c: As agreed during last meeting, Objective 1 depends on whether RAN5 identifies the same issues with high DL/low UL power and whether NF based solutions are required to overcome relaxations identified for IFF/DFF, but we can assume the same list of high DL and low UL test cases identified for FR2-1. Objective 2, the solutions from TR 38.810 can be applied to FR2-2, currently we do not see any need for further investigation. Objective 5, the solutions from TR 38.810 should be applicable to FR2-2 as well. |
| Lenovo, Motorola Mobility | Issue 2-2a: To R&S: We are not sure of the relevance of the 2x1 case (H is 2x1 and 1 layer) to the MIMO case (H is 2x2 and 2 layers). Above you say “the UEs are configured for using uplink MIMO and thus the matrix will always have a full rank,” and this makes sense to us. With this condition, it is always true that HHH-1HH=H-1. So why not just use H-1 for uplink MIMO? It changes nothing at all for the 2x1 case. This text proposal is only for “Section 5.2.3.1.1.3 Method 1 MIMO Equalization” and does not touch any other section. Specifically, it does not touch “Section 5.2.3.1.1.3 Method 1 Maximum Ratio Combining” which still contains the pseudo-inverse. |
| MediaTek | Issue 2-2c: Enhanced test methodology We echo Rohde & Schwarz’s below comments: “Objective 2, the solutions from TR 38.810 can be applied to FR2-2, currently we do not see any need for further investigation. “ “Objective 5, the solutions from TR 38.810 should be applicable to FR2-2 as well.” |
| Samsung | Issue 2-2c: Enhanced test methodology Agree to postpone objective 3. Generally we are fine with moderator’s recommendation, especially for objective 2 and 5. |
| vivo | Issue 2-2b: Permitted test methods Agree with the proposal. Issue 2-2c: Enhanced test methodology More feasibility discussion is needed for Objective 1. For Objective 4, we also need to wait the conclusion from main session on FR2 ETC core requirement discussion. |
| Intel | Issue 2-2a TR 38.884 defines enhanced methods for EVM measurement for 2x2 and 2x1 channel matrices. Same time proposed direct matrix inverse is applicable only for square matrices. In this case, how to cope with 2x1 configurations is not clear for us. Besides that, can companies clarify what is the reason to matrix to be not a full rank? In our understanding, the static channel model that always has full rank is considered for EVM measurements. |

Sub-topic 2-3: Test methodology for UE RRM

Issue 2-3a: FR2-1 RRM test methodology reuse

Issue 2-3b: Noc level derivation

Issue 2-3c: Informative assessment of the testable RRM DL SNR range

| Company | Comments |
| --- | --- |
| Rohde & Schwarz | Issue 2-3a: FR2-1 RRM test methodology reuse In order to ensure completeness, and as detailed in the SID, test methods in TS 38.508-1 should also be mentioned in order to cover the additional RRM test setups identified in RAN5 (i.e. Enhanced IFF and IFF+DFF Hybrid test setups). |
| Qualcomm | Issue 2-3a: In general, we agree with the proposal 1. Just to point it out, two modes, i.e., signal + noise, signal are considered for RRM testing. So we need to consider both two modes for FR2-2. Issue 2-3b: Agree with proposal 1. Issue 2-3c: OK with proposal 1 |

Sub-topic 2-4: Test methodology for UE demodulation and CSI

Issue 2-4a: Multi-path fading channel model

Issue 2-4b: Path delay grid

Issue 2-4c: Noc level derivation

Issue 2-4d: Informative assessment of the testable Demodulation DL SNR range

| Company | Comments |
| --- | --- |
| Rohde & Schwarz | Issue 2-4a: We can use the proposal as a starting point. In case issues are observed with this starting channel model, we need to discuss if a different model is better suited. Issue 2-4b: As already commented in the last meeting we don’t agree with using 2 GHz. We suggest to focus on either 400 or 800 MHz, since we will have better SNR testing with smaller bandwidth and also the 2 GHz bandwidth are only optional, so UEs may not even support it. Issue 2-4c: The proposal seems to be ok. Given that the available SNR range is still unknown and may be quite limited, it may be necessary to revisit some values, e.g. ∆thermal since lowering the Noc level would improve the SNR. Issue 2-4d: While in the past we have derived the SNR based on all the parameters listed by Intel, we would prefer to discuss based on the achievable DL level (CW) in the center of the QZ. This value is also used by RAN5 as a reference, since different implementations lead to different cable length, antennas, amplifiers, but in the end result in similar DL levels. For backoff from P1dB we suggest to go with the latest RAN5 agreement for FR2-1 as a starting point. Since the current reference implementation for all test systems in the market in FR2-1 is the IFF implementation, we suggest to base the estimation of DL SNR on this system implementation. |
| Samsung | Issue 2-4c: Noc level derivation Support proposal 1. Proposal 2 is also reasonable, and also agree with R&S that reducing ∆thermal could improve SNR range, but on the other hand it would also increase uncerntity. Issue 2-4d: Informative assessment of the testable Demodulation DL SNR range Thanks for the observations and proposals. Based on that it seems the achievable SNR range is not useable if deriving based on parameters used for FR2-1. When the CBW comes higher, the SNR issue is even worse. |
| Qualcomm | Issue 2-4a: OK with option 1 as the starting point. It can be revisited if any issues are identified later. Issue 2-4b: Prefer Option 1 to avoid the potential impact on the performance with 2GHz CBW. But we understand the concerns from TE vendors. Considering the limited SNR range for FR2-2, we might have to reduce the CBW. Then Fsaming can be decreased. Issue 2-4c: Agree with Samsung. Reducing the ∆thermal will lead to uncertainty. Issue 2-4d: The reference range length of 0.725m assuming carrier is up to 43GHz. The reference length of 0.725m can be kept only for IFF. CBW should be reduced to reach to achieve higher SNR. P1 backoff should follow the latest conclusion in RAN5. |
| Intel | Issue 2-4a TDL-A channel model with 5-20ns delay spread and 3km/h UE speed was considered as a baseline during RAN4 study. We suggest considering the same assumptions for FR2-2 analysis in RAN4. Issue 2-4b 2GHz CBW is optional to be support but requirement definition is not precluded for it. Therefore, RAN4 should ensure that defined channel models are applicable up to 2GHz CBW. For 2GHz CBW 2GHz sampling frequency for path delay grid definition can be a good trade-off between accuracy of channel model definition and TE complexity. To move forward we can decouple two issues as: 1) Max applicable CBW for FR2-2 channel models; 2) Sampling frequency for path-delay grid definition. For the first one, as we explained above, we propose considering 2GHz. As for sampling frequency, we would like to understand TE vendors views which sampling frequency is feasible from TE design perspective. Issue 2-4d For R&S proposal on using DL CW power in center of QZ, for FR2-2 calculations we prefer to have some consistence with DL SNR calculation methodology used for FR2-1. Even if DL CW power will be used in final SNR derivation, some background on assumptions for TE amplifier 1dB compression”, “Cable loss”, “FS path loss” and “Probe antenna gain” can be added. Also, if RAN4 is going to perform assessment only for the limited number of methods like 1-2, then there is no problem to use conventional FR2-2 methodology directly. |

### CRs/TPs comments collection

Moderator suggests companies to comment directly for the CR below. in 1.3.2 CRs/TPs comment collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2201873 | Rohde & Schwarz: Similar comments as for the open issues above. For RRM, we propose to add TS 38.508-1 as a reference into section 6.2.1 so all current RRM methodologies are covered. For Demod SNR we propose to focus on IFF, since this is the commonly used test setup at the moment. Also in table 7.2.3-1 we suggest to remove “TE amplifier 1dB compression”, “Cable loss”, “FS path loss” and “Probe antenna gain” and add “DL CW power in center of QZ” instead, as also commented in issue 2-4d |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #2-1: UE types | Issue 2-1a: Vehicular UE – ground plane - Proposal 1: Avoid developing a standardized ground plane for FR2 vehicular UEs as part of this SI - Proposal 2: Consider the optional ground plane designed and manufactured by the OEM an integral part of the FR2 vehicular UE DUT submitted for conformance testing. Tentative agreement: Approve Proposal 1 and Proposal 2 Issue 2-1b: Vehicular UE – power supply and routings - Proposal 3: Pending feedback from vehicular UE OEMs, consider battery and AC/DC powered operation acceptable for FR2 vehicular UEs and request manufacturers to provide proper guidance on cable routings. Tentative agreement: Approve Proposal 3 Recommendations for 2nd round: Continue discussion in 2nd round if OEMs can provide additional feedback Issue 2-1c: Handheld UE – worst-case antenna array size Companies provided their views on what is currently feasible to consider as the worst-case array size. The majority reiterated their preference to wait for the core requirement discussion to conclude this issue. Note that in these discussions 4, 8 and 16 antenna elements are being considered for PC3. Recommendations for 2nd round: Wait for the outcome of GTW core discussion and assess whether we can choose the worst-case array size with the available information. Issue 2-1d: Handheld UE – MU assessment The possibility of having MU assessments for more than one array size. Recommendations for 2nd round: Wait for the outcome of GTW core discussion. If there is no conclusion, discuss how we can make progress on MU in this meeting. |
| Sub-topic #2-2: Test Methods for UE RF | Issue 2-2a: MIMO EVM Measurement - Proposal 1: If H has full rank, then GZF=H-1. If H has does not have full rank, the two layers cannot be separated and the EVM requirement is failed. (Lenovo, Motorola Mobility) - Proposal 2: Agree to the text proposal for Section 5.2.3.1.1.2 of TR38.884-120 in the Appendix. (Lenovo, Motorola Mobility) More discussion is needed for Proposal 1 and should clarify whether the matrix is full rank. Recommended WF: Continue the discussion in round 2 Issue 2-2b: Permitted test methods Candidate option: - Proposal: Unless otherwise stated, FR2-2 will follow the baseline UE RF methodology detailed in TR 38.810. MU assessment will be revised to reflect proper frequency-dependent parameters and worst-case array size. Tentative agreement: Approve the proposal Recommendations for 2nd round: Confirm tentative agreement Issue 2-2c: Enhanced test methodology Candidate option: - Proposal: Applicability of methodology enhancements of three methods in Objective 5 can be extended to FR2-2. Objective 3 discussions should be postponed until core requirements are discussed. Lastly, we should further discuss the remaining objectives. Tentative agreement: Approve the proposal and extend applicability of Objective 2 and Objective 5 solutions to FR2-2. Recommendations for 2nd round: Confirm tentative agreement |
| Sub-topic #2-3: Test Methods for RRM | Issue 2-3a: FR2-1 RRM test methodology reuse Candidate option: - Proposal 1: All test methods and measurement setup for FR2 RRM methodology defined in TR 38.810 [3] Clause 6 are applicable for FR2-2, except for Noc derivation (TR 38.810 - Clause 6.2.1.4.3) and Maximum SNR derivation (TR 38.810 - Annex B.2) Recommendations for 2nd round: Add test methods in TS 38.508-1 to Proposal 1 content and work on the text that will be captured in the TP revision of R4-2201873. Issue 2-3b: Noc level derivation Candidate option: - Proposal 1: For FR2-2 RRM testing the Noc level is derived similar to the Noc level for UE demodulation test methods Tentative agreement: Approve the proposal Recommendations for 2nd round: Confirm tentative agreement Issue 2-3c: Informative assessment of the testable RRM DL SNR range Candidate option: - Proposal 1: Perform an informative assessment of testable RRM DL SNR range for FR2-2 for maximum frequency (~71GHz) using TR38.810 methodology as starting point Tentative agreement: Approve the proposal Recommendations for 2nd round: Confirm tentative agreement |
| Sub-topic #2-4: Test Methods for UE demodulation and CSI | Issue 2-4a: Multi-path fading channel model Candidate option: - For analysis of FR2-2 test methodology definition for multi-path fading channels consider TDL-A channel model with RMS delay spread is in range of 5-20ns and with 3km/h UE mobility. Tentative agreement: Consider TDL-A channel model with RMS delay spread as in range of 5-20ns and with 3km/h UE mobility as starting point for the analysis. Recommendations for 2nd round: Provide comments and edits for the tentative agreement. Issue 2-4b: Path delay grid Candidate option: - For multi-path fading channel modelling use Fsample = 2000MHz for channel bandwidth up to 2000MHz as long the value is feasible from TE implementation perspective Tentative agreement: - Max applicable channel bandwidth: - Option 1: 2GHz - Option 2: Smaller than 2GHz - Sampling frequency: - Option 1: 2GHz - Option 2: 800/400MHz Recommendations for 2nd round: Further discuss max applicable channel bandwidth and sampling frequency for channel model definition. Issue 2-4c: Noc level derivation Candidate option: - Define Noc levels for FR2-2 UE demodulation testing only based on the following definition: Noc (PC_X, Band_Y) = RESFENSPCX, BandY -10log10(SCSREFSENS x PRBREFSENS x 12) - SNRREFSENS + ∆thermal - Consider parameters from Table 2 for Noc level definition Parameter; Clarification/Value. REFSENSPCX, BandY; The REFSENS value in dBm specified for Power Class X UE in band Y. CBW for REFSENS; 100 MHz. SCSREFSENS; 120 kHz. PRBREFSENS; NRB associated with subcarrier spacing SCSREFSENS for channel bandwidth used for REFSENS calcualtion. SNRREFSENS; -1 dB. ∆thermal; An amount of dB that the wanted noise is set above UE thermal noise, giving a rise in total noise of ∆BB. ∆thermal = 6dB, giving a rise in total noise of 1dB. Tentative agreement: - Define Noc levels for FR2-2 UE demodulation testing only based on the following definition: Noc (PC_X, Band_Y) = RESFENSPCX, BandY -10log10(SCSREFSENS x PRBREFSENS x 12) - SNRREFSENS + ∆thermal - Consider the following parameters for Noc level definition: Parameter; Clarification/Value. REFSENSPCX, BandY; The REFSENS value in dBm specified for Power Class X UE in band Y. CBW for REFSENS; 100 MHz. SCSREFSENS; 120 kHz. PRBREFSENS; NRB associated with subcarrier spacing SCSREFSENS for channel bandwidth used for REFSENS calcualtion. SNRREFSENS; -1 dB. ∆thermal; An amount of dB that the wanted noise is set above UE thermal noise, giving a rise in total noise of ∆BB. ∆thermal = 6dB, giving a rise in total noise of 1dB. - FFS on ∆thermal value revision to cope with potential low max DL SNR. Recommendations for 2nd round: Provide comments and edits for the tentative agreement. Issue 2-4d: Informative assessment of the testable Demodulation DL SNR range Candidate option: - Ask inputs from TE vendors on the following test system parameters to derive max testable SNR: - TE amplifier 1dB compression, dBm; - backoff from 1dB compression, dB; - Cable loss, dB; - Free space path loss, dB; - TE DL absolute power setting uncertainty, dB; - Probe antenna gain, dB; - Beam peak search procedure/measurement error, dB. Tentative agreement: - Prioritize IFF method for the testable DL SNR assessment - Assume backoff from 1dB compression as [] following the latest conclusion in RAN5 for FR2-1 as starting point value for further assessment - Assume reference range length as 0.725m for IFF method. - FFS on replacing “TE amplifier 1dB compression”, “Cable loss”, “FS path loss” and “Probe antenna gain” by “DL CW power in center of QZ” in the methodology for SNR calculation. Recommendations for 2nd round: Confirm backoff from 1dB compression value. Further discuss proposed parameters replacement. |
| Sub-topic #2-5: TP for TR 38.884 | Issue 2-5: TP on test methods extension to FR2-2 TP R4-2201873 will be revised to address issues highlighted in the RRM and Demod sub-topics. Recommended WF: Confirm whether the TP is agreeable |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2201873 | To be revised |

## Discussion on 2nd round

Please provide feedback on Topic #2 issues as suggested in the 1st round summary

## Companies views’ collection for 2st round

### Open issues

## Summary for 2nd round

### Open issues

|  | Status summary |
| --- | --- |
| Issue 2-1b: |  |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on OTA test methods for FR2-2 | Intel Corporation |  |
| TP to TR 38.884 on extension of NR test methods to FR2-2 | Intel Corporation | Revision of R4-2201873 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2200452 | TP to TR 38.884 on release independence applicability of test method enhancements | Apple | Return to | Need to confirm if TP is agreeable in GTW session or 2nd round |
| R4-2200907 | On permitted test methods for demodulation in FR2-2 | Apple | Noted |  |
| R4-2201873 | TP to TR 38.884 on extension of NR test methods to FR2-2 | Intel Corporation | To be revised | Revision will include agreements from this meeting |
| R4-2201874 | NR FR2-2 OTA test methods for RRM | Intel Corporation | Noted |  |
| R4-2201875 | NR FR2-2 OTA test methods for UE demodulation | Intel Corporation | Noted |  |
| R4-2201990 | Issues with MIMO EVM Measurement Using the Pseudo-Inverse | Lenovo, Motorola Mobility | Noted |  |
| R4-2201921 | On Vehicular UEs | Keysight Technologies UK Ltd | Noted |  |
| R4-2201927 | UE types and permitted methods for FR2-2 | Intel Corporation | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents