3GPP TSG-RAN WG4 Meeting # 102-e                                  																		R4-2203538

Electronic Meeting, February 21 – March 3, 2022

Source:	Charter Communications

Title:	TP to TR 38.717.02-01 for CA_n46-n96

Agenda item:	9.6.1

Document for:	Approval

# Background

This contribution provides text proposal on the NR CA band combination CA_n46-n96 as defined in revised WID on Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2) UL RP-212877[1].

# Text Proposal

##### ---Start of changes---

## 6.X	CA_n46-n96

### 6.X.1	Common for 1 band UL and 2 bands UL CA

#### 6.X.1.1 Operating bands for CA

Table 6.X.1.1-1: CA band combination of band n48+ n96

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_ n46-n96X,Y,Z,ZZ | n46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
|  | n96 | 5925 MHz | – | 7125 MHz | 5925 MHz | – | 7125 MHz | TDD |
| NOTE X: Simultaneous Rx/Tx capability does not apply for UEs supporting CA_n46-n96. Same restrictions are applied to related higher order configurations NOTE Y: The minimum requirements for intra-band non-contiguous CA/DC apply for CA_n46-n96 and related higher order CA/DC configurations. NOTE Z: The combination is not used alone as fall back mode of other band combinations in which UL in Band 48 is not used. NOTE ZZ: The minimum requirements for inter-band CA apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. The power spectral density imbalance condition also applies for these carriers when applicable CA configuration is a subset of a higher order CA configuration. |  |  |  |  |  |  |  |  |

#### 6.X.1.2	Channel bandwidths per operating band for CA

Table 6.X.1.2-1: Supported bandwidths per CA band combination of band n46 + n96

| NR CA configuration | Uplink CA configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n46A-n96A | - | n46 |  | 10 |  | 20 |  |  | 40 |  | 60 |  | 80 |  |  | 0 |
|  |  | n96 |  | 10 |  | 20 |  |  | 40 |  | 60 |  | 80 |  |  |  |
| CA_n46B-n96A | - | n46 | See CA_n46B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n96 |  | 10 |  | 20 |  |  | 40 |  | 60 |  | 80 |  |  |  |
| CA_n46C-n96A | - | n46 | See CA_n46C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n96 |  | 10 |  | 20 |  |  | 40 |  | 60 |  | 80 |  |  |  |
| CA_n46D-n96A | - | n46 | See CA_n46D Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n96 |  | 10 |  | 20 |  |  | 40 |  | 60 |  | 80 |  |  |  |
| CA_n46N-n96A | - | n46 | See CA_n46N Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n96 |  | 10 |  | 20 |  |  | 40 |  | 60 |  | 80 |  |  |  |
| CA_n46A-n96B | - | n46 |  | 10 |  | 20 |  |  | 40 |  | 60 |  | 80 |  |  | 0 |
|  |  | n96 | See CA_n96B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46B-n96B | - | n46 | See CA_n46B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n96 | See CA_n96B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46C-n96B | - | n46 | See CA_n46C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n96 | See CA_n96B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46D-n96B | - | n46 | See CA_n46D Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n96 | See CA_n96B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46N-n96B | - | n46 | See CA_n46N Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n96 | See CA_n96B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Note:  CA_n46-n96 with UL is not specified and it is only used on higher order BC’s. Only 1UL in n46 or n96 will be allowed in higher order combos

#### 6.X.1.3	UE Co-existence studies

Table 6.X.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n46-n96.

Table 6.X.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n46 | 5150 | 5925 | 5150 | 5925 | 10300 | 11850 | 15450 | 17775 | 20600 | 23700 | 25750 | 29625 | 30900 | 35550 | 36050 | 41475 |
| n96 | 5925 | 7125 | 5925 | 7125 | 11850 | 14250 | 17775 | 21375 | 23700 | 28500 | 29625 | 35625 | 35550 | 42750 | 41475 | 49875 |

Based on above table, there is no harmonic interference.

Table 5.X.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n46 | 5150 | 5925 | 5150 | 5925 | 10300 | 11850 | 15450 | 17775 | 20600 | 23700 | 25750 | 29625 | 30900 | 35550 | 36050 | 41475 |
| n96 | 5925 | 7125 | 5925 | 7125 | 11850 | 14250 | 17775 | 21375 | 23700 | 28500 | 29625 | 35625 | 35550 | 42750 | 41475 | 49875 |

Based on above table, there is no harmonic issue for CA_n46-n96.

#### 6.X.1.4	∆TIB and ∆RIB values

For CA_n48-n96, the TIB,c and RIB are given in the tables below.

Table 6.X.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n46-n96 | n46 | 0 |
|  | n96 | 0.5 |

Table 5.X.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_ n46-n96 | n46 | 0 |
|  | n96 | 0 |

#### 6.X.1.5	REFSENs requirements

There are no harmonic or IMD overlaps as a result of non-simultaneous Tx/Rx.

Note: if support for simultaneous Tx/Rx for combinations with n96 are needed then these combinations will be subject to MSD or exclusion in the region where MSD would occur

#### 6.X.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirement for CA_n48-n96.

#### 7.3G.5.X	Reference sensitivity exceptions due to cross band isolation

Based on the notes in Table 6.X.1.1-1: CA band combination of band n48+ n96,  the re no reference sensitivity exceptions due to cross band isolation between n46 and n96 bands  for CA_n46-n96.

##### ---End of changes---

# Reference

[1]	RP-212877, “Revised WID on Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2)”, ZTE Corporation