3GPP TSG-RAN WG4 Meeting #102-e              	R4-2203545

Electrical Meeting, 21th Feb- 3rd March, 2022

Agenda item:	10.9.4.3.1

Source:	Samsung

Title:	Discussion and simulation results of PUSCH requirement for Rel-17 FR2 HST

Document for:	Discussion

# 1	Introduction

In the last RAN4 meeting, the remaining issue on the test parameters on PUSCH requirement for Rel-17 FR2 HST were further discussed. The related agreement was captured in the WF [1] as

- Test applicability rule - Option 1: FR2 HST PUSCH requirement test shall apply only for the additional DM-RS position declared to be supported. If more than one DMRS configuration is declared to be supported, the test shall be done for the minimum number of DMRS supported. - Option 2: FR2 HST PUSCH requirement test shall apply only for the additional DM-RS position declared to be supported. If more than one DMRS configuration is declared to be supported, a pass with either of the possibilities is sufficient to demonstrate compliance to the core requirement. Capture this in a note to the performance requirement. - Manufacturer declaration on HST FR2 DM-RS supported - Define manufacturer declaration, applicable to BS Type 2-O, for PUSCH additional DM-RS in FR2 HST scenario. The intention is that all combinations of pos0, pos1, and pos2 should be possible to declare. Exact wording is FFS. - Receiver baseline assumption for simulation - Assume a receiver with post FFT FOC - MCS - Option 1: only MCS 20 - Option 2: only MCS 16 - Option 3: Both MCS16 and MCS 20, With applicability rule that PUSCH performance requirements needs to be tested only with highest supported MCS - Option 4: Continue simulation results alignment with range as MCS16~MCS20 - Option 5: Use MCS 20 only as baseline, Change to MCS 16 only, if SNR after requirement derivation is larger than 20dB - Option 6: Continue simulation results alignment with MCS 16 and MCS20 only

In this contribution, the view on remaining issue are provided for PUSCH requirement. Meanwhile, the initial simulation results are provided for alignment.

# 2	Discussion

Test Applicability rule for RS configuration

For FR2 HST, 1 DMRS, 2DMRS and 3 DMRS configurations are considered for PUSCH requirement. Since PTRS is configured, the number of DMRS is not the bottleneck of frequency tracking for FR2. The basic Doppler tracking performance can be guaranteed with 1 DMRS configuration. Based on the simulation results, similar performance can be achieved for 2 DMRS and 3 DMRS configurations. With more DMRS symbols, the overhead of RS is higher than 1 DMRS+PTRS. The following is overhead analysis based on different RS configurations.

Table 2-1: Overhead of RS with different RS configurations

| RS configuration | BW (MHz) | Position of DMRS | Overhead of DMRS (length of data is 10, number of CDM group =2 Mapping type B | Overhead of PTRS (length of data is 10, number of CDM group=2, mapping type B) | Total |
| --- | --- | --- | --- | --- | --- |
| 1 DMRS +PTRS (L=1,K=2) | 200 | {0} | 792/15840 | 594/15840 | 1386/15840 |
| 2DMRS + PTRS (L=1, K=2) | 200 | {0,8} | 1584/15840 | 528/15840 | 2112/15840 |
| 3DMRS+ PTRS (L=1, K=2) | 200 | {0,4,8} | 2376/15840 | 462/15840 | 2838/15840 |
| 1 DMRS +PTRS (L=1,K=2) | 50 | {0} | 192/3840 | 144/3840 | 336/3840 |
| 2DMRS + PTRS (L=1, K=2) | 50 | {0,8} | 384/3840 | 128/3840 | 512/3840 |
| 3DMRS+ PTRS (L=1, K=2) | 50 | {0,4,8} | 576/3840 | 112/3840 | 688/3840 |

Observation 1:  The overhead of 1DMRS +PTRS (L=1, K=2) configuration is the smallest compared with other RS configuration schemes.

Observation 2:  Similar performance can be achieved for 2 DMRS configuration and 3 DMRS configuration.

In Rel-15 BS demod requirement for FR2, both 1 DMRS and 2 DMRS configuration are configured for PUSCH requirement. The related test applicability and BS manufacture for difference RS configuration are defined as follows

- Unless otherwise stated, for BS type 2-O, PUSCH requirement tests shall apply only for the additional DM-RS position declared to be supported (see D.101 in table 4.6-1). If both options (i.e., pos0 and pos1) are declared to be supported, the tests shall be done for pos1.

| D.101 | PUSCH additional DM-RS positions | Declaration of the supported additional DM-RS position(s) for FR2, i.e., pos0, pos1, or both. | n/a | n/a | x |
| --- | --- | --- | --- | --- | --- |

Observation 3:  Existing Rel-15 test applicability rule and BS manufacture with different RS configuration cannot guarantee Rel-17 FR2 HST BS test with more than 2 DMRS configuration.

Observation 4: The test is clearly defined non-HST scenario in Rel-15, in case both options (i.e., pos 0 and pos 1) are declared to be supported, the tests shall be done for pos 1

BS manufacture may declare to support with all the three DMRS configurations or subset of DMRS configurations. To reduce the test effort, and guarantee the basic baseband performance, it is not necessary to test all the supported DMRS configurations.

Since the bottleneck of Doppler tracking is not the number of DMRS, with more DMRS, the performance can be improved. Meanwhile, RAN4 is to define the minimum requirement, with less DMRS number, it is more challenging in HST FR2 deployment. Therefore, it is necessary to test the minimum number of DMRS to guarantee the basic baseband processing.

Proposal 1:  FR2 HST PUSCH requirement test shall apply only for the additional DM-RS position declared to be supported. If more than one DMRS configuration is declared to be supported, the test shall be done for the minimum number of DMRS supported

Proposal 2:  RAN4 applies the following manufacturer on HST FR2 DM-RS supported

| D.1XX | PUSCH additional DM-RS positions | Declaration of the supported additional DM-RS position(s) for FR2 high speed train scenario, i.e., {pos0},{pos1},{pos2},{pos0,pos1}, {pos0 pos1}, {pos0,pos2}, {pos1,pos2} and all | n/a | n/a | x |
| --- | --- | --- | --- | --- | --- |

MCS

Regarding the FOC, RAN agreed that the FOC method is up to BS implementation.  For simulation, the receiver

Baseline assumption is assuming a receiver with Post FFT FOC.

In the last meeting, companies provide the simulation results with both Post FFT FOC and Pre FFT FOC. Based on result summary [2], we still observed that large gap among companies’ results. It is necessary to further align the results.

# 3	Initial Simulation Results

In this subsection, the initial simulation results for scenario B under bi-directional,. Also, the performance with different MCS level are investigated to check the impact on high modulation order with different FOC method

Table 3-1 Simulation Assumption for PUSCH requirement

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| Antenna layout |  | 1T2R, ULA Low Correlation |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Option11+0 Option2: 1+1 Option3: 1+1+1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
|  | Bandwidth | 50MHz and 200MHz |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | KPT-RS=2 |
|  | Time density (LPT-RS) | LPT-RS = 1 |
| MCS |  | MCS16 to MCS20 (64 QAM table) |
| Propagation channel |  | Bi-directional with scenario B |

The following table is provided the initial results with MCS 16 to MCS20 are considered for different DMRS configuration

Table 3-1:  Initial results for different MCS level for scenario B with Bi-directional deployment scenario (Ds=700m, Dmin=150m), 120KHz SCS, 200MHz CBW

| Test Case | Tx/Rx | SCS&BW | Mapping type | Symbol length | MCS (64QAM) | DMRS configuration | SNR@70 of TP (Pre-FFT FOC) | SNR@70 of TP (Post-FFT FOC) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 1T2R | 120KHz, 200MHz | B | 10 | 16 | 1+0 | 7.2 | 9.7 |
| Case 2 | 1T2R | 120KHz, 200MHz | B | 10 | 16 | 1+1 (0,8) | 7.1 | 9.5 |
| Case 3 | 1T2R | 120KHz 200MHz | B | 10 | 16 | 1+1+1(0.4.8) | 7.1 | 9.5 |
| Case4 | 1T2R | 120KHz 200MHz | B | 10 | 17 | 1+0 | 7.9 | 11.1 |
| Case 5 | 1T2R | 120KHz | B | 10 | 17 | 1+1 (0.8) | 7.6 | 10.7 |
| Case 6 | 1T2R | 200MHz | B | 10 | 17 | 1+1 (0,4,8) | 7.9 | 10.9 |
| Case 7 | 1T2R | 120KHz | B | 10 | 18 | 1+0 | 8.4 | 12.1 |
| Case 8 | 1T2R | 200MHz | B | 10 | 18 | 1+1 (0.8) | 8.4 | 11.9 |
| Case 9 | 1T2R | 120KHz | B | 10 | 18 | 1+1 (0,4,8) | 8.3 | 11.7 |
| Case 10 | 1T2R | 120KHz 200MHz | B | 10 | 19 | 1+0 | 9.6 | 14.5 |
| Case 11 | 1T2R | 120KHz, 200MHz | B | 10 | 19 | 1+1 (0.8) | 9.2 | 13.8 |
| Case 12 | 1T2R | 120KHz, 200MHz | B | 10 | 19 | 1+1 (0,4,8) | 9.5 | 14.4 |
| Case 13 | 1T2R | 120KHz 200MHz | B | 10 | 20 | 1+0 | 10.4 | 17.7 |
| Case 14 | 1T2R | 120KHz 200MHz | B | 10 | 20 | 1+1(0,8) | 10.3 | 17.2 |
| Case 15 | 1T2R | 120KHz 200MHz | B | 10 | 20 | 1+1+1(0,4,8) | 10.4 | 17.5 |

Table 3-2:  Initial results for different MCS level for scenario B with Bi-directional deployment scenario (Ds=700m, Dmin=150m), 120KHz SCS, 50MHz CBW

| Test Case | Tx/Rx | SCS&BW | Mapping type | Symbol length | MCS (64QAM) | DMRS configuration | SNR@70 of TP (Pre-FFT FOC) | SNR@70 of TP (Post-FFT FOC) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 1T2R | 120KHz, 50MHz | B | 10 | 16 | 1+0 | 7.4 | 10.0 |
| Case 2 | 1T2R | 120KHz,50MHz | B | 10 | 16 | 1+1 (0,8) | 7.1 | 9.5 |
| Case 3 | 1T2R | 120KHz, 50MHz | B | 10 | 16 | 1+1+1(0.4.8) | 7.1 | 9.4 |
| Case4 | 1T2R | 120KHz 50MHz | B | 10 | 17 | 1+0 | 8.0 | 11.3 |
| Case5 | 1T2R | 120KHz, 50MHz | B | 10 | 17 | 1+1 (0.8) | 7.6 | 10.5 |
| Case6 | 1T2R | 120KHz 50MHz | B | 10 | 17 | 1+1+1 (0,4,8) | 7.7 | 10.7 |
| Case7 | 1T2R | 120KHz 50MHz, | B | 10 | 18 | 1+0 | 8.6 | 12.2 |
| Case8 | 1T2R | 120KHz 50MHz | B | 10 | 18 | 1+1 (0.8) | 8.5 | 12.1 |
| Case9 | 1T2R | 120KHz, 50MHz | B | 10 | 18 | 1+1 +1(0,4,8) | 8.2 | 11.6 |
| Case 10 | 1T2R | 120KHz 50MHz | B | 10 | 19 | 1+0 | 9.7 | 14.7 |
| Case 11 | 1T2R | 120KHz 50MHz | B | 10 | 19 | 1+1 (0.8) | 9.5 | 14.2 |
| Case 12 | 1T2R | 120KHz 50MHz | B | 10 | 19 | 1+1 +1(0,4,8) | 9.2 | 13.7 |
| Case 13 | 1T2R | 120KHz 50MHz | B | 10 | 20 | 1+0 | 10.6 | 18.3 |
| Case 14 | 1T2R | 120KHz 50MHz | B | 10 | 20 | 1+1(0,8) | 10.5 | 17.8 |
| Case 15 | 1T2R | 120KHz, 50MHz | B | 10 | 20 | 1+1+1(0,4,8) | 10.5 | 17.9 |

In general, MCS 16 is specified for PUSCH requirement in FR1 HST.  Based on our results, both Post-FFT FOC and Pre-FFT FOC are feasible, meanwhile, the performance different is around 2dB. Also, the performance gap for MCS16 among companies is the smallest.

For MCS 17 to MCS20, the large gap can be existed for two kinds of FOE implementation.  Also, the performance gap among companies is lager

Since RAN4 is to define minimum requirement, to verify the basic baseband processing, MCS16 can be considered for requirement with considering small gap between two kinds of implementation, and smallest span among companies.

Observation 5:  Small performance gap between two kinds of FOC implementation methods for MCS 16 and MCS 17, around 2 or 3 dB difference between post-FFT and pre-FFT FOC methods for MCS 16 and MCS17.

Observation 6: Around 7dB difference between post-FFT and pre-FFT FOC methods for MCS 20

Proposal 2:  RAN4 apply only MCS 16 for PUSCH requirement with FR2 HST

# 4	Conclusion

In this contribution, the view on the PUSCH requirement for Rel-17 FR2 HST is provided. Meanwhile, the initial simulation results with different MCS level are provided to check the feasibility of high modulation order in HST with different FOC methods.

Observation 1:  The overhead of 1DMRS +PTRS (L=1, K=2) configuration is the smallest compared with other RS configuration schemes.

Observation 2:  Similar performance can be achieved for 2 DMRS configuration and 3 DMRS configuration.

Observation 3:  Existing Rel-15 test applicability rule and BS manufacture with different RS configuration cannot guarantee Rel-17 FR2 HST BS test with more than 2 DMRS configuration.

Observation 4: The test is clearly defined non-HST scenario in Rel-15, in case both options (i.e., pos 0 and pos 1) are declared to be supported, the tests shall be done for pos 1

Proposal 1:  FR2 HST PUSCH requirement test shall apply only for the additional DM-RS position declared to be supported. If more than one DMRS configuration is declared to be supported, the test shall be done for the minimum number of DMRS supported

Proposal 2:  RAN4 applies the following manufacturer on HST FR2 DM-RS supported

| D.1XX | PUSCH additional DM-RS positions | Declaration of the supported additional DM-RS position(s) for FR2 high speed train scenario, i.e., {pos0},{pos1},{pos2},{pos0,pos1}, {pos0 pos1}, {pos0,pos2}, {pos1,pos2} and all | n/a | n/a | x |
| --- | --- | --- | --- | --- | --- |

Observation 5:  Small performance gap between two kinds of FOC implementation methods for MCS 16 and MCS 17, around 2 or 3 dB difference between post-FFT and pre-FFT FOC methods for MCS 16 and MCS17.

Observation 6: Around 7dB difference between post-FFT and pre-FFT FOC methods for MCS 20

Proposal 2:  RAN4 apply only MCS 16 for PUSCH requirement with FR2 HST

# References

R4-2120703, WF on BS demodulation requirement for FR2 HST,  Nokia, Samsung