3GPP TSG-RAN WG4 Meeting #102-e             	R4-2203549

Electrical Meeting, 21th Feb- 3rd March, 2022

Agenda item:	12.9.6.2.2

Source:	Samsung

Title:	View on demodulation requirement for Rel-17 NB-IoT

Document for:	Discussion

# 1	Introduction

In the last RAN4 meeting, the performance part of NB-IoT was discussed based on the WID of Rel-17 additional enhancements for NB-IoT and eMTC WI. The related agreement was captured in the WF [2] as

- Define NUSCH format1 requirements with 16QAM and single-TB scheduling - Test parameters - Number of antennas: 1T2R - Frequency offset and time offset: No frequency offset and timing offset considered - Propagation conditions: ETU 1Hz Low - Repetition - Option 1: 3,6,12 tones - Option 2: 12 tones - Number of scheduled RUs and TBS - Option 1: (IRU, ITBS)=(0,15), TBS=280 bits - Option 2: (IRU, ITBS)=(3,19), TBS=1736 bits - Option 3: (IRU, ITBS)=(0,14), TBS=256 bits - Other options are not precluded - Interested companies are welcome to bring simulation results for next meeting and select one based on the evaluation results in the next meeting.

In this contribution, the general view on the test setup for NB-IoT with 16QAM are provided, and initial results are provided to analysis the impact of  IRU, ITBS on 16QAM

# 2	Discussion

Frequency resource

Based on RAN1 design, 16QAM can be applied for the basic Rel-13 NPUSCH format1 with 3, 6 and 12 tones transmission. RAN4 has already defined the related requirement for format1 with QPSK..

In general, the purpose of 16QAM is to increase the peak date rate while retaining differentiation of LTE-MTC and NB-IoT. It is more benefit to schedule with 16QAM for small number of tones, considering the number of available RE is small for 3 or 6 tones.

From the typical scenario for 16QAM scheduling, we think small number tones should be prioritized. Since the test purpose is to verify the 16QAM functionality, it may not need to specify the requirements with all the tones. With considering the test complexity, we prefer to specify the requirement with one tone configuration. While, we are also open to further discuss whether all the tones should be considered for requirements. If the requirements for all the tones are introduced, we prefer to define the test applicability rule

Proposal 1: Prioritized the requirement with 3 tones. FFS the test applicability rule if requirements with 3 tone, 6 tone and 12 tones are defined

Simulation assumption.

Regarding the number of RU configured and Index of TBS, from RAN1 aspect, 16QAM is applied for I_TBS from 14 to 21, and applied for I_RU from 0 to 6, the following TBS table can be supported in RAN1.

Table 1: TBS table for 16QAM

| I_TBS | I_RU |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |

In general, with large number of TBS, the coding rate is large, less coding gain is achieved, and with large number of RU, the related TBS is large and transmission period is long, which will increase the test and simulation time.

In Rel-13, the NPUSCH format1 requirement with both 3.75 KHz and 15KHz SCS were specified, the related FRC was introduced as

Table 2: FRC parameters for NB-IoT NPUSCH format 1

| Reference channel | A16-1 | A16-2 | A16-3 | A16-4 | A16-5 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 | 15 | 15 | 15 |
| Number of allocated subcarriers | 1 | 1 | 3 | 6 | 12 |
| Diversity | No | No | No | No | No |
| Modulation | BPSK | BPSK | QPSK | QPSK | QPSK |
| ITBS / IRU | 0 / 1 | 0 / 1 | 3 / 0 | 7 / 0 | 9 / 0 |
| Payload size (bits) | 32 | 32 | 40 | 104 | 136 |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | 1/3 | 1/3 | 2/3 |
| Code rate (effective) | 0.29 | 0.29 | 0.22 | 0.44 | 0.56 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 288 | 288 | 288 |
| Total symbols per resource unit | 96 | 96 | 144 | 144 | 144 |
| Channel estimation length (ms) Note 1 | 16 | 4 | 4 | 4 | 2 (when repetition = 2) 4 (when repetition > 2) |
| Note 1: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

Observation 1: small number of RU is considered for NPUSCH format1 requirements.

# 2	Initial simulation results

In this subsection, the performance with different RU and TBS, for different tones are provided.

Figure 1: 10% BLER for all TBS entries intended to be used for 16QAM in UL for 3 subcarriers allocation

Figure 2: 10% BLER for all TBS entries intended to be used for 16QAM in UL for 6 subcarriers allocation

Figure 3: 10% BLER for all TBS entries intended to be used for 16QAM in UL for 12 subcarriers allocation

Observation 2: Minor performance difference with different number of RU configured for 3, 6 and 12 subcarrier allocation.

Observation 3: Similar performance can be achieved for 3, 6 and 12 tones for given the number of RU.

Observation 4: About 0.5dB performance gap compared with I_TBS as 14 and 15 for N_RU=0, and about 3.5dB performance gap compared with (IRU, ITBS) = (0, 14) for (IRU, ITBS) = (3, 19).

In LTE Rel-13, the target SNR with 70%TP without repetition is about -3dB for 3 subcarrier allocation, and about -0.6dB for 12 subcarriers allocation.  To compare with LTE Rel-13 with QPSK, we prefer to choose the small number of I_TBS and I_RU for requirement with 16QAM.

Proposal 2: RAN4 applies (IRU, ITBS) = (0, 14) for NPUSCH format1 requirement with 16QAM

In summary, the following simulation assumption for 16QAM requirement can be considered

Proposal 3: RAN4 applies the following simulation assumption for NPUSCH format1 requirement with 16QAM

| Parameter | Value/Description |
| --- | --- |
| Number of antennas | 1T2R |
| SCS | 15KHz |
| Frequency Resource | 3 |
| Number of Repetition | 1 |
| Noise Estimation | Practical |
| Channel Estimation | Practical |
| Frequency offset | 0 |
| Time offset | 0 |
| I_RU | 0 |
| I_TBS | 14 |

# 3	Conclusion

In this contribution, the view on the test setup for NPUSCH format 1 with 16QAM was provided.

Proposal 1: Prioritized the requirement with 3 tones. FFS the test applicability rule if requirements with 3 tone, 6 tone and 12 tones are defined

Observation 1: Small number of RU is considered for NPUSCH format1 requirements.

Observation 2: Minor performance difference with different number of RU configured for 3, 6 and 12 subcarrier allocation.

Observation 3: Similar performance can be achieved for 3, 6 and 12 tones for given the number of RU.

Observation 4: About 0.5dB performance gap compared with I_TBS as 14 and 15 for N_RU=0, and about 3.5dB performance gap compared with (IRU, ITBS) = (0, 14) for (IRU, ITBS) = (3, 19).

Proposal 2: RAN4 applies (IRU, ITBS) = (0, 14) for NPUSCH format1 requirement with 16QAM

Proposal 3: RAN4 applies the following simulation assumption for NPUSCH format1 requirement with 16QAM

| Parameter | Value/Description |
| --- | --- |
| Number of antennas | 1T2R |
| SCS | 15KHz |
| Frequency Resource | 3 |
| Number of Repetition | 1 |
| Noise Estimation | Practical |
| Channel Estimation | Practical |
| Frequency offset | 0 |
| Time offset | 0 |
| I_RU | 0 |
| I_TBS | 14 |

# References

RP-201306, WID on Additional enhancements for NB-IoT and LTE-MTC, Huawei

R4-2203041, WF on Rel-17 NB-IoT and eMTC performance requirement, Huawei