3GPP TSG-RAN WG4 Meeting # 102-e	R4-2203584

Electronic Meeting, February 21 – March 3, 2022

Source:	ZTE Corporation

Title:	Requirements under RRC Connected mode for RedCap UEs

Agenda item:	10.20.3.1.2

Document for:	Discussion

# 1	Introduction

In RAN4 101-e-bis, there were discussions on mobility issues for RedCap UEs.

In the WF [1] the options were captured, and we copy paste some of the issues here to further discuss.

Requirements for HD-FDD in HO - Option 1 (Xiaomi, Apple, vivo, HW, MTK): - RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in HO requirement. - Option 2 (vivo, CMCC, E///, Nokia, ZTE, QC,MTK): The UE shall meet HO requirements provided the following is met: - SSB is available at the UE once every SMTC period during Tsearch. - One SSB is available during T∆ - One SSB is available during Tiu Impact due to early identification in RA - Option 1 (ZTE, HW, E///, Apple, vivo): Early identification doesn’t impact RA requirements. - Option 2 (E///, MTK, Xiaomi, Nokia): RAN4 capture the RedCap UE early indication procedure in TS38.133 6.2.2 after RAN2 conclude the procedures

- This paper discusses the issues listed above and provide our views.

2	Discussions

Requirements for HD-FDD in HO

We think that the intention can be understood, but there are some details to check. First of all what is included in the scope of “UL” transmission in the proposal of “DL measurement is prioritized over the UL transmission”? In our view, RACH shall have the top priority and RACH is transmitted in UL. Thus, this proposal may not be beneficial.

- Observation 1: RACH is transmitted in UL and shall have the top priority during HO.

We also prefer not to capture anything that would limit the UE or network behavior. Any such proposals can be captured in the spec in a way that describes the applicability of the requirements, instead of the specific UE/NW behavior.

Any agreement can be captured in the spec in a way that describes the applicability of the requirements, instead of mandating specific UE/NW behavior.

Thus, we propose to go with Option 2, which is

  - The UE shall meet HO requirements provided the following is met:

    - SSB is available at the UE once every SMTC period during Tsearch.

    - One SSB is available during T∆

    - One SSB is available during Tiu

- The UE shall meet HO requirements provided the following is met:

    - SSB is available at the UE once every SMTC period during Tsearch.

    - One SSB is available during T∆

    - One SSB is available during Tiu

Impact due to early identification in RA

We have one thing to clarify on, which is the early identification. From our understanding early identification is to help the network to identify that a UE belongs to RedCap category or not, and is a purely network behavior / function. We don’t see why this would impact RA requirements on the UE side.

Early identification doesn’t impact RA requirements.

3	Conclusion

- Observation 1: RACH is transmitted in UL and shall have the top priority during HO.

Proposal 1: Any agreement can be captured in the spec in a way that describes the applicability of the requirements, instead of mandating specific UE/NW behavior.

- Proposal 2: The UE shall meet HO requirements provided the following is met:

    - SSB is available at the UE once every SMTC period during Tsearch.

    - One SSB is available during T∆

    - One SSB is available during Tiu

Proposal 3: Early identification doesn’t impact RA requirements.

References

- R4-2202670