3GPP TSG-RAN4 Meeting #102e	R4-2203611

# Online, 21st February – 3rd March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | - | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Rel-16 FR2 DL RMCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DL256QAM_FR2-Core |  |  |  |  |  | Date: |  |  | 2022-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The number of binary channel bits per slot does not match the other parameters defined in the RMC in many cases. The number of alloced slots per frame is misleading, since the RMCs are defined over two frames and only the value for the first frame is given, which is different from the value for the second frame. The Max thoughput averaged over 1 frame does currently not always match the number of allocated slots per frame and should consider the larger number of slots per frame. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct Number of Binary Channel Bits Per Slot values. Correct max throughput per frame. Update number of allocated slots per frame. Add clarifying notes to DL RMC tables |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong values remain in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.521-2 CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### A.3.3.5	FRC for receiver requirements for 256QAM

Table A.3.3.5-1 Fixed Reference Channel for Receiver Requirements (SCS 60 kHz, TDD)

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 50 | 100 | 200 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 |
| Allocated resource blocks |  | 66 | 132 | 264 |
| Subcarriers per resource block |  | 12 | 12 | 12 |
| Allocated slots per Frame (NOTE 6) |  | 23 / 24 | 23 / 24 | 23 / 24 |
| MCS index |  | 24 | 24 | 24 |
| Modulation |  | 256QAM | 256QAM | 256QAM |
| Target Coding Rate |  | 4/5 | 4/5 | 4/5 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79} | Bits | 44040 | 88064 | 176208 |
| Transport block CRC | Bits | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} | CBs | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79} | CBs | 6 | 11 | 21 |
| Binary Channel Bits Per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79} | Bits | 53856 | 107712 | 215424 |
| Max. Throughput averaged over 1 frame (NOTE 7) | Mbps | 105.696 | 211.354 | 422.899 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per 2 frames NOTE 5: PTRS is configured on symbols containing PDSCH with 1 port, per 2PRB in frequency domain, per symbol in time domain. Overhead for TBS calculation is assumed to be 6. NOTE 6: First number corresponds to the number slots allocated in the first frame of the RMC; second number corresponds to the number slots allocated in the second frame of the RMC. NOTE 7: Throughput is averaged over 2nd frame of RMC. |  |  |  |  |

Table A.3.3.5-2 Fixed Reference Channel for Receiver Requirements (SCS 120 kHz, TDD)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 50 | 100 | 200 | 400 |
| Subcarrier spacing configuration |  | 3 | 3 | 3 | 3 |
| Allocated resource blocks |  | 32 | 66 | 132 | 264 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame (NOTE 6) |  | 47 / 48 | 47 / 48 | 47 / 48 | 47 / 48 |
| MCS index |  | 24 | 24 | 24 | 24 |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM |
| Target Coding Rate |  | 4/5 | 4/5 | 4/5 | 4/5 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 21504 | 44040 | 88064 | 176208 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | CBs | 3 | 6 | 11 | 21 |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 26112 | 53856 | 107712 | 215424 |
| Max. Throughput averaged over 1 frame (NOTE 7) | Mbps | 103.219 | 211.392 | 422.707 | 845.798 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per 2 frames NOTE 5: PTRS is configured on symbols containing PDSCH with 1 port, per 2PRB in frequency domain, per symbol in time domain. Overhead for TBS calculation is assumed to be 6. NOTE 6: First number corresponds to the number slots allocated in the first frame of the RMC; second number corresponds to the number slots allocated in the second frame of the RMC. NOTE 7: Throughput is averaged over 2nd frame of RMC. |  |  |  |  |  |

< End of changes >