3GPP TSG-RAN WG4 Meeting # 102-e	R4-2203653

# Electronic Meeting, February 21- March 3, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	REFSENS for 6GHz licensed band

Agenda Item:	9.3.3 UE RF requirements [NR_6 GHz]

Document for:	Approval

# Introduction

In R4#101b-e, UE requirements for 6GHz licensed band based on RCC input in [1] was discussed and some aspects were captured in [2]. Notably, REFSENS is discussed but not agreed. In this contribution, we discuss the 6GHz licensed band REFSENS requirement by comparing it to existing >2GHz NR bands and unlicensed band n46 and n96 cases.

# Discussion

In last meeting way forward [3] the following is captured on REFSENS:

Companies are encouraged to provide further analysis on the UE NF for this frequency range and check the following options:

  - Option 1: Same REFSENS as n78 and n79 for the new band

  - Option 2: Adopt 10.5 dB NF for 6GHz NR band.

  - Option 3: FFS

It should be first noted that the TR [3] on the >6GHz bands had no consensus on the applicable REFSENS, under chapter 7.2.1, Noise figure, the following is given:

The noise figure of 9 dB is the baseline assumption and 13 dB is the optional assumption for co-existence study for 6425 – 7125 MHz and 10 - 10.5 GHz.

A noise figure in the [9, 13] dB interval was finally agreed for reporting to ITU WP5D sharing studies. Note that the noise figure value in [9, 13] dB interval should be used only for WP5D response. The actual noise figure to be used to define RF requirements should be further studied in the WI phase.

Obviously this 4dB range that starts from the initial LTE band 1 (2GHz) assumption reflects the fact that there was no agreement more than anything else.

## REFSENS comparisons

In order to bring some technical insight it is better to look at specified REFSENS in the 2GHz to 7GHz range and look for potential scaling. This is what is shown in Table 1.

Table 1: Comparison of 20MHz REFSEN for different NR bands in the 2 to 7GHz range

| Band | Flow | Fhigh | %BW | 20MHz REFSENS | NF | comment |
| --- | --- | --- | --- | --- | --- | --- |
| n1 | 2110 | 2170 | 1.4% | -93.8 | 8.9 | baseline |
| n40 | 2300 | 2400 | 2.1% | -93.7 | 9.0 |  |
| n41 | 2496 | 2690 | 3.7% | -91.7 | 11.0 | sharp filter impact |
| n78 | 3300 | 3800 | 7.0% | -92.7 | 10.0 |  |
| n77 | 3300 | 4200 | 12.0% | -92.2 | 10.5 | wide filter impact |
| n79 | 4400 | 5000 | 6.4% | -92.7 | 10.0 |  |
| n46 | 5150 | 5925 | 7.0% | -89.7 | 13.0 | same %BW than n78 |
| n96 | 5925 | 7125 | 9.2% | -89.2 | 13.5 | %BW between n78 and n77 |
| 6GHz Lic. | 6425 | 7125 | 5.2% | ?? |  | %BW similar to n79 |

If at a first look there could questions on the scaling, in fact there is rationale behind higher REFSENS:

- First, the losses scale with frequency and thus REFSENS also does

- Band 41 requires a very sharp filter to cope with coexistence with adjacent 2.4GHz connectivity band, but also to meet the strict NS04 requirements in UL

- Band n77 has a 0.5dB impact versus n78 to enable the large 12% fractional bandwidth

- Band n46 and n96 are impacted by the additional losses due to hardware sharing with 5 and 6GHz WiFi and a 1GHz or 2GHz higher frequency than n79 respectively.

## REFSENS scaling

One way to define the 6GHz licensed band REFSENS is to look at different scaling options. For this, we leave bands n41 and n77 aside as they are impacted by aspects that are not fully relevant to the new band.

Table 2: 20MHz REFSEN for 6GHz licensed band based on different scaling scenarios

| Scaling | 20MHz REFSENS | NF | comment |
| --- | --- | --- | --- |
| n1 to n78 scalled | -90.5 | 12.2 |  |
| n1 to n79 scalled | -91.9 | 10.8 | Close to 10.5dB |
| n40 to n78 scalled | -90.3 | 12.4 |  |
| n40 to n79 scalled | -91.8 | 10.8 | Close to 10.5dB |
| 10.5dB NF based | -92.2 | 10.5 |  |
| 13dB NF based | -89.7 | 13.0 | Equivalent to n46 |
| Average of above | -91.1 | 11.6 | dB average! |
| Average of above | -91.0 | 11.7 | Linear average |
| n96 based | -89.2 | 13.5 | Same band but > range provided to ITU |
| n79 based | -92.7 | 10.0 | Not realistic as the band is 2GHz above |

Based on these different scaling scenarios the REFSENS could range from -89.2 to -92.7 dBm but some may not be reasonable:

- reusing n79 is not reasonable 2GHz above in frequency

- n96 based NF is 0.5dB higher than the range already provided to ITU

Therefore, the range to choose from is rather -89.7 to -92.2 dBm and the average of the different valid scaling scenarios is ~ -91dBm.

## Architecture considerations

Although we agree that n96 REFSENS is too high for the 6GHz licensed given that is already communicated to ITU, we think the HW reuse between the 6GHz licensed band and n96/n102 and to some extent n46 is still needed, at a minimum they will share antennas and thus some multiplexing losses need to be taken into account.

Based on the need for antenna reuse, a NF assumption of 11.5dB is appropriate further accounting for the 2GHz frequency increase vs n79 and very close to the average calculated from the valid scaling scenarios which correspond to a -91.2dBm REFSENS at 20MHz and 15kHz SCS.

Proposal: 11.5dB NF figure is assumed for 6GHz licensed band (-91.2dBm REFSENS at 20MHz and 15kHz SCS).

# Conclusions

In this contribution, we discuss the 6GHz licensed band REFSENS requirements based on scaling from other bands >2GHz and antenna combining architecture and formulate the following proposal.

Proposal: 11.5dB NF figure is assumed for 6GHz licensed band (-91.2dBm REFSENS at 20MHz and 15kHz SCS).

# References

- RP-213605, RCC Recommendation 1/21, “Harmonization of the technical conditions for 5G-NR / IMT-2020 systems in the RCC countries in the frequency band 6 425-7 125 MHz or in its portions” RCC Commission for the Regulation of the Usage of the Radio Frequency Spectrum and Satellite Orbits, Document: Commission Decision 20/5 Annex 1, December 2021

- R4-2202267 WF on UE RF requirements for 6GHz licensed band, Ericsson, R4#101b-e

- 3GPP TR 38.921 V17.0.0 (2021-03), Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz.