3GPP RAN WG4 Meeting #102-e		R4-2203697

Online, Feb 21st – Mar 3rd, 2022

Agenda item:	10.4.5.1

Source:	Apple

Title:	Views on new FR2 CA BW classes

WI/SI:	NR_RF_FR2_req_enh2-Core

Release:	Rel-17

Document for:	Discussion

# 1	Introduction

In the need of supporting more than 1GHz contiguous spectrum in certain FR2 band which has been made available in US [1], in RAN #91-e meeting, an objective of introducing new FR2 CA BW classes and the related Rx requirements to support contiguous downlink aggregated channel BW up to 1600 MHz has been approved to be included in the revised WID on further enhancements of NR RF requirements for frequency range 2 [2]. In RAN4 #99e meeting, it was first agreed to introduce 4 new CA BW classes R, S, T, and U in fallback group 2 (FBG2) to support aggregated channel BW up to 1600 MHz [3]. However, considering the backward compatibility with the existing FR2 cells at 100MHz in fallback group 3 (FBG3), in RAN4 #100e meeting, it was further proposed to introduce 4 additional CA BW classes MA, MD, ME, and MF by combing FGB3 and FBG2 [4] which was later named as Alt 1 option as in contrast to an alternative option named Alt 2 in [5]. Due to the limited flexibility in fallback combination support in both Alt 1 and Alt 2 options, two more alternatives were proposed in last RAN4 meeting [6-7], where the outcome of the discussions as captured in the approved WF [8] was to down-select the 4 alternatives to two options for decision in this meeting. In this contribution, we share our views on the two options for the new FR2 CA BW classes definition.

# Discussion

The definition of the two new FR2 CA BW classes being considered are shown in Figure 2-1 and Table 2-1, which were called as Option 2 and Option 4 respectively as to follow the approved WF in last RAN4 meeting [8]. In the contexts below, we compare the two options from the aspects of new cell migration, specifications complexity, CA configurations, signalling, fallback combination and cell arrangement flexibility, and future proof to facilitate the RAN4 decision on finalizing the new FR2 CA BW classes definition in Rel-17.

| Class | Carrier configuration |  |  |  |  |  |  |  |  |  |  |  | Number of contiguous CC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FBG3 |  |  |  |  |  |  |  | FBG2 |  |  |  | FBG3 | FBG2 |
| MA |  |  |  |  |  |  |  |  |  |  |  |  | 8 | 1 |
| MD |  |  |  |  |  |  |  |  |  |  |  |  | 8 | 2 |
| ME |  |  |  |  |  |  |  |  |  |  |  |  | 8 | 3 |
| MF |  |  |  |  |  |  |  |  |  |  |  |  | 8 | 4 |
| AF |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 4 |
| GF |  |  |  |  |  |  |  |  |  |  |  |  | 2 | 4 |
| HF |  |  |  |  |  |  |  |  |  |  |  |  | 3 | 4 |
| IF |  |  |  |  |  |  |  |  |  |  |  |  | 4 | 4 |
| JF |  |  |  |  |  |  |  |  |  |  |  |  | 5 | 4 |
| KF |  |  |  |  |  |  |  |  |  |  |  |  | 6 | 4 |
| LF |  |  |  |  |  |  |  |  |  |  |  |  | 7 | 4 |

Figure 2-1 New FR2 CA BW classes definition: Option 2

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| V2 | 150 MHz ≤ BWChannel_CA ≤ 400 MHz | 2 | 5 |
| V3 | 250 MHz ≤ BWChannel_CA ≤ 600 MHz | 3 |  |
| V4 | 350 MHz ≤ BWChannel_CA ≤ 800 MHz | 4 |  |
| V5 | 450 MHz ≤ BWChannel_CA ≤ 900 MHz | 5 |  |
| V6 | 550 MHz ≤ BWChannel_CA ≤ 1000 MHz | 6 |  |
| V7 | 650 MHz ≤ BWChannel_CA ≤ 1100 MHz | 7 |  |
| V8 | 750 MHz ≤ BWChannel_CA ≤ 1200 MHz | 8 |  |
| V9 | 950 MHz ≤ BWChannel_CA ≤ 1300 MHz | 9 |  |
| V10 | 1150 MHz ≤ BWChannel_CA ≤ 1400 MHz | 10 |  |
| V11 | 1350 MHz ≤ BWChannel_CA ≤ 1500 MHz | 11 |  |
| V12 | 1550 MHz ≤ BWChannel_CA ≤ 1600 MHz | 12 |  |

Table 2-1 New FR2 CA BW classes definition: Option 4

Support of cell migration

Option 2 as a combination of FBG3+FBG2 was originated with the consideration of backward compatibility and cell transition from 100 MHz to 200 MHz. Option 4 inherently also adopts the combination of FGB3+FBG2, for example, class V11 can support the configurations of 7x100MHz + 4x200MHz or 8x100MHz + 3x200MHz. Therefore, both options can support the cell upgrade and migration from 100 MHz to 200 MHz.

Specifications complexity

As both options introduce 11 new CA BW classes, from the number of CA combinations within FR2 or with FR1 bands perspective, there would be no difference between Option 2 and Option 4.

CA configurations

For Option 2, each CA BW class maps to one exact single CA configuration, as shown in Table 2-2.

| CA Configuration | Carrier Bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CC1 | CC2 | CC3 | CC4 | CC5 | CC6 | CC7 | CC8 | CC9 | CC10 | CC11 | CC12 |
| CA_nxxxMA | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 200 |  |  |  |
| CA_nxxxMD | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 200 | 200 |  |  |
| CA_nxxxME | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 200 | 200 | 200 |  |
| CA_nxxxMF | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 200 | 200 | 200 | 200 |
| CA_nxxxAF | 100 | 200 | 200 | 200 | 200 |  |  |  |  |  |  |  |
| CA_nxxxGF | 100 | 100 | 200 | 200 | 200 | 200 |  |  |  |  |  |  |
| CA_nxxxHF | 100 | 100 | 100 | 200 | 200 | 200 | 200 |  |  |  |  |  |
| CA_nxxxIF | 100 | 100 | 100 | 100 | 200 | 200 | 200 | 200 |  |  |  |  |
| CA_nxxxJF | 100 | 100 | 100 | 100 | 100 | 200 | 200 | 200 | 200 |  |  |  |
| CA_nxxxKF | 100 | 100 | 100 | 100 | 100 | 100 | 200 | 200 | 200 | 200 |  |  |
| CA_nxxxLF | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 200 | 200 | 200 | 200 |  |

Table 2-2 Option 2 CA configurations

For Option 4, each CA BW class can support more than one CA configurations, as shown in Table 2-3.

| CA Configuration | Carrier Bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CC1 | CC2 | CC3 | CC4 | CC5 | CC6 | CC7 | CC8 | CC9 | CC10 | CC11 | CC12 |
| CA_nxxxV2 | 50, 100, 200 | 100, 200 |  |  |  |  |  |  |  |  |  |  |
| CA_nxxxV3 | 50, 100, 200 | 100, 200 | 100, 200 |  |  |  |  |  |  |  |  |  |
| CA_nxxxV4 | 50, 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  |  |  |  |  |
| CA_nxxxV5 | 50, 100 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  |  |  |  |
| CA_nxxxV6 | 50, 100 | 100 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  |  |  |
| CA_nxxxV7 | 50, 100 | 100 | 100 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  |  |
| CA_nxxxV8 | 50, 100 | 100 | 100 | 100 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  |
| CA_nxxxV9 | 50, 100 | 100 | 100 | 100 | 100 | 100, 200 | 100, 200 | 100, 200 | 200 |  |  |  |
| CA_nxxxV10 | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100, 200 | 100, 200 | 200 | 200 |  |  |
| CA_nxxxV11 | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100, 200 | 200 | 200 | 200 |  |
| CA_nxxxV12 | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 200 | 200 | 200 | 200 |

Table 2-3 Option 4 CA configurations

Notice that the configurations highlighted in red in Table 2-3 are the ones supported in Option 2. Therefore, Option 4 has the advantage that less CA BW classes are needed to support the same CA configurations as in Option 2.

Signalling

From signalling perspective, for Option 2, if a UE would support all the specified BW classes as fallback, then it would need to signal two supported highest order BW classes except for class MF. On the other hand, for Option 4, UE only needs to signal one supported highest order BW class, and all the lower order BW classes can automatically be supported.

Fallback combination and cell arrangement flexibility

Option 4 provides the full flexibility for the combination of FBG3+FBG2 with any carrier number in each of the fallback group. However, Option 2 can only support the fallback in one fallback group while the other fallback group is at maximum number of carriers. On the other hand, the naming of Option 2 CA BW classes may have an implication that the 100MHz cells can only be at the lower frequency side than 200MHz cells, while Option 4 would not have such restriction.

Future proof

If in future, a new configuration such as 6x100 + 2x200 is needed, Option 2 would not be able to accommodate it and new CA BW classes may need to be further defined which could be another grinding process and may load up more CA combinations. Option 4 on the other hand does not have such issue and is more future proof.

Based on the above assessments, the comparison between the two options can be summarized in Table 2-4 where the Option with the advantage is marked with a “+” sign.

| Aspect | Option 2 | Option 4 |
| --- | --- | --- |
| Support of cell migration | Yes | Yes |
| Specifications complexity | Same |  |
| CA configurations |  | + |
| Signaling |  | + |
| Fallback combination and cell arrangement flexibility |  | + |
| Future proof |  | + |

Table 2-4 Comparison between Option 2 and Option 4 CA BW classes definition

# 3	Conclusion

In this contribution, we share our views on the two options for the new FR2 CA BW classes definition and the comparison of the two options is summarized in the table below.

| Aspect | Option 2 | Option 4 |
| --- | --- | --- |
| Support of cell migration | Yes | Yes |
| Specifications complexity | Same |  |
| CA configurations |  | + |
| Signaling |  | + |
| Fallback combination and cell arrangement flexibility |  | + |
| Future proof |  | + |

# 4	References

R4-2100264 “Release 17 FR2 bandwidth class”, Verizon, 3GPP TSG-RAN WG4 Meeting #98-e, January 25th – February 5th, 2021

RP-210914 “Revised WID Further enhancements of NR RF requirements for frequency range 2 (FR2)”, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN Meeting #91-e, March 22nd – 26th, 2021

R4-2107860 “CR for TS 38.101-2: Introduction of FR2 new CA BW classes”, Qualcomm, TSG-RAN WG4 Meeting #99-e, May 19th – 27th, 2021

R4-2114963 “WF on FR2 new CA BW Class”, Nokia, TSG-RAN WG4 Meeting #100-e, August 16th – 27th, 2021

R4-2117541 “Fallback behaviour of FBG3+2”, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 Meeting #101-e, November 1st – 12th, 2021

R4-2200909 “Alternatives of FR2 new CA BW classes”, Apple, 3GPP TSG-RAN WG4 Meeting #101bis-e, January 17th – 25th, 2022

R4-2200857 “FR2 bandwidth classes covering up to 1600 MHz aggregated bandwidth with mixed carrier bandwidths”, Ericsson, 3GPP TSG-RAN WG4 Meeting #101bis-e, January 17th – 25th, 2022

R4-2202347 “WF on FR2 CA BW classes”, Xiaomi, 3GPP TSG-RAN WG4 Meeting #101bis-e, January 17th – 25th, 2022