3GPP TSG-RAN WG4 Meeting # 102-e	R4-2203760

# Electronic Meeting, Feb 21- Mar 3, 2022

Agenda item:	10.6.5.2

Source:	Apple Inc.

Title:	Discussion on PDSCH demod requirements with 1KQAM

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#101-bis-e PDSCH demod requirements with 1024QAM in NR DL were discussed and way forward [1] was agreed.

- Interested companies are encouraged to provide the PDSCH simulation results according to Table 1 in the next meeting to decide the SNR test points. - RAN4 will discuss whether to add extra margin after deriving the SNR test points to achieve 70% of maximum throughput based on the impairment results.

In this contribution we provide simulation results and our views on the open issues for PDSCH demod requirements with 1024QAM.

# 2. Discussion

The agreed simulation assumptions for PDSCH demod requirements are captured in Table 1.

Table 1: Simulation parameters for PDSCH demod with 1024QAM

| Parameters | FDD | TDD |
| --- | --- | --- |
| SCS/CBW | 15kHz and 10MHz | 30kHz and 40MHz |
| TDD UL/DL configuration | N/A | 7D1S2U, with S=6:4:4 Schedule PDSCH in the special slots |
| MCS (MCS index table 4) | MCS23 |  |
| Rank | 1 |  |
| Antenna configuration | 2x2 with ULA Low 2x4 with ULA Low |  |
| Propagation channel | Option 1: TDLD30-5 Option 2: TDLA30-10 |  |
| PDSCH configuration | Type A mapping, Start symbol 2, Duration 12 |  |
| PDSCH DMRS configuration | Type 1, Single symbol, additional DMRS: pos1 |  |
| Tx EVM assumed for simulation | 2.5% |  |
| SSB configuration | Periodicity 20 ms, Allocated in first slot within 20ms |  |
| TRS configuration | 20 ms periodicity, 2 slots, Offset 10 ms |  |
| Number of HARQ processes | 4 for FDD 15kHz, 8 for TDD 30kHz |  |
| Transform precoding | CP-OFDM |  |
| Allocated RBs | Full BWP |  |
| PRB bundling | 2 |  |
| Precoding model | Random Precoding, per slot and per RB bundling size granularity (codebook configuration: Single panel Type 1) |  |
| Receiver type | MMSE-IRC |  |
| Test metric | 70% of maximum throughput |  |

For the simulation assumptions in Table 1 we provide simulation results for PDSCH demod.

TX EVM: 2.5%

MCS: 23

Channel Model: TDLA30-10, TDLD30-5

TDD

SCS: 30KHz

CBW: 40MHz

S slots: Enable PDSCH scheduling

Figure 1: Simulation results for TDD

FDD

SCS: 15KHz

CBW: 10MHz

Figure 2: Simulation results for FDD

The simulation results are summarized in the Table below.

Table 2: Results summary

| Duplex Mode | Antenna Config | MCS | Propagation Model | SNR @70% Max TP |
| --- | --- | --- | --- | --- |
| FDD | 2x2 ULA Low | 23 | TDLA30-10 | 27.9 |
|  |  |  | TDLD30-5 | 27.6 |
|  | 2x4 ULA Low | 23 | TDLA30-10 | 23.6 |
|  |  |  | TDLD30-5 | 23.0 |
| TDD | 2x2 ULA Low | 23 | TDLA30-10 | 28.2 |
|  |  |  | TDLD30-5 | 28.1 |
|  | 2x4 ULA Low | 23 | TDLA30-10 | 24.3 |
|  |  |  | TDLD30-5 | 23.4 |

Comparing the performance of TDLA and TDLD propagation channel, TDLD is better than TDLA by around 0.1 to 0.6 dB. Also, TDLD is a LOS channel model not very suitable for FR1 conditions.

Observation #1: There is no significant performance improvement with TDLD channel over TDLA.

Observation #2: TDLD is LOS channel more suitable for FR2 conditions.

Based on the performance results and observations, we propose to define PDSCH demod requirements with 1024QAM with TDLA channel model.

Proposal #2: Define PDSCH demod requirements for 1024QAM with TDLA30-10 channel model.

# 3. Conclusion

In this paper, we provide simulation results and our views on the open issues for PDSCH demod requirements with 1024QAM. Our observations and proposals are captured below:

Observation #1: There is no significant performance improvement with TDLD channel over TDLA.

Observation #2: TDLD is LOS channel more suitable for FR2 conditions.

Proposal #2: Define PDSCH demod requirements for 1024QAM with TDLA30-10 channel model.

# Reference

- R4-2203044, “Way forward for NR DL1024QAM demodulation and CQI reporting requirements”, Ericsson.