3GPP TSG-RAN WG4 Meeting # 102-e	R4-2203764

# Electronic Meeting, Feb 21- Mar 3, 2022

Agenda item:	10.12.2.1.1

Source:	Apple Inc.

Title:	Discussion on PDSCH requirements in intercell interference scenarios

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#101-bis-e PDSCH demodulation requirements with MMSE-IRC for inter-cell interference was discussed and way forward [1] was agreed. In this contribution we present our views on the open issues for requirements definition for PDSCH demod with ICI and simulation results.

# 2. Requirements for Scenario 1

## Common Test Parameters

The open issues on common test parameters are:

- Network Type

- SSB Configuration

Network Type

Options discussed in previous meeting for network type:

  - Option 1: Only consider synchronized network

  - Option 2: Include FDD asynchronized network type with applicability rule:

    - For 2Rx/4Rx UE that only support FDD mode, we can have 1 HomNet test for aync scenario and 1 HetNet test for sync scenario.

    - For 2Rx/4Rx UE that support both FDD and TDD modes, we can have 1 test for HomNet FDD async and 1 test for HetNet TDD sync respectively.

Synchronized network for FDD and TDD is the baseline assumption for requirements in ICI with MMSE-IRC receiver. For ICI scenarios the interference and target cells might not be synchronized but considering synchronized network would have the worse impact on UE performance. Moreover, we don’t see any impact to MMSE-IRC receiver processing for asynchronized network. We don’t see a benefit of adding extra test cases and requirements for asynchronized network.

Proposal #1: Only consider synchronized network for ICI requirements.

SSB Configuration

The options for SSB configuration for interference cell in [1]:

- SSB Option 1: All SSBs (serving cell and interference cell(s)) are in the same time/frequency resources

- SSB Option 2: Serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources

In [1] the SSB configuration was discussed and following options for requirements definition were presented:

- Option 1: Use SSB Option 1 (All SSBs are in the same time/frequency resources) for all test

- Option 2: Use SSB Option 2 (Serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources) for all test

- Option 3: Use different assumptions for different deployment scenarios: SSB Option 1 for homogeneous deployment assumptions and SSB Option 2 for heterogeneous deployment assumptions

Our preference is to have non-overlapping SSBs for serving and interference cells. But based on discussion it is our understanding that in real deployment scenarios for homogeneous networks at least it is common to have overlapping SSBs. Hence, we are supportive of option 3 to use overlapping SSB for homogeneous network and non-overlapping SSB for heterogeneous network.

Proposal #2: Use different assumptions for different deployment scenarios: SSB Option 1 (overlapping) for homogeneous deployment and SSB Option 2 (non-overlapping) for heterogeneous deployment assumptions.

## Interference model

In RAN4#101-bis-e the agreements for common parameters were:

- INR values for homogenous Deployment and synchronous network - INRs 7.77 and 2.29 dB in case of 2 interference cells and INR 5.49 dB in case of 1 interference cell

Open issues related to interference model are:

- INR values for heterogenous deployment

- Number of explicitly modeled interference cells

- Time and frequency offsets for synchronized network

- Interference modelling in PDCCH region

INR Values for Heterogeneous Deployment

The INR values for heterogenous network was discussed and following options were finalized:

  - Use INRs 11.39 and 5.45 dB in case of 2 interference cells are modelled

  - Select one of the following options for scenario with 1 interference cell

    - Option 1: INR 4.84 dB.

    - Option 2: INR 7.58 dB

We provide the simulation results for with different INR values for FDD below for single interference cell for homogeneous network and heterogeneous network assumption.

Table 1: Performance with different interference levels for single interference cell

| Ant. Config. | INR1 (dB) | Prop. Channel | SNR (dB) |  |  | SINR with MMSE-IRC |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MMSE | MMSE-IRC | Gain |  |
| 2x2 | 5.49 | TDLC300-100 | 12.1 | 11.3 | 0.7 | 4.8 |
| 2x4 | 5.49 | TDLC300-100 | 9.3 | 7.9 | 1.4 | 1.3 |
| 2x2 | 4.84 | TDLA30-10 | 10.6 | 9.7 | 0.9 | 3.6 |
| 2x4 | 4.84 | TDLA30-10 | 8.0 | 5.7 | 2.2 | -0.4 |
| 2x2 | 7.58 | TDLA30-10 | 12.5 | 11.0 | 1.5 | 2.7 |
| 2x4 | 7.58 | TDLA30-10 | 9.7 | 6.2 | 3.5 | -2.1 |

The operating SNR and gain with option 2 is higher compared to option 1 because of higher INR. The INR in option1 is lower than INR for homogeneous deployment. Typically, the INR levels for heterogenous deployment would be higher than homogenous deployment. Give that the operating SINR is not very low, we propose to use option 2 for single cell INR value for heterogenous deployment assumption.

Proposal #3: Use INR value of 7.58dB for heterogeneous deployment assumption with 1 interference cell.

Number of explicitly modelled interference cells

Options discussed for number of interference cells:

  - Option 1: 1 interference cell for all tests

  - Option 2: 2 interference cells for all tests

  - Option 3: Use different assumptions for different deployment scenarios: 2 interference cells for homogeneous deployment assumptions and 1 interference cell for heterogeneous deployment assumptions

The number of interference cells doesn’t impact UE processing with MMSE-IRC. Also, we from the results presented in subsequent section that 1 and 2 cell interference have comparable gains and operating SINR. Hence, we prefer to simplify the test setup and only enable 1 interference cell.

Proposal #4: Define requirements for homogeneous and heterogeneous deployment with 1 interference cell.

Time offset for synchronized network with 30KHz SCS

In the WF [1] from last meeting the following options were discussed for TDD 30KHz SCS:

    - Option 1: The serving cell is 3 us for interfering cell 1 and -1 us for interfering cell 2 (in case modeled)

    - Option 2: The serving cell is 1 us for interfering cell 1 and -0.25 us for interfering cell 2 (in case modeled)

    - Other options are not precluded

- Companies are encouraged to bring simulation results for both options next meeting to identify whether significant performance difference can be observed

We evaluate performance for homogenous and heterogenous deployments for 1 interference cell with different TO options.

Table 2: Performance with different TO for TDD with 1 interference cell-2x2

| INR (dB) | Deployment/ TO option | Prop. Channel | SNR (dB) |  |  | SINR w/ MMSE-IRC |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MMSE | MMSE-IRC | Gain |  |
| 5.49 | HomoNet Opt1 | TDLC300-100 | 12.5 | 11.6 | 0.9 | 5.0 |
| 5.49 | HomoNet Opt2 | TDLC300-100 | 12.5 | 11.7 | 0.8 | 5.1 |
| 7.58 | HetNet Opt1 | TDLA30-10 | 12.8 | 10.9 | 1.9 | 2.6 |
| 7.58 | HetNet Opt2 | TDLA30-10 | 12.8 | 11.0 | 1.8 | 2.7 |

The results show that there is no significant delta between the 2 options for time offset.

Observation #1: No significant performance delta with different TO assumptions with 1 interference cell.

For synchronized network with TDD, the time offset of 3us in TDD with 30KHz SCS would no longer meet the synchronized network assumption since time offset is > CP length for 30KHz SCS and option 2 above would be more suitable. Hence, we propose to use time offset of interference cell as 1us.

Observation #2: With time offset of 3us in 30KHz, the synchronized network assumption is no longer met.

Proposal #5: Define requirements with time offset of interference cell in TDD as 1us.

Interference modelling in PDCCH region

Interference modeling in PDCCH region was discussed with the options below :

  - Option 1: NR interference model to have unallocated RE’s in control region filled with QPSK randomly modulated symbols with random precoding for the number of antenna ports in the requirement scenario.

  - Option 2: No interference signal in PDCCH region

  - Option 3: Assume PDCCH transmission from interference cells

With option 2 to assume no interference in PDCCH region is not practical when we are defining requirements for PDSCH demod in ICI. It’s not practical to assume that there will be no interference on PDCCH. Typically, PDCCH would see interference from PDCCH in a synchronized network and it would be suitable to model it the same for ICI requirements. Since we are not evaluating PDCCH performance here, we should not have to evaluate impact of randomly modulated and precoded symbols vs PDCCH interference. PDCCH seeing PDCCH interference is more practical in our understanding.

Proposal #6: Model PDCCH interference from interference cells in PDCCH region.

## Simulation Results

For the simulation assumptions agreed in [1], we present the simulation results for FDD and TDD for 1 and 2 interference cells.

HomoNet: INRs 7.77 and 2.29 dB in for 2 interference cells and INR 5.49 dB for 1 interference cell

HetNet: INRs 11.39 and 5.45 dB for 2 interference cells

HetNet Opt1: INR 4.84 dB for 1 interference cell.

HetNet Opt2: INR 7.58 dB for 1 interference cell.

FDD

Time offset: The serving cell is 3 us for interfering cell 1 and -1 us for interfering cell 2 (in case modeled)

Frequency shift: The serving cell is 300 Hz for interfering cell 1 and -100 Hz for interfering cell 2 (in case modeled)

Table 3: Simulation results with FDD

| Ant. Config. | # Interf Cells | Deployment | Prop. Channel | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MMSE | MMSE-IRC | Gain |
| 2x2 | 1 | HomoNet | TDLC300-100 | 12.1 | 11.3 | 0.7 |
| 2x4 | 1 | HomoNet | TDLC300-100 | 9.3 | 7.9 | 1.4 |
| 2x2 | 1 | HetNet Opt1 | TDLA30-10 | 10.6 | 9.7 | 0.9 |
| 2x4 | 1 | HetNet Opt1 | TDLA30-10 | 8.0 | 5.7 | 2.2 |
| 2x2 | 1 | HetNet Opt2 | TDLA30-10 | 12.5 | 11.0 | 1.5 |
| 2x4 | 1 | HetNet Opt2 | TDLA30-10 | 9.7 | 6.2 | 3.5 |
| 2x2 | 2 | HomoNet | TDLC300-100 | 15.0 | 14.4 | 0.5 |
| 2x4 | 2 | HomoNet | TDLC300-100 | 12.3 | 10.8 | 1.6 |
| 2x2 | 2 | HetNet | TDLA30-10 | 17.1 | 15.8 | 1.3 |
| 2x4 | 2 | HetNet | TDLA30-10 | 14.3 | 10.5 | 3.7 |

TDD

Time offset: The serving cell is 1us for interfering cell 1 and -0.25 us for interfering cell 2 (in case modeled)

Frequency shift: The serving cell is 300 Hz for interfering cell 1 and -100 Hz for interfering cell 2 (in case modeled)

Table 4: Simulation results with TDD

| Ant. Config. | # Interf Cells | Deployment | Prop. Channel | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MMSE | MMSE-IRC | Gain |
| 2x2 | 1 | HomoNet | TDLC300-100 | 12.5 | 11.6 | 0.9 |
| 2x4 | 1 | HomoNet | TDLC300-100 | 9.8 | 8.3 | 1.5 |
| 2x2 | 1 | HetNet Opt1 | TDLA30-10 | 10.8 | 9.7 | 1.2 |
| 2x4 | 1 | HetNet Opt1 | TDLA30-10 | 8.4 | 6.1 | 2.3 |
| 2x2 | 1 | HetNet Opt2 | TDLA30-10 | 12.8 | 10.9 | 1.9 |
| 2x4 | 1 | HetNet Opt2 | TDLA30-10 | 10.3 | 6.6 | 3.7 |
| 2x2 | 2 | HomoNet | TDLC300-100 | 15.5 | 14.6 | 0.9 |
| 2x4 | 2 | HomoNet | TDLC300-100 | 12.6 | 11.2 | 1.4 |
| 2x2 | 2 | HetNet | TDLA30-10 | 17.3 | 15.7 | 1.6 |
| 2x4 | 2 | HetNet | TDLA30-10 | 14.8 | 10.7 | 4.1 |

# 3. Requirements for scenario 2

Based on the WID scenario 2 with non-slot-based transmission was to be discussed after scenario 1 was stable. This is like uneven interference scenario discussed in Rel-18 demod enhancements. We don’t think that non-slot-based transmission is widely used in networks today. Also, based on the proposals in [2] it looks like an enhancement over MMSE-IRC receiver would be needed to handle this scenario. Also, additional signalling to indicate the interfered symbols would be needed for correct UE processing.

Observation #3: Non-slot-based transmission is not widely deployed in networks.

Observation #4: Handling of partial slot transmission needs some enhancements to baseline MMSE-IRC receiver assumed for ICI requirements

Observation #5: Additional signalling is needed for symbols with interference for correct UE processing

Given the observations above, we don’t support to further discuss requirements for scenario 2 in Rel-17.

Proposal #7: Do not introduce requirements in Rel-17 for non-slot-based transmission for ICI.

# 3. Conclusion

In this paper, we provide our views on the open issues for requirements definition for PDSCH demod with ICI and simulation results. Our observations and proposals are captured below:

Common Test Parameters for Scenario 1

Proposal #1: Only consider synchronized network for ICI requirements.

Proposal #2: Use different assumptions for different deployment scenarios: SSB Option 1 (overlapping) for homogeneous deployment and SSB Option 2 (non-overlapping) for heterogeneous deployment assumptions.

Interference Model for Scenario 1

Proposal #3: Use INR value of 7.58dB for heterogeneous deployment assumption with 1 interference cell.

Proposal #4: Define requirements for homogeneous and heterogeneous deployment with 1 interference cell.

Observation #1: No significant performance delta with different TO assumptions with 1 interference cell.

Observation #2: With time offset of 3us in 30KHz, the synchronized network assumption is no longer met.

Proposal #5: Define requirements with time offset of interference cell in TDD as 1us.

Proposal #6: Model PDCCH interference from interference cells in PDCCH region.

Requirements for Scenario 2

Observation #3: Non-slot-based transmission is not widely deployed in networks.

Observation #4: Handling of partial slot transmission needs some enhancements to baseline MMSE-IRC receiver assumed for ICI requirements

Observation #5: Additional signalling is needed for symbols with interference for correct UE processing

Proposal #7: Do not introduce requirements in Rel-17 for non-slot-based transmission for ICI.

# Reference

- R4-2203008, “WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, Intel.

- R4-2200512, “Discussion on PDSCH demodulation MMSE-IRC requirements for scenario with inter-cell interference”, Intel.