3GPP TSG-RAN WG4 Meeting # 102-e	R4-2203768

# Electronic Meeting, Feb 21- Mar 3, 2022

Agenda item:	10.12.2.2

Source:	Apple Inc.

Title:	TP to TR 38.833: MU-MIMO-Receiver structure

Release:	Rel-17

Document for:	Approval

# 1. Introduction

This contribution is a TP to update the receiver structure section of MU-MIMO in TR 38.833. The sub-clauses in square brackets are deleted since they are not applicable to the receiver assumption for the study phase of inter-user intra-cell interference.

# Reference

- R4-2115733, “Way Forward on MMSE-IRC for intra-cell inter-user interference”, Huawei, HiSilicon.

## <<< Start of text proposal >>

## 4.2	Receiver structure

### 4.2.1	General

In this clause, we provide the system equations for evaluating the performance of intra-cell inter-user interference mitigation.

The NRx-dimensional received signal vector r of the k-th subcarrier and the l-th OFDM symbol is assumed to be expressed as a sum of target’s UE own signal H1k,ld1k,l, and co-scheduled UEs’ interference signals Hjk,ldjk,l (j>1) and the white noise n(k,l);

rk, l= H1k,ld1k,l+j=2NUEHjk,ldjk,l+n(k,l)

Where,

djk,l and Hjk,l, j={2,…NUE} represent the Nlayer,j x1 transmitted signal vector and the (NRx x Nlayer,j) channel matrix between the j-th co-scheduled UE’s interference and the UE containing the contribution from receiver branches, with Hj=(Hj,1)H(Hj,2)H for two receiver antennas and Hj=(Hj,1)H(Hj,2)H(Hj,3)H(Hj,4)H for four receiver antennas, where, Hj,i channel-matrix of size Nlayer,j x1 for the i-th receiver antenna, respectively.

NUE is the number of paired UEs plus one (the target UE), and NUE=2 in this technical report.

The recovered Nlayer,1 x 1 signal vector at the UE, d1(k,l) is detected by using the (Nlayer,1 x NRx) receiver weight matrix WRX,1k,l as follows.

d1k,l=WRX,1k,lrk, l

### 4.2.2	MMSE IRC receiver

To suppress the co-scheduled UE’s interference, the candidate MMSE IRC receiver type is captured in this subclause. The MMSE IRC receiver weight matrix is expressed as follow:

WRX,1k,l=H1H(k,l)R-1

R= P1H1k,lH1Hk,l+1Nrek,l∈DMRS of NUEr(k,l)r(k,l)H,

rk,l=rk, l-H1(k,l)d1k,l if co-scheduled UE is in the same CDM group with target UE, and

rk,l=rk, l, if co-scheduled UE is in the different CDM group with target UE.

Where,

HjHk,l and d1(k,l) denote the estimated channel matrix and the transmit signal of target UE’s DMRS symbols, respectively, where the estimated channel matrix is also based on DMRS.

Nre is the number of sampling REs of intra-user’s DMRS.

P1 is the transmission power of the serving cell and is equal to Ed1k,l2.

## <<< End of text proposal >>