3GPP TSG-RAN WG4 Meeting # 102-e						R4-2203830

Electronic Meeting, February 21 – March 3, 2022

Source:		Verizon

Title:			TP for TR 38.841: CA_n66-n77

Agenda item:		9.30.2

Document for:		Approval

# 1. Introduction

This is a text proposal to include MSD due to cross band isolation for the approved PC2 CA_n66-n77 combo in [1].

# 2. Reference

- 3GPP TR 38.841

# 3. Text Proposal

<Start of Text Proposal>

## 5.7 	CA_n66-n77

### 5.7.1	Configurations

Table 5.7.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n66A-n77A CA_n66A-n77(2A) | CA_n66A-n77A | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.7.2	Maximum output power

| Uplink CA configuration | Power class 2 cases for CA_n66A-n77A | CA power class | Carrier n66 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n66A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied. Also when the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

### 5.7.3	REFSENS requirements

According to the PC3 CA_n66A-n77A study, the 2nd and 5th order IMD products generated from dual uplinks of band 66 and n77 may fall into band 66 Rx frequency range. Thus additional MSD should be considered to mitigate the impact of the interference for the PC2 CA_n66A-n77A combination.

#### 5.7.3.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 CA_n66A-n77A are defined in table 5.7.3.1-1.

Table 5.7.3.1-1: MSD test points for PCell due to dual uplink operation for PC2 NR CA in NR FR1 (two bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD for PC2 | Duplex mode | Source of IMD |
| Configuration |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n66A-n77A CA_n66A-n77(2A) | n66 | 1730 | 5 | 25 | 2130 | 34.33 | FDD | IMD2 |
|  | n77 | 3860 | 10 | 50 | 3860 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | 11.27 | FDD | IMD5 |
|  | n77 | 3660 | 10 | 50 | 3660 | N/A | TDD | N/A |

#### 5.7.3.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B CA_n66A-n77A are same as the Case A defined in table 5.7.3.1-1.

For CA_n66A-n77A with 1UL, Case b is applied as baseline to analyze the MSD requirements due to cross band isolation. The MSD requirements are defined in Table 5.7.3.2-1 and Table 5.7.3.2-2 when n77 transmitting with PC2.

Table 5.7.3.2-1: sensitivity exceptions (MSD) due to cross band isolation

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n66 | 10.5 | 10.5 | 10.5 | 10.5 | 10.5 | 10.5 | 10.5 |  |  |  |  |  |  |

Table 5.7.3.2-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n77 | n66 | 15 | 25 | 50 | 75 | 100 | 128 | 160 | 216 |  |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.7.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n66A-n77A requirements are applied for PC2 CA_n66A-n77A.

<End of Text Proposal>