3GPP TSG-RAN WG4 Meeting # 102-e		                       R4-2203920

Electronic Meeting, February 21 – March 3, 2022

Title: 	UE RF requirements for 6GHz licensed band

Source: 	CATT

Agenda item:	9.3.3

Document for:	Approval

- Introduction

In RAN4#101-bis-e meeting, UE RF requirements for 6GHz licensed band were discussed with one WF [1] approved. This contribution will provide our views on UE RF requirements for 6GHz licensed band.

- Discussion

As per [1], the open issues of UE RF requirements are given as below:

## Maximum output power Tentative agreement: Specify PC3 1 Tx (FFS if 2Tx) as first priority and default power class. Additionally specify PC2 (1Tx) if time allows (2Tx is FFS) Note: FFS if any clarification from RCC is needed on UE max output power, this is handled in the WF on general aspects. ## MPR Way Forward: FFS, also pending on ACLR and SEM. Transceiver and PA performance as well as trace loss and component losses for this frequency range could also be further evaluated for next meeting. Further investigate if current MPR should be maintained, relaxed, or tightened. ## Spectral emission mask Tentative agreement: Option 1: Adopt the SEM requirement defined in TR 38.921 for 6425-7125MHz assuming UE PC3 is specified. Option 2: Adopt SEM as defined in sub-clause 6.5.2.2 of TS 38.101-1 assuming UE PC3 is specified ## ACLR Tentative agreement: Option 1: Adopt 26 dB ACLR for 6.425 - 7.125 GHz band (PC3 UE). Option 2: Adopt 30 dB ACLR for 6.425 – 7.125 GHz band (PC3 UE). - REFSENS Way forward: Companies are encouraged to provide further analysis on the UE NF for this frequency range and check the following options: - Option 1: Same REFSENS as n78 and n79 for the new band - Option 2: Adopt 10.5 dB NF for 6GHz NR band. - Option 3: FFS - ACS Tentative agreement: Option 1: Adopt 32 dBc adjacent channel selectivity (ACS) for 6.425 - 7.125 GHz band. Option 2: Adopt 33 dB adjacent channel selectivity (ACS) for 6.425 – 7.125 GHz band in accordance with sub-clause 7.5 of TS 38.101-1 - Blocking Way forward Further investigate the blocking requirement. Note: FFS if any clarification from RCC is needed, this is handled in the WF on general aspects.

MPR

If SEM and ACLR requirements is relaxed based on TR 38.921, the current MPR would be maintained as a starting point.

Proposal 1: To keep the current MPR requirements as a starting point.

SEM

RAN4 has made agreements on SEM requirements captured in TR 38.921 during the study phase. There is no need to revisit SEM requirements unless technical issue is identified. It is proposed to directly adopt the SEM requirement defined in TR 38.921 for 6425-7125MHz and pay more attention to other controversial issues.

Proposal 2: To adopt the SEM requirement defined in TR 38.921 for 6425-7125MHz.

ACLR

RAN4 has made agreements on ACLR requirements captured in TR 38.921 during the study phase. There is no need to revisit ACLR requirements unless technical issue is identified. It is proposed to directly adopt the ACLR requirement defined in TR 38.921 for 6425-7125MHz and pay more attention to other controversial issues.

Proposal 3: To adopt 26 dB ACLR for 6425-7125MHz. (PC3 UE).

ACS

RAN4 has made agreements on ACS requirements captured in TR 38.921 during the study phase. There is no need to revisit ACS requirements unless technical issue is identified. It is proposed to directly adopt the ACS requirement defined in TR 38.921 for 6425-7125MHz and pay more attention to other controversial issues.

Proposal 4: To introduce 32dBc ACS for 6425-7125MHz.

- Conclusion

This contribution mainly discusses UE RF requirements for 6GHz licensed band. The following proposals are derived:

Proposal 1: To keep the current MPR requirements as a starting point.

Proposal 2: To adopt the SEM requirement defined in TR 38.921 for 6425-7125MHz.

Proposal 3: To adopt 26 dB ACLR for 6425-7125MHz. (PC3 UE).

Proposal 4: To introduce 32dBc ACS for 6425-7125MHz.

- Reference

- R4-2202267, WF on UE RF requirements for 6GHz licensed band, Ericsson, RAN4#101-bis-e