3GPP TSG-RAN WG4 Meeting # 102-e 		                       R4-2203949

Electronic Meeting, February 21 – March 3, 2022

Title: 	Open issue for Rx RF requirements for SAN type 1-O

Source: 	CATT

Agenda item:	10.13.3.2

Document for:	Discussion

- Introduction

Way Forward on RF requirements for satellite access node was agreed in [1]. The following 3 Rx requirements remain open.

- OTA REFSENS

Option 1: OTA REFSENS shall not be specified

Option 2: OTA REFSENS shall be specified using the same limits than 1-H ones, adjusted with NTN ΔOTAREFSENS.

- OTA In-channel selectivity

Option 1: OTA In-channel selectivity shall be specified using the same limits as 1-H ones but adjusted with the SAN ΔminSENS  for both wanted signal and interfere values.

Option 2: OTA In-channel selectivity will not be specified.

- In-band blocking

This paper will further discuss these open issues and give our proposals.

- Discussion

  - OTA REFSENS

We don’t have a strong preference on whether to introduce this requirement or not. If OTA sensitivity requirement is enough for satellite operation, we are fine to omit this requirement.

Proposal 1: OTA REFSENS requirement can be omitted if OTA sensitivity requirement is seen enough for SAN.

  - OTA In-channel selectivity

As discussed in [2], ICS requirements are proposed to be defined as 16dB, 21dB and 24dB respectively for SAN type 1-H loaded on GEO, LEO1200 and LEO600. It is proposed that OTA In-channel selectivity shall be specified using the same limits as 1-H ones but adjusted with the SAN ΔminSENS  for both wanted signal and interfere values.

Proposal 2: It is proposed that OTA In-channel selectivity is specified using the same limits as SAN type 1-H ones but adjusted with the SAN ΔminSENS  for both wanted signal and interfere values.

  - In-band blocking level

The in-band blocking level for SAN type 1-H is discussed in [2]. It is proposed that OTA in-band blocking level is specified using the same limits as SAN type 1-H but adjusted with the SAN ΔminSENS  for both wanted signal and interfering signal.

Proposal 3:  OTA in-band blocking level is specified using the same limits as SAN type 1-H but adjusted with the SAN ΔminSENS  for both wanted signal and interfering signal.

- Conclusion

This paper further discussed satellite access node Rx RF requirements for BS type 1-O and concluded the following proposals.

Proposal 1: OTA REFSENS requirement can be omitted if OTA sensitivity requirement is seen enough for SAN.

Proposal 2: It is proposed that OTA In-channel selectivity is specified using the same limits as SAN type 1-H ones but adjusted with the SAN ΔminSENS  for both wanted signal and interfere values.

Proposal 3:  OTA in-band blocking level is specified using the same limits as SAN type 1-H but adjusted with the SAN ΔminSENS  for both wanted signal and interfering signal.

- Reference

[1]	R4-2203035, NTN - WF on Rx RF requirements for SAN type 1-O, Ericsson

[2]	R4-2203951, Open issue for Rx RF requirements for SAN type 1-H, CATT