3GPP TSG-RAN WG4 Meeting # 102-e 		                       R4-2203955

Electronic Meeting, February 21 – March 3, 2022

Title: 	TP for 38.108: clause 7.1&7.2 on Rx REFSENS

Source: 	CATT

Agenda item:	10.13.3.4

Document for:	Approval

- Introduction

This contribution provides a text proposal for 38.108: clause 7.1&7.2 on Rx REFSENS [1].

- Reference

[1] TR 38.108, 0.0.1

- Text proposal

---------------------------------------------------Start of Text proposal---------------------------------------------------------

7.1	General

Conducted receiver characteristics are specified at the TAB connector for SAN type 1-H, with full complement of transceivers for the configuration in normal operating condition.

Unless otherwise stated, the following arrangements apply for conducted receiver characteristics requirements in clause 7:

-	Requirements shall be met for any transmitter setting.

-	The requirements shall be met with the transmitter unit(s) ON.

-	Throughput requirements do not assume HARQ retransmissions.

-	When SAN is configured to receive multiple carriers, all the throughput requirements are applicable for each received carrier.

-	For ACS, blocking and intermodulation characteristics, the negative offsets of the interfering signal apply relative to the lower SAN RF Bandwidth edge or sub-block edge inside a sub-block gap, and the positive offsets of the interfering signal apply relative to the upper SAN RF Bandwidth edge or sub-block edge inside a sub-block gap.

NOTE 1:	In normal operating condition the SAN is configured to transmit and receive at the same time.

7.2	Reference sensitivity level

7.2.1	General

The reference sensitivity power level PREFSENS is the minimum mean power received at the TAB connector for SAN type 1-H at which a throughput requirement shall be met for a specified reference measurement channel.

7.2.2	Minimum requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.2.2-1 and 7.2.2-2 for SAN type 1-H in all operating band in FR1.

Table 7.2.2-1: SAN reference sensitivity levels (GEO payload)

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -99.3 |
| 10, 15 | 30 | G-FR1-A1-2 (Note 1) | -99.4 |
| 10, 15 | 60 | G-FR1-A1-3 (Note 1) | -96.5 |
| 20 | 15 | G-FR1-A1-4 (Note 1) | -92.9 |
| 20 | 30 | G-FR1-A1-5 (Note 1) | -93.2 |
| 20 | 60 | G-FR1-A1-6 (Note 1) | -93.3 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. |  |  |  |

- Table 7.2.2-2: SAN reference sensitivity levels (LEO payload)

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -102.4 |
| 10, 15 | 30 | G-FR1-A1-2 (Note 1) | -102.5 |
| 10, 15 | 60 | G-FR1-A1-3 (Note 1) | -99.6 |
| 20 | 15 | G-FR1-A1-4 (Note 1) | -96.0 |
| 20 | 30 | G-FR1-A1-5 (Note 1) | -96.3 |
| 20 | 60 | G-FR1-A1-6 (Note 1) | -96.4 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. |  |  |  |

---------------------------------------------------End of Text proposal---------------------------------------------------------