3GPP TSG-RAN WG4 Meeting #102-e                                                                                      R4-2203975

Electronic Meeting, February 21 – March 3, 2022

Source: 	CATT

Title: 	Discussion on the BS TX RF requirements for 52.6-71GHz

Agenda item:	10.16.4.1

Document for:	Approval

- Introduction

In RAN4#101-bis-e, a WF [1] on BS RF TX requirements for 52.6 – 71 GHz was approved. The TAE, EVM and upper bound of FDL,high – FDL,low for ΔfOBUE  need to be further discussed

In this contribution, we provide our views on the remaining issues in the WF.

- Discussion

TAE

As per the WF [1], the remaining issues concerning TAE are shown as below:

TAE: - Inter-band CA: 3us. - Contiguous intra-band CA (baseline assumption): 65 ns for 480 kHz SCS, 32.5 ns for 960 kHz SCS. RAN4 will make final decision on February RAN4#102-e meeting. - Non-contiguous intra-band CA: [260 ns]. (Note: Whether the requirement need to be specified in Rel-17 specification pending on further confirmation whether NC CA is within Rel-17 scope). - MIMO: Option 1 baseline assumption; RAN4 will make final decision on February RAN4#102-e meeting. - Option 1: 65 ns for 480 kHz SCS, 32.5 ns for 960 kHz SCS. - Option 2: 32.5 ns for 480 kHz, 16.25 ns for 960kHz SCS.

Figure 2-1: The impact of TAE on throughput for 480kHz SCS(left) and 960 kHz SCS(right),

The simulation results with different TAEs for two layers transmission without HARQ are shown in Figure 2-1.  And the link level simulation assumptions for TAE are shown in Table A5-1 in annex. From Figure 2-1, it is observed that,

- For 480kHz SCS at TDLA with 5ns delay spread:

  - For 65ns TAE, throughput can’t reach 70% of maximum throughput.

  - For 32.5ns TAE, throughput is close to that of 0ns TAE, and throughput can reach 70% of maximum throughput.

- For 960kHz SCS at TDLA with 5ns delay spread:

  - For 32.5ns TAE, throughput can reach 70% of maximum throughput, but throughput has about 5dB performance degradation relative to that of 0ns TAE.

  - For 16.25ns TAE, throughput s close to that of 0ns TAE, and throughput can reach 70% of maximum throughput.

Based on above simulation analysis, for MIMO TAE, 32.5 ns for 480 kHz and 16.25 ns for 960kHz SCS(Option 2) is more reasonable.

Proposal 1: For MIMO TAE，to adopt 32.5 ns for 480 kHz and 16.25 ns for 960kHz SCS(Option 2).

OBUE

As per the WF [1], the remaining issues concerning OBUE are shown as below:

OBUE: - ΔfOBUE = 3500 MHz for FDL,high – FDL,low > 4000 MHz for 52.6-71GHz. - FFS for the upper bound of FDL,high – FDL,low.

Figure 2-１: FR2 approach spurious emission mask for frequency >=18GHz[3].

The FR2 approach for frequency >=18GHz is referenced to Figure 7 in ERC Recommendation 74-01[3], the spurious emission mask is shown in Figure 2-1. the step frequencies can be calculated by the following equation.

WB = FDL,high-FDL,low

Fstep,1 = max(18, FDL,low-max(10,4*WB))

Fstep,2 = FDL,low-max(2,WB)

Fstep,3 = FDL,low-ΔfOBUE

Fstep,4 =FDL,high+ΔfOBUE

Fstep,5 =FDL,high+max(2,WB)

Fstep,6 =FDL,high+max(10,4*WB)

For n263, the WB = FDL,high-FDL,low = 71-57 = 14GHz.  When frequency is out of 14GHz, it is spurious emission range. It is reasonable to set upper bound of FDL,high – FDL,low to 14GHz

Proposal 2: To specify 4000 MHz < FUL_high – FUL_low ≤14000 MHz.

- Conclusion

This contribution provides analysis on BS TX RF requirements for 52.6-71GHz. The following proposals and observations are concluded as follows:

Proposal 1: For MIMO TAE，to adopt 32.5 ns for 480 kHz and 16.25 ns for 960kHz SCS(Option 2).

Proposal 2: To specify 4000 MHz < FUL_high – FUL_low ≤14000 MHz.

- References

- R4-2203016, WF on BS RF Tx requirements, Nokia, Nokia Shanghai Bell, RAN4#101-bis-e

- Recommendation ITU-R M.2003-2, “Multiple Gigabit Wireless Systems in frequencies around 60 GHz”, ITU-R

- ERC Recommendation 74-01, unwanted emissions in the spurious domain, ECC

- Annex

The link level simulation assumptions for TAE are shown in Table A5-1.

Table A5-1 link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 60GHz |
| CBW | 400MHz |
| SCS | 480kHz, 960kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 5ns delay spread, Maximum Doppler frequency: 5Hz |
| MCS | MCS 16 in 64QAM table of TS 38.214 |
| Pre-coding | Fixed PMI |
| Symbol type | CP-OFDM |
| HARQ | NO |
| RANK | 2 |
| BS antenna configuration | 2 |
| UE antenna configuration | 2 |
| Antenna correlation (Tx and Rx) | Low correlation |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PDSCH configuration | Type A mapping, Start symbol 0, Duration 14 (for D slots) |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | NO |
| PN model | NO |
| PN compensation | NO |