3GPP TSG-RAN WG4 Meeting #102-e                                                                                                      R4-2203976

Electronic Meeting, February 21 – March 3, 2022

Source: 	CATT

Title: 	Discussion on the BS RX RF requirements for 52.6-71GHz

Agenda item:	10.16.4.2

Document for:	Approval

- Introduction

In RAN4#101-bis-e, a WF [1] on BS RF RX requirements for 52.6 – 71 GHz was approved. The ACS interfering signal type and ACS interferer frequency offset need to be further discussed.

In this contribution, we provide our views on the remaining issues in the WF [1].

- Discussion

ACS

As per the WF [1], the remaining issues concerning ACS are shown as below:

ACS: For ACS define interfering signal type as 100 MHz DFT-s-OFDM NR signal, 120 kHz SCS, 32 RBs. Confirm the PRB value once SU is agreed.

The ACS interfering signal type and ACS interferer frequency offset for BS type 1-O in table 10.5.1.2-2 and BS type 2-O in table 10.5.1.3-2 of TS 38.104 are copied as below.

Table 10.5.1.2-2: OTA ACS interferer frequency offset for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±2.5025 |  |
| 10 | ±2.5075 | 5 MHz DFT-s-OFDM NR signal, |
| 15 | ±2.5125 | 15 kHz SCS, 25 RBs |
| 20 | ±2.5025 |  |
| 25 | ±9.4675 |  |
| 30 | ±9.4725 |  |
| 40 | ±9.4675 |  |
| 50 | ±9.4625 | 20 MHz DFT-s-OFDM NR signal, |
| 60 | ±9.4725 | 15 kHz SCS, 100 RBs |
| 70 | ±9.4675 |  |
| 80 | ±9.4625 |  |
| 90 | ±9.4725 |  |
| 100 | ±9.4675 |  |

Table 10.5.1.3-2: OTA ACS interferer frequency offset for BS type 2-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 50 | ±24.29 |  |
| 100 | ±24.31 | 50 MHz DFT-s-OFDM NR |
| 200 | ±24.29 | signal,60 kHz SCS, 64 RBs |
| 400 | ±24.31 |  |

From above type of interfering signal  in table 10.5.1.2-2 and table 10.5.1.3-2 of TS 38.104, for FR1, the RBs for 5 MHz DFT-s-OFDM NR signal with 15 kHz SCS is 25, and RBs for 20 MHz DFT-s-OFDM NR signal with 15 kHz SCS is 100. And for FR2, RBs for 50MHz DFT-s-OFDM NR signal with 60 kHz SCS is 64.

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 93 | 107 | 121 | 135 |

Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |

The transmission bandwidth configurations NRB for FR1 in table 5.3.2-1 and FR2 in table 5.3.2-1 of TS 38.104 are copied as below. For FR1 OFDM, RBs for 5MHz CBW with 15kHz SCS is 25, and its RBs for DFT-s-OFM is also 25. RBs for 20MHz CBW with 15kHz SCS is 106, and its RBs for DFT-s-OFM is 100. For FR2 OFDM, RBs for 50MHz CBW with 60kHz SCS is 66 and its RBs for DFT-s-OFM is 64.

From above discussion, RBs for ACS interfering signal should be full transmission bandwidth configuration of DFT-s-OFDM NR signal instead of part of transmission bandwidth configuration. From the above table 5.3.2-2, RBs for 100MHz CBW with 120kHz SCS is 66 and its RBs for DFT-s-OFM is 64. So, for ACS interfering signal type for 52.6GHz, RBs for 100 MHz DFT-s-OFDM NR signal with 120 kHz SCS should be 64 instead of 32.

Observation 1: For ACS interfering signal type for 52.6GHz, RBs for 100 MHz DFT-s-OFDM NR signal with 120 kHz SCS should be 64 instead of 32.

Proposal 1:  To specify 100 MHz DFT-s-OFDM NR signal, 120 kHz SCS, 64 RBs as ACS interfering signal type for FR2-2.

Referring to section 7.4.1 of TR 38.817-2[2], the interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz)  can be calculated by the following equation(1).

(1)

Where,

is the channel bandwidth of the interfering signal in MHz.

is required frequency offset increment in MHz.

(2)

is subcarrier spacing of the interfering signal in Hz.

(3)

is nominal frequency offset in MHz, which is is equal to half of the sum of the channel bandwidth of the wanted signal and the channel bandwidth of the interfering signal, shown as following equation.

(4)

is channel bandwidth of the wanted signal in MHz.

is required frequency offset reduction, because the number of RBs of DFT-s-OFDM is reduced relative to  that of OFDM, shown as following equation.

(5)

is RB number of OFDM NR signal  for subcarrier spacing of the interfering signal.

is RB number of DFT-s-OFDM NR signal  for subcarrier spacing of the interfering signal.

Using equation (1), the ACS interferer frequency offset, ACS interferer frequency offsets for FR2-1 and FR2-2 are shown in table 2-1.

Table 2-1 ACS interferer frequency offsets for FR2-1 and FR2-2.

| Frequency range | BS channel bandwidth of the lowest/highest carrier received (MHz) | Type of interfering signal | BWWanted(MHz) | SCS(Hz) | BWInterfer（MHz) | FInterfer(MHz) | BWInterferer/2+FInterfer | NRB,OFDM | NRB,DFT-s-OFDM | FDFT-s-OFDM | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2-1 | 50 | 50 MHz DFT-s-OFDM NR signal, 60 kHz SCS, 64 RBs | 50 | 60000 | 50 | 0.01 | 25.01 | 66 | 64 | 0.72 | 24.29 |
|  | 100 |  | 100 | 60000 | 50 | 0.03 | 25.03 | 66 | 64 | 0.72 | 24.31 |
|  | 200 |  | 200 | 60000 | 50 | 0.01 | 25.01 | 66 | 64 | 0.72 | 24.29 |
|  | 400 |  | 400 | 60000 | 50 | 0.03 | 25.03 | 66 | 64 | 0.72 | 24.31 |
| FR2-2 | 100 | 100 MHz DFT-s-OFDM NR signal, 120 kHz SCS, 64 RBs | 100 | 120000 | 100 | 0.02 | 50.02 | 66 | 64 | 1.44 | 48.58 |
|  | 400 |  | 400 | 120000 | 100 | 0.02 | 50.02 | 66 | 64 | 1.44 | 48.58 |
|  | 800 |  | 800 | 120000 | 100 | 0.06 | 50.06 | 66 | 64 | 1.44 | 48.62 |
|  | 1600 |  | 1600 | 120000 | 100 | 0.02 | 50.02 | 66 | 64 | 1.44 | 48.58 |
|  | 2000 |  | 2000 | 120000 | 100 | 0.06 | 50.06 | 66 | 64 | 1.44 | 48.62 |

Based on above analysis, we propose,

Proposal 2: To adopt the following ACS interferer frequency offsets for FR2-2.

Table 10.5.1.3-2: OTA ACS interferer frequency offset for BS type 2-O

| Frequency Range | BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- |
| FR2-1 | 50 | ±24.29 |  |
|  | 100 | ±24.31 | 50 MHz DFT-s-OFDM NR |
|  | 200 | ±24.29 | signal,60 kHz SCS, 64 RBs |
|  | 400 | ±24.31 |  |
| FR2-2 | 100 | ±48.58 | 100 MHz DFT-s-OFDM NR signal,120 kHz SCS, 64RBs |
|  | 400 | ±48.58 |  |
|  | 800 | ±48.62 |  |
|  | 1600 | ±48.58 |  |
|  | 2000 | ±48.62 |  |

- Conclusion

This contribution provides analysis on BS RX RF requirements for 52.6-71GHz. The following proposals and observations are concluded as follows:

Observation 1: For ACS interfering signal type for 52.6GHz, RBs for 100 MHz DFT-s-OFDM NR signal with 120 kHz SCS should be 64 instead of 32.

Proposal 1:  To specify 100 MHz DFT-s-OFDM NR signal, 120 kHz SCS, 64 RBs as ACS interfering signal type for FR2-2.

Proposal 2: To adopt the following ACS interferer frequency offsets for FR2-2.

Table 10.5.1.3-2: OTA ACS interferer frequency offset for BS type 2-O

| Frequency Range | BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- |
| FR2-1 | 50 | ±24.29 |  |
|  | 100 | ±24.31 | 50 MHz DFT-s-OFDM NR |
|  | 200 | ±24.29 | signal,60 kHz SCS, 64 RBs |
|  | 400 | ±24.31 |  |
| FR2-2 | 100 | ±48.58 | 100 MHz DFT-s-OFDM NR signal,120 kHz SCS, 64RBs |
|  | 400 | ±48.58 |  |
|  | 800 | ±48.62 |  |
|  | 1600 | ±48.58 |  |
|  | 2000 | ±48.62 |  |

- References

- R4-2203017, WF on BS RF Rx requirements, Ericsson, RAN4#101-bis-e

- TR38.817-2.