3GPP TSG-RAN WG4 Meeting #102-e                                                    R4-2204011

# Electronic Meeting, February 21 - March 3, 2022

Source:	ZTE Corporation

Title:	TP to TR 38.862 on template of delta TIB and RIB tables

Agenda Item:	11.3.3.1

Document for:	Approval

1	Introduction

In RAN4#101-e meeting, a new template for delta TIB and RIB tables has been approved in [1]. In this proposal, a TP to capture the new template for delta TIB and RIB tables is proposed.

Proposal 1:	 It is suggested to adopt the following TP for the new template of delta TIB and RIB tables in TR 38.862.

2	References

[1] R4-2120019, WF on approaches for delta TIB and RIB optimization, ZTE

Text Proposal

<< begin text proposal >>

<< Unchanged sections omitted >>

8.3.2.x	New template for ΔTIB,c and ΔRIB,c tables in Rel-18

For the UE which supports CA/DC configurations, the allowed maximum configured output power relaxation has been set for the inter-band or SUL operation as ΔTIB,c. The allowed reference sensitivity relaxation has been set for the inter-band operation as ΔRIB,c. However, with the explosive growth of the number of combinations, the ΔTIB,c and ΔRIB,c tables in the specifications are seriously oversized and the readability is deteriorated. To optimize the tables of ΔTIB,c and ΔRIB,c, a new template in Table 8.3.2.x-1 and Table 8.3.2.x-2 is proposed in Rel-18 respectively.

Table 8.3.2.x-1: New template for ΔTIB,c tables in Rel-18

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration |  |  |  |  |
| DC_1-3-7-40_n78 | 0.6 | 0.6 | 0.5 | 0.3 | 0.8 |
| DC_1-3-8-11_n28 | 0.3 | 0.8 | 0.6 | 0.9 | 0.6 |
| … | … | … | … | … | … |
| Note 1: “-” denotes ΔTIB,c = 0. Note 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-48_(n)5 the band order from left to right is 2, 5, 48 and n5. |  |  |  |  |  |

Table 8.3.2.x-2: New template for ΔRIB,c tables in Rel-18

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration |  |  |  |  |
| DC_1-3-7-40_n78 | 0.2 | 0.2 | - | 0.4 | 0.5 |
| DC_1-3-8-11_n28 | - | 0.3 | 0.2 | 0.5 | 0.2 |
| … | … | … | … | … | … |
| Note 1: “-” denotes ΔRIB,c = 0. Note 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-48_(n)5 the band order from left to right is 2, 5, 48 and n5. |  |  |  |  |  |

<< end text proposal >>