3GPP TSG-RAN WG4 Meeting #102-e	R4-2204015

21st February – 3rd March 2022

Source:	Qualcomm Incorporated

Title:	RF switching for V2X intra-band con-current operation with different carriers in TDD bands

Agenda item:	10.15.3.2

Document for:	Approval

# 1.	Introduction

In RAN4#101-bis-e the need for a RF switching time test was discussed given that the RRM session has already agreed to define a scheduling restriction calculation for TDM based intra-band con-current SL operation with 1 slot as baseline for Uu to SL switching. This issue was left for further discussion for RAN4#102-e.  In this paper we would like to further discuss issues related to this topic.

# 2. 	Discussion

In RAN4#101-bis-e it was discussed whether RF switching time needs to be tested as the RRM session had agreed in RAN4#101-e [1] that the scheduling restriction calculation be based on 1 slot as baseline for Uu to SL switching. Furthermore, it was agreed in RAN4#101-bis-e [2] that the scheduling availability requirement for Uu to SL switching would be defined similar to that for intra-RAT LTE SL and NR SL switching where the UE is not expected to transmit or receive on either the Uu or SL link on the slot in which it switches. Based on this agreement it is seen that as 1 slot is much larger than the RF switching time that no test is required for verifying the RF switching time. Based on these agreements the following proposal is made

Proposal 1: No RF test is required for testing Uu to SL or SL to Uu TDM intra-band con-current RF switching time.

We think that the above agreement should be captured in TR38.785

Proposal 2: The agreement stating that a test for intra-band con-current RF switching time is not required should be captured in TR38.785

Also, an LS should be sent to RAN5 informing them of the RAN4 agreement requiring no RF test for intra-band Uu to SL or SL to Uu switching.

Proposal 3: Send LS to RAN5 stating that no RF test is required for Uu to SL or SL to Uu TDM intra-band con-current RF switching time.

# Conclusion

In this paper the need for testing RF switching time was discussed and the following proposals are made:

Proposal 1: No RF test is required for testing Uu to SL or SL to Uu TDM intra-band con-current RF switching time

Proposal 2: The agreement stating that a test for intra-band con-current RF switching time is not required should be captured in TR38.785

Proposal 3: Send LS to RAN5 stating that no RF test is required for Uu to SL or SL to Uu TDM intra-band con-current RF switching time.

# References

[1] R4-2120315, WF on NR SL enhancements RRM requirements, RAN4#101-e, November 2021

[2] R4-2202732, 3GPP TSG-RAN WG4 Meeting # 101bis-e, January 2022