3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204303

Electronic Meeting, February 21 – March 3, 2022

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to scheduling availability of UE during RSTD measurement without gap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  |  | Date: |  | 2022-02-14 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RSTD measurement without gap is introduced in Rel-17. The corresponding measurement requirements are missed in the current spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the scheduling availability of UE during RSTD measurement without gap, based on the following RAN1 conclusions. Working assumption: Subject to UE capability, support PRS measurement outside the MG, within a PRS processing window, and UE measurement inside the active DL BWP with PRS having the same numerology as the active DL BWP. - Inside the PRS processing window, subject to the UE determining that DL PRS to be higher priority, support the following UE capabilities: - Capability 1: PRS prioritization over all other DL signals/channels in all symbols inside the window. - Cap. 1A: The DL signals/channels from all DL CCs (per UE) are affected. - Cap. 1B: Only the DL signals/channels from a certain band/CC are affected. - FFS: band or CC - Capability 2: PRS prioritization over other DL signals/channels only in the PRS symbols inside the window - A UE shall be able to declare a PRS processing capability outside MG. - FFS: Details of capability signalling (e.g., per UE or per band, etc.) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The scheduling availability of UE during RSTD measurement without gap will be incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# < Start of change 1>

### 9.9.2.8	Scheduling availability of UE during RSTD measurement without gap

The UE shall be capable of measuring PRS from a neighbouring cell within a PRS processing window but without measurement gap when

- the PRS resource is contained within the active DL BWP with the same numerology as the active DL BWP, and

- the expected Rx timing difference between neighbouring cell and the serving cell is smaller than [TBD].

  - The expected Rx timing difference is determined by expected RSTD and expected RSTD uncertainty in the assistance data.

For a UE indicating PRS is higher priority than all PDCCH/PDSCH/TRS/CSI-RS, the following scheduling availability applies:

- When declaring capability [cap. 1A] for PRS measurement outside gap, the UE is not expected to receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within PRS processing window.

- When declaring capability [cap. 1B] for PRS measurement outside gap, the UE is not expected to receive PDCCH/PDSCH/TRS/CSI-RS for CQI from a certain [band or CC] on all symbols within PRS processing window.

- When declaring capability [cap. 2] for PRS measurement outside gap, the UE is not expected to receive PDCCH/PDSCH/TRS/CSI-RS for CQI on the PRS symbols within PRS processing window.

FFS: scheduling availability for a UE indicating PRS measurement is lower priority than PDCCH/PDSCH/TRS/CSI-RS.

FFS: whether RLM, BFD and L1/L3 measurement is higher priority over PRS measurement.

# < End of change 1>