3GPP TSG-RAN WG4 Meeting #102-e				     		 R4- 2204333

Electronic Meeting, February 21 - March 3, 2022

Agenda item:			10.13.2

Source:	Samsung

Title:	Draft text proposal to update TR 38.863 Chapter 6

Document for:	Approval

# Introduction

This contribution proposed draft text proposal to update Chapter 6 of TR 38.863.

The WF of RAN4 101-bis-e R4-2203130 agreed on the consideration of Scenario 6 results and the ACLR and ACS for NR-NTN SAN. And this contribution is to propose the corresponding changes to reflect these agreements, and some editorial changes in Chapter 6.

# Reference

[1] R4-2203130, WF on [307] NTN_Solutions_Part2, Samsung

[2] R4-2203132, draft TR for 38.863v0.2.0, Samsung

# Text proposal for TR38.863

---------------------------------------------<Start of Change>--------------------------------------------

## 6.1	Co-existence simulation scenario

Scenarios for coexistence study are listed in Table 6.1-1.

Table 6.1-1 Scenarios for NTN-NTN/TN co-existence

| FR1: 2GHz |  | NTN1,4,5 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Set 1 |  |  | Set 22 |  |  | HAPS |
|  |  | GEO3 | LEO 600km | LEO 1200km | GEO | LEO 600km | LEO 1200km |  |
| NR / NB-IoT | Rural | X | X | X | X | X | X | FFS |
|  | Urban macro | X | X | X | X | X | X | FFS |
|  | Dense Urban6 | N/A | N/A | N/A | N/A | N/A | N/A | FFS |
| HAPS |  | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Note 1: Earth Fixed beam has been considered for co-existence studies. Note 2: Set 1 satellite antenna has been used for co-existence studies. Note 3: GEO and LEO only operate at adjacent channel. Note 4: Use GEO and LEO@600km when TN is victim. Note 5: The satellite-to-satellite coexistence scenarios are not in the scope of this study considering this is already addressed by ITU (ITU RR Article 9 etc.) and regional regulations (e.g. FCC rules). Note 6: Dense Urban is not considered as it is expected NTN UE will connect to terrestrial networks rather than satellite networks in such scenario. |  |  |  |  |  |  |  |  |

-----------------------------------<Unchanged sections omitted>------------------------------------

The frequency and bandwidth are listed in table 6.1-3.

Table 6.1-3.  Proposed frequency and bandwidth for co-existence study

|  | Frequency | Bandwidth | Duplex mode | Frequency reuse factor |
| --- | --- | --- | --- | --- |
| TN Rural | 2 GHz | 20MHz | FDD, TDD | 1 |
| TN Urban macro | 2 GHz | 20MHz | FDD, TDD | 1 |
| GEO | 2 GHz | 5/10/15/20 MHz for FR1 | FDD | 1, 31 |
| LEO | 2 GHz | 5/10/15/20 MHz for FR1 | FDD | 1, 31 |
| HAPS | 2 GHz | TBD | FDD | 1 |
| Note 1: Only FRF=1 has been used in co-existence studies for simplification. |  |  |  |  |

-----------------------------------<Unchanged sections omitted>------------------------------------

The beam layout definition for a single satellite simulation in S-Band is defined in Table 6.2.2.1-1.

Table 6.2.2.1-1: Beam layout definition for single satellite simulation

| …… | …… |
| --- | --- |
| Central beam bore sight direction definition | Baseline: Case 1: Central beam center is considered at nadir point Case 2: 45° for GEO and LEO |
| …… | …… |

-----------------------------------<Unchanged sections omitted>------------------------------------

Table 6.2.2.1-4 Other parameters for NTN

| Parameters | NTN | Remark |
| --- | --- | --- |
| Carrier frequency | 2GHz |  |
| The number of active UE (UL) | 9 UEs and 2RBs per UE for GEO and LEO1 |  |
| The number of active UE (DL) | 1 | Same with TN |
| Traffic model | Full buffer |  |
| DL power control | NO |  |
| UL power control | See Session 2.6.2 |  |
| NTN satellite Noise figure in dB | See Table 2.3-3-1 |  |
| Handover margin | 3dB |  |
| Note 1: UEs are equally splitted inside the channel bandwidth into ACIR 3 regions. Scheduled PRB position for UE1 per satellite beam should be also fully aligned to simulate the worst case for co-channel interference and this is also aligned with full buffer case. |  |  |

-----------------------------------<Unchanged sections omitted>------------------------------------

#### 6.2.2.4	TN parameters

TN parameters for co-existence study are given in Table 6.2.2.4-1, 6.2.2.4-2 and 6.2.2.4-3.

Table 6.2.2.4-1 Simulation assumptions of TN respectively based on NB-IoT and NR

|  | NB-IoT standalone | NR |
| --- | --- | --- |
| …… | …… | …… |
| Network layout | 19-sites [57 sectors] with wrap-around | 19-sites 57 sectors with wrap-around |
| Inter-site distance in meter | 500 for 2GHz band for UMA TBD For Rural | Deployment scenario related, see Table 2.3-6 |
| System loading and activity | Full buffer 100% | See Table 6.2.1.1-1 |
| Network location | TBD | See Table 6.2.1.1-1 |
| …… | …… | …… |

-----------------------------------<Unchanged sections omitted>------------------------------------

### 6.2.4	ACIR model

ACLR modelling for TN and NTN co-existence study referring to clause 5.1.1.4.1 and 5.1.1.4.2 in TR 36.942[8] is used as baseline. The number of RBs refers to Table 6.2.2.1-4 and Table 6.2.2.4-1 respectively.

-----------------------------------<Unchanged sections omitted>------------------------------------

#### 6.2.6.2	NTN UL TPC

For the coexistence study, the same TPC model of TN for NTN UL scenarios is adopted to align with UE UL power control parameters used in TR 38.821[6].

-----------------------------------<Unchanged sections omitted>------------------------------------

### 6.2.9	Throughput ~ SNR mapping

Adopt Section 5.2.7 of TR 38.803[20] as the SINR-Throughput performance metrics.

-----------------------------------<Unchanged sections omitted>------------------------------------

## 6.4	Co-existence simulation results

In order to process the co-existence simulation results received for all different scenarios and assumptions, the following steps are adopted:

-	Step 1: Discuss and agree on the most stringent scenario(s) for each scenario (Scenario 1, 2, 3…,6);

-	Step 2: Discuss and determine the required ACIR from results of the most stringent case(s) for each scenario;

-	Step 3: Use equation to derive corresponding ACLR or ACS from the agreed ACIR for each scenario

It is noted that the averaged ACIR for the most stringent case in each scenario would be derived by taking the average among the interpolated ACIR results derived from each company’s results for that case.

Moreover, the following considerations are adopted to deal with major disputes for the worst case results in each scenario:

-	If the required ACIR results, from the contributor who did not participate or their results is still not well-aligned in calibration table, has a difference larger than 10 dB with most others, this result can be not considered in the discussion.

-	If the required ACIR results, from one contributor, has a difference larger than 10 dB with most others, this result can be not considered in the discussion.

The following sub-clauses of this section captures the processed results by adopting above principles and methodologies for scenarios 1 to 6 which are identified in Table 6.1-2. It is noted that due to the space limitation, only part of the simulation results for each case are presented, the whole results for all studied options, as listed in Table 6.1-1 and section 6.2, can be found in Annex C.

Table 6.4-1 Worst case option for each scenario

| Scenario | Aggressor system | Victim system | Environment | Contributing |
| --- | --- | --- | --- | --- |
| 1 | TN DL | NTN GEO DL | Urban | NTN UE ACS |
| 2 | TN UL | NTN GEO UL | Urban | NTN SAN ACS |
| 3 | NTN LEO-600 DL | TN DL | Rural | NTN SAN ACLR |
| 4 | NTN GEO UL | TN UL | Urban | NTN UE ACLR |
| 5 | NTN GEO UL | TN DL | Rural | NTN UE ACLR |
| 61 | NR-TN DL | NTN LEO-600 UL | Rural2 | NTN SAN ACS |
| NOTE 1: Agreed representative case for Scenario 6. NOTE 2: Some results suggested that the NR-NTN SAN would suffer more interference in urban deployment scenario. It was further discussed and agreed that simply using 20% active rate and assuming fully deployed NR stations with urban deployment assumptions in the whole NR-NTN beam coverage area would overestimate the density and aggregated interference from NR stations to NR-NTN. |  |  |  |  |

-----------------------------------<Unchanged sections omitted>------------------------------------

### 6.4.6	Scenario 6: TN DL interfering NTN UL

The co-ex results from all concerned options in this scenario were evaluated, and some results suggested that the NR-NTN SAN would suffer more interference in urban deployment scenario. It was further discussed and agreed that simply using 20% active rate and assuming fully deployed NR stations with urban deployment assumptions in the whole NR-NTN beam coverage area would overestimate the density and aggregated interference from NR stations to NR-NTN in this scenario. In the end, it has been agreed to select the NR DL equipped with non-AAS antenna interfering the NR-NTN LEO-600 UL that deployed in Rural environment as the representative case.

Table 6.4.6-1 Simulation results for average throughput loss

| ACIR[dB] | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 | 38 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| THALES |  | 51.17 |  | 41.28 |  | 31.94 | 23.87 | 17.01 | 11.87 | 7.91 | 5.26 | 3.4 | 2.22 |
| MTK |  | 3.05 |  | 1.98 |  | 1.28 | 0.82 | 0.52 | 0.33 | 0.21 | 0.13 | 0.08 | 0.05 |

Figure 6.4.6-1 Simulation results for average throughput loss

Table 6.4.6-2 Simulation results for 5%-tile throughput loss

| ACIR[dB] | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 | 38 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| THALES | 53.49 | 43.02 | 33.72 | 25.58 | 17.44 | 12.79 | 8.14 | 5.81 | 3.49 | 2.33 |
| MTK | 8.85 | 5.78 | 3.73 | 2.39 | 1.52 | 0.97 | 0.61 | 0.39 | 0.24 | 0.15 |

Figure 6.4.6-2 Simulation results for 5%-tile throughput loss

Table 6.4.6-3 Interpolated ACIR values for Scenario 5 to meet the 5% throughput loss criteria

| Source |  |  |  | Interpolated ACIR[dB] |  |
| --- | --- | --- | --- | --- | --- |
| THALES |  | Average |  | 36.69 |  |
|  |  | 5%-tile |  | 36.7 |  |
| MTK |  | Average |  | 19.66 |  |
|  |  | 5%-tile |  | 24.671 |  |
| NOTE 1: According to the principles, this value is not treated for later process. |  |  |  |  |  |

Table 6.4.6-4 Average ACIR values in the above worse case for Scenario 6

|  | Scenario 6 |
| --- | --- |
| ACIR value [dB] | 36.7 |

## 6.5	Summary of co-existence study

This sub-clause captures the summary of the co-existence studies. The averaged interpolate ACIR values for each scenario are presented in the table below.

Table 6.5-1 Average ACIR values for each scenario

| Scenario | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| ACIR value [dB] | 23.18 | 28.03 | 23.32 | 28.11 | 26.43 | 36.7 |

Then, by considering the following ACLR and ACS of TN BS and UE in Table 6.5-2, the suggested ACLR and ACS of NTN SAN and UE from each scenario are given in Table 6.5-3. It should be noted that the values in Table 6.5-3 are directly derived from the worst case option of each scenario, and it is limited by the nature of assumptions and methodologies adopted in the co-ex studies.

Table 6.5-2 ACLR and ACS of TN

| TN |  | Values |
| --- | --- | --- |
| BS | ACLR | 45 dB |
|  | ACS | 46 dB |
| UE | ACLR | 30 dB |
|  | ACS | 33 dB |

Table 6.5-3 Co-ex results suggested ACLR and ACS of NR-NTN

| Scenario | Contributing | ACLR ACS values |
| --- | --- | --- |
| 1 | NTN UE ACS | 23.21 dB |
| 2 | NTN SAN ACS | 32.41 dB |
| 3 | NTN SAN ACLR | 23.81 dB |
| 4 | NTN UE ACLR | 28.18 dB |
| 5 | NTN UE ACLR | 27.51 dB |
| 6 | NTN SAN ACS | 37.4 dB |

Considering the above suggested values, the agreed ACLR and ACS of NR-NTN are given in Table 6.5-3.

Table 6.5-3 ACLR and ACS of NR-NTN

| NR-NTN |  | Values |
| --- | --- | --- |
| SAN | ACLR | 24 dB |
|  | ACS | 38 dB |
| UE | ACLR | 30 dB |
|  | ACS | 33 dB |

---------------------------------------------<End of Change>---------------------------------------------