3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204367

Electronic Meeting, February 21 – March 3, 2022

Agenda Item: 10.19.3.3

Source: MediaTek Inc.

Title: Discussion on general and RRM requirements impacts in R17 feMIMO

Document for: Discussion

- Introduction

According to the WF [1] in the last meeting, RAN4 agreed to define the requirement for TRP specific BFR. In this paper, our views on the TRP specific BFR is provided.

- Discussion

There are two open issues for TRP specific BFR.

Issue: Requirement for TRP specific Beam Failure Recovery - Agreements - In FR2, introduce sharing factor PTRP for BFD and CBD evaluation period when BFD-RS or CBD-RS from 2 TRPs are overlapped on the same symbol. - FFS: whether to introduce the sharing factor PTRP for FR1. - FFS: value and conditions to apply the sharing factor PTRP.

First open issue: whether to introduce the sharing factor PTRP for FR1

For the FR1, the sharing factor is not needed. Because the omni-directional antenna is applied in FR1, the two BFD-RSs transmitted from two TRPs in the same OFDM symbol can be received by UE.

Proposal 1: For TRP specific BFR, for FR1, not to introduce the scaling factor PTRP to extend the evaluation period of BFD and CBD.

Second open issue: value and conditions to apply the sharing factor PTRP

The same PTRP should be applied for BFD-RS and CBD-RS. Because both BFD-RS and CBD-RS may be transmitted from two different TRPs. For convenience, the following discussion will base on BFD-RS and same analysis will apply for CBD-RS.

According to the agreement in RAN1 as below, there are 2 BFD-RS sets will be used per BWP.

Agreement in RAN1 #104-e for reference

Agreement For M-TRP BFR - Support 2 BFD-RS sets per BWP, and up to N resources per BFD-RS set - FFS: value of N (e.g. fixed in specification, or UE capability) - FFS: number of BFD RSs across all BFD-RS sets per DL BWP (e.g. fixed maximum value or UE capability)

To our understanding, typically, two different BFD-RS sets will be applied for two different TRPs, respectively. Thus, the PTRP should be 2 to extend the evaluation period for the case when two BFD-RSs are transmitted in the same OFDM symbol.

Proposal 2: For the TRP specific BFR in FR2, PTRP is 2.

- Summary

In this paper, the discussion of RRM impact in R17 feMIMO is provided. We have the following proposal:

Proposal 1: For TRP specific BFR, for FR1, not to introduce the scaling factor PTRP to extend the evaluation period of BFD and CBD.

Proposal 2: For the TRP specific BFR in FR2, PTRP is 2.

- Reference

- R4-2202668, WF on other FeMIMO RRM requirements, Intel, RAN4 #101bis-e