3GPP TSG-RAN WG4 Meeting #102-e	R4-2204391

# Electronic Meeting, February 21 – March 3, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR |  | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to TS 38.141-2: FRC for HST FR2 PUSCH performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2-Perf |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of FRC for HST-FR2 PUSCH requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | FRC for HST FR2 PUSCH requirements FRC tables will be further updated once agreement on MCS for requirements definition will be reached |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HST FR2 PUSCH performance will not be guaranteed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.104 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

Table A.X-X: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos0 and 1 transmission layer (TBA, R=TBA)

| Reference channel | G-FR2-AX-X | G-FR2-AX-X |
| --- | --- | --- |
| Subcarrier spacing [kHz] | 120 | 120 |
| Allocated resource blocks | 32 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 |
| Modulation | TBA | TBA |
| Code rate (Note 2) | TBA | TBA |
| Payload size (bits) | TBA | TBA |
| Transport block CRC (bits) | TBA | TBA |
| Code block CRC size (bits) | TBA | TBA |
| Number of code blocks - C | TBA | TBA |
| Code block size including CRC (bits) (Note 2) | TBA | TBA |
| Total number of bits per slot without PT-RS | 13824 | 57024 |
| Total number of bits per slot with PT-RS (Note 3) | 13248 | 54648 |
| Total symbols per slot without PT-RS | 3456 | 14256 |
| Total symbols per slot with PT-RS (Note 3) | 3312 | 13662 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0 with l0= 0 as per Table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1. |  |  |

Table A.X-X: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layer (TBA, R=TBA)

| Reference channel | G-FR2-AX-X | G-FR2-AX-X |
| --- | --- | --- |
| Subcarrier spacing [kHz] | 120 | 120 |
| Allocated resource blocks | 32 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 8 | 8 |
| Modulation | TBA | TBA |
| Code rate (Note 2) | TBA | TBA |
| Payload size (bits) | TBA | TBA |
| Transport block CRC (bits) | TBA | TBA |
| Code block CRC size (bits) | TBA | TBA |
| Number of code blocks - C | TBA | TBA |
| Code block size including CRC (bits) (Note 2) | TBA | TBA |
| Total number of bits per slot without PT-RS | 12288 | 50688 |
| Total number of bits per slot with PT-RS (Note 3) | 11776 | 48576 |
| Total symbols per slot without PT-RS | 3072 | 12672 |
| Total symbols per slot with PT-RS (Note 3) | 2944 | 12144 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0 with l0= 0 and l =8 as per Table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1. |  |  |

Table A.X-X: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos2 and 1 transmission layer (TBA, R=TBA)

| Reference channel | G-FR2-AX-X | G-FR2-AX-X |
| --- | --- | --- |
| Subcarrier spacing [kHz] | 120 | 120 |
| Allocated resource blocks | 32 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 7 | 7 |
| Modulation | TBA | TBA |
| Code rate (Note 2) | TBA | TBA |
| Payload size (bits) | TBA | TBA |
| Transport block CRC (bits) | TBA | TBA |
| Code block CRC size (bits) | TBA | TBA |
| Number of code blocks - C | TBA | TBA |
| Code block size including CRC (bits) (Note 2) | TBA | TBA |
| Total number of bits per slot without PT-RS | 10752 | 44352 |
| Total number of bits per slot with PT-RS (Note 3) | 10304 | 42504 |
| Total symbols per slot without PT-RS | 2688 | 11088 |
| Total symbols per slot with PT-RS (Note 3) | 2576 | 10626 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos2 with l0= 0 and l =4,8 as per Table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1. |  |  |

END OF CHANGE