3GPP TSG-RAN WG4 Meeting #101bis-e	R4-2204466

# Electronic Meeting, 21 Feb – 3 Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR – RSTD measurement requirements in RRC_INACTIVE state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce a new section to specify RSTD measurement requirements when the UE is in RRC_INACTIVE state |  |  |  |  |  |  |  |  |
| Summary of change: |  | Measurement period and applicability requirements for RSTD measurements performed in RRC_INACTIVE state are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE requirements for RSTD measurements performed in RRC_INACTIVE state would be unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### < Start of change 1>

### 5.5.2	RSTD measurements

#### 5.5.2.1	Introduction

The requirements in clause 5.5.2 shall apply provided the UE has received NR-DL-TDOA-RequestLocationInformation message from the LMF via LPP [34] requesting the UE to measure and report DL RSTD measurements defined in TS 38.215 [4].

#### 5.5.2.2	Requirements Applicability

The requirements in clause 5.5.2 apply for periodic and triggered RSTD measurements, provided:

-	PRS-RSTD related side conditions given in clause 10.1.23 for FR1 and FR2 are fulfilled, for a corresponding Band.

#### 5.5.2.3	Measurement Capability

UE PRS RSTD measurement capability [in RRC_INACTIVE state] is as indicated by the UE in NR-DL-TDOA-ProvideCapabilities, according to TS 37.355[34].

#### 5.5.2.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE is ready to transmit the measurement report over the air interface. If the UE supports reporting of NR positioning measurements via SDT, the UE may be able to report the measurements while it remains in RRC_INACTIVE state; otherwise, the UE will transition to RRC_CONNECTED state prior to transmitting the measurement report.

For RSTD measurements performed by the UE in RRC_INACTIVE state, The measurement reporting delay excludes all of the following:

- additional delay caused other LPP signalling on the DCCH,

- delay uncertainty introduced when inserting the measurement report in the TTI of the uplink DCCH, equal to  2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration,

- any delay caused by unavailability of UL resources to transmit the measurement report,

- any transmission delay needed by SDT,

- the time needed to transition to RRC_CONNECTED state to report the measurements.

The reported RSTD measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses 10.1.23.3.

The RSTD measurements performed and reported according to this section shall meet the RSTD measurement accuracy requirements in clause [ 10.1.23 ], for each measured DL PRS resource.

#### 5.5.2.5	Measurements Period Requirements

After receiving both NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from the LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 5.5.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i

Where ,

i is the index of positioning frequency layer,

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i

TRSTD,i is the measurement period for PRS RSTD measurement in positioning frequency layer i as specified below:

TRSTD,i=[ KPRS*NRxBeam,i*NPRS,islotN'*Nocc,i*Nsample-1*Teffect,i+Tlast,i ],

where:

NRxBeam,i is the UE Rx beam sweeping factor. In FR1, NRxBeam,i = 1; and in FR2,  NRxBeam,i = 8.

[ KPRS is a scaling factor for PRS-based NR positioning measurements in RRC_INACTIVE, where KPRS=1 or KPRS=Kcarrier+1 (or Nlayers+1, if only higher-priority NR and E-UTRA layers are measured by the UE), depending on UE capability. Kcarrier and Nlayers are defined in clause 4.2.2.4 and 4.2.2.7. ]

NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot.

[ Nocc,i is the number of measurement/processing occasions of length Locc,i needed to cover all the PRS resources counted in Lavailable_PRS,i, taking into account that the UE can measure at most PRS duration of Ni ms per occasion. The UE is required to measure PRS only during one occasion per Tavailable PRS,i. Locc,i may be requested by the LMF, subject to a maximum value of 20 ms. Otherwise, Locc,i=min20 ms, Ti. ]

[ Lavailable_PRS,i is the time duration of available PRS in positioning frequency layer i to be measured during Tavailable PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered. ]

Nsample is the number of PRS RSTD samples and Nsample= [1, 2, or 4 as requested by the LMF].

Tlast,i is the measurement duration for the last PRS RSTD sample in positioning frequency layer i, including the sampling time and processing time, Tlast,i = Ti + Tavailable_PRS,i ,

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i defined as:

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i

Where,

Ti	corresponds to [ durationOfPRS-ProcessingSymbolsInEveryTms  ] in TS 37.355 [34],

Tavailable_PRS,i=[LCMTPRS,i,DRX_cycle, the least common multiple between TPRS,i and DRX_cycle],

DRX_cycle is the DRX cycle length,

TPRS,i is the periodicity of DL PRS resource with muting on positioning frequency layer i.

If more than one PRS periodicities are configured in positioning frequency layer i, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets in the positioning frequency layer is used to derive TPRS,i, where,

TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource,

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where

TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

{N,T} is the UE capability combination per band for RRC_INACTIVE state where N is a duration of DL PRS symbols in ms corresponding to [durationOfPRS-ProcessingSysmbols] in TS 37.355 [34],  T (ms) corresponds to [durationOfPRS-ProcessingSymbolsInEveryTms] in TS 37.355 [34], [ and T-N (>0) is the time required to process duration N of DL PRS symbols already buffered in memory], for a given maximum bandwidth supported by UE corresponding to [supportedBandwidthPRS] in TS 37.355 [34],

N’ is UE capability for number of DL PRS resources that it can process in a slot [in RRC_INACTIVE state] as indicated by [maxNumOfDL-PRS-ResProcessedPerSlot] specified in TS 37.355 [34].

The time TRSTD,Total starts from [the first DRX cycle containing] a DL PRS resource(s) in the assistance data after both the NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

When PRS-RSRP is configured for DL-TDOA, RSTD and PRS-RSRP are performed over the same measurement period.

[ When PRS-RSRPP is configured for DL-TDOA, RSTD and PRS-RSRPP are performed over the same measurement period. ]

Reception of DL PRS has lower priority than the following DL signals/channels: SSB, SIB1, CORESET0, MSG2/MSGB, paging monitoring, and DL SDT. A collision between DL PRS and other higher-priority DL signals/channels occurs when

- [Any other higher-priority DL signal/channel occurs within X1 ms before the expected start time of (the first repetition of) a PRS resource, where X1 is the sum of receiver retuning time (RRT) and the expected RSTD-uncertainty of the PRS resource, and RRT equals 0.5 ms for serving cell in FR1, and  0.25 ms for serving cell in FR2.

  - Editor’s note: FFS if and under what conditions the retuning time RRT can be excluded from X1

- Any other higher-priority DL signal/channel occurs within X2 ms after the expected start time of (the first repetition of) a PRS resource, where X2 equals the sum of X1 plus T, and T (ms) corresponds to [durationOfPRS-ProcessingSymbolsInEveryTms] in TS 37.355 [34].  ]

The measurement requirements do not apply to any PRS resource that always collides with other higher-priority DL signals/channels.

Longer PRS measurement period is expected when there are collisions between PRS resources and other higher-priority DL signals/channels.

[If KPRS,i changes for any PFL during the measurement period, the measurement period could be longer.]

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 5.5.2 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If cell re-selection occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements after a new cell is selected. The RSTD measurement period can be longer. The UE shall meet the RSTD measurement accuracy requirements in clause [ 10.1.23 ].

### < End of change 1>