3GPP TSG-RAN WG4 Meeting # 102-e 	R4-2204501

# Electronic Meeting, February 21 – March 3, 2022

Agenda Item:	10.1.2.3

Source: 		Qualcomm Incorporated

Title: 			On preliminary MU assessment for FR2 MIMO OTA

Document for:	Approval

# Introduction

In the revised WID [1], it was agreed that RAN4 will develop the preliminary MU for FR1 and FR2 MIMO OTA. In this paper, we provide the proposals for the preliminary MU for FR2 MIMO OTA.

# 2. Discussion

In RAN#94e meeting, the preliminary MU for FR2 MIMO OTA shall be studied in RAN4. And the final MU and TT will be decided by RAN5.

-	Develop the preliminary Measurement Uncertainty (MU) assessment for FR1 and FR2 MIMO OTA

- Example expanded uncertainty for test method shall be defined.

The final MU and test tolerances (TT) will be decided by RAN5.

In [2], we proposed the preliminary MU budget per TR38.827 and TR 38.810. It was suggest referring to the latest MU budget specified in TR 38.903. Therefore, per the agreed MU elements in TR 38.827 and MU specified for FR2 Demodulation/RRM in TR3 8.903, the preliminary example values for frequency range of 26.5GHz≤f≤29.5GHz are proposed in Table 1.

Table 1: Measurement uncertainty budget for FR2 3D-MPAC

| UID | Description of uncertainty contribution | Example value (26.5GHz≤f≤29.5GHz) | Example value (37GHz ≤f≤40GHz) | Distribution of the probability | Details in |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Mismatch for measurement process | [1.30] | TBD | U-Shaped | B.2.2.1 |
| 2 | Measure distance uncertainty | [0.15] | TBD | [Rectangular] | B.2.2.2 |
| 3 | Quality of quiet zone | [1.20] | TBD | [Actual] | B.2.2.3 |
| 4 | Base Station simulator | TBD | TBD | [Normal] | B.2.2.4 |
| 5 | Channel Emulator -absolute value -stability -linearity | [2.90] | TBD | Normal | B.2.2.5 |
| 6 | Amplifier uncertainties | [2.10] | TBD | [Normal] | B.2.2.6 |
| 7 | Random uncertainty | [0.50] | TBD | [Normal] | B.2.2.7 |
| 8 | Throughput measurement: output level step resolution | [0.23] | TBD | Rectangular | B.2.2.8 |
| 9 | DUT sensitivity drift | TBD | TBD | Rectangular | B.2.2.9 |
| 10 | Signal flatness | TBD | TBD | Normal | B.2.2.10 |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 11 | Mismatch for calibration process - loopback cable path - system input path - reference antenna | [0.00] | TBD | U-Shaped | B.2.2.11 |
| 12 | Reference antenna positioning misalignment | [0.00] | TBD | [Rectangular] | B.2.2.12 |
| 13 | Quality of quiet zone | [0.4] | TBD | [Actual] | B.2.2.3 |
| 14 | Total uncertainty of the Network Analyzer | [0.73] | TBD | [Normal] | B.2.2.13 |
| 15 | Uncertainty of an absolute gain of the calibration antenna | [0.6dB] | TBD | [Normal] | B.2.2.14 |
| 16 | Offset of the Phase Center of the Reference Antenna | [0.47] | TBD | [Rectangular] | B.2.2.16 |

Proposal 1: To agree the above MU budget for FR2 MIMO OTA 3D-MPAC as the start point.

# 3. 	Conclusion

In this paper, we study the preliminary MU for FR2 MIMO OTA and have the following proposal:

Table 1: Measurement uncertainty budget for FR2 3D-MPAC

| UID | Description of uncertainty contribution | Example value (26.5GHz≤f≤29.5GHz) | Example value (37GHz ≤f≤40GHz) | Distribution of the probability | Details in |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Mismatch for measurement process | [1.30] | TBD | U-Shaped | B.2.2.1 |
| 2 | Measure distance uncertainty | [0.15] | TBD | [Rectangular] | B.2.2.2 |
| 3 | Quality of quiet zone | [1.20] | TBD | [Actual] | B.2.2.3 |
| 4 | Base Station simulator | TBD | TBD | [Normal] | B.2.2.4 |
| 5 | Channel Emulator -absolute value -stability -linearity | [2.90] | TBD | Normal | B.2.2.5 |
| 6 | Amplifier uncertainties | [2.10] | TBD | [Normal] | B.2.2.6 |
| 7 | Random uncertainty | [0.50] | TBD | [Normal] | B.2.2.7 |
| 8 | Throughput measurement: output level step resolution | [0.23] | TBD | Rectangular | B.2.2.8 |
| 9 | DUT sensitivity drift | TBD | TBD | Rectangular | B.2.2.9 |
| 10 | Signal flatness | TBD | TBD | Normal | B.2.2.10 |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 11 | Mismatch for calibration process - loopback cable path - system input path - reference antenna | [0.00] | TBD | U-Shaped | B.2.2.11 |
| 12 | Reference antenna positioning misalignment | [0.00] | TBD | [Rectangular] | B.2.2.12 |
| 13 | Quality of quiet zone | [0.4] | TBD | [Actual] | B.2.2.3 |
| 14 | Total uncertainty of the Network Analyzer | [0.73] | TBD | [Normal] | B.2.2.13 |
| 15 | Uncertainty of an absolute gain of the calibration antenna | [0.6dB] | TBD | [Normal] | B.2.2.14 |
| 16 | Offset of the Phase Center of the Reference Antenna | [0.47] | TBD | [Rectangular] | B.2.2.16 |

Proposal 1: To agree the above MU budget for FR2 MIMO OTA 3D-MPAC as the start point.

# 4.	References

- RP-213101, Revised WID: Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs, CAICT, OPPO

- R4-2200779, Discussion on preliminary MU assessment for FR2 MIMO OTA, Qualcomm Incorporated

- TR 38.903,  NR; Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases

- TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)