3GPP TSG RAN WG4 Meeting #102-e	R4-2204554

Online, 21th Feb. – 3rd Mar. 2022

Agenda item:	9.15.2

Source:	SoftBank Corp., LG Electronics

Title:	TP for TR 37.717-21-11: EN-DC_8-11_n1

Document for:	Approval

1	Introduction

EN-DC of 2B LTE and 1B NR of DC_8-11_n1 was approved in RAN#93[1]. This TP is to capture the basic aspects for the EN-DC.

2	Combination proposed

Combinations proposed in [1] are as below.

Table 2-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-11A_n1A | DC_8A_n1A DC_11A_n1A |

3	Co-existence

When Uplink EN-DC configuration is DC_8A_n1A, IMD3 and IMD4 of (B11 - n1) will fall into Rx band of Band 8. Thus, it is proposed to evaluate.

4	Delta Tib/Rib

The delta Tib/Rib of subset EN-DC/CA are as follows. EN-DC values are picked.

Table 4-1: ΔTIB of constituent 2 band combo.

| ΔTIB | B8+B11 | B8+n1 | B11+n1 | Proposed Value |
| --- | --- | --- | --- | --- |
| B8 | 0.3 | 0.3 |  | 0.3 |
| B11 | 0.4 |  | 0.3 | 0.4 |
| n1 |  | 0.3 | 0.3 | 0.3 |

Table 4-2: ΔRIB of constituent 2 band combo.

| ΔRIB | B8+B11 | B8+n1 | B11+n1 | Proposed Value |
| --- | --- | --- | --- | --- |
| B8 | 0 | 0 |  | 0 |
| B11 | 0 |  | 0 | 0 |
| n1 |  | 0 | 0 | 0 |

Then the proposal for this 2LTE-1NR EN-DC is:

Table 4-3: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n1 | 8 | 0.3 |
|  | 11 | 0.4 |
|  | n1 | 0.3 |

Table 4-4: ΔRIB,c

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n1 | 8 | 0 |
|  | 11 | 0 |
|  | n1 | 0 |

5	Reference sensitivity exceptions

As mentioned in section 3, REFSENS exceptions due to IMD3/4 are expected. The following values are proposed.

Table 5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A-11A_n1A | 11 | 1435 | 5 | 25 | 1483 | N/A | FDD | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | 8 | 885 | 5 | 25 | 930 | 16.6 | FDD | IMD35 |
| NOTE 5: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

6	Conclusion

This paper proposes study results of EN-DC 8-11_n1.

TP is shown in Annex.

7	Reference

[1]		RP-212174 WID revision: Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL); rapporteur: Huawei

Annex	Proposed text for TR37.717-21-11:

[Unchanged Parts Skipped]

5.x	DC_8-11_n1

5.x.1	Configurations for DC_8-11_n1

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-11A_n1A | DC_8A_n1A DC_11A_n1A |

5.x.2	Co-existence studies

5.x.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.x.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n1 | 8 | 0.3 |
|  | 11 | 0.4 |
|  | n1 | 0.3 |

Table 5.x.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n1 | 8 | 0 |
|  | 11 | 0 |
|  | n1 | 0 |

5.x.4	Reference sensitivity exceptions

As mentioned above, IMD of B11 and n1 to Band8 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.x.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A-11A_n1A | 11 | 1435 | 5 | 25 | 1483 | N/A | FDD | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | 8 | 885 | 5 | 25 | 930 | 16.6 | FDD | IMD35 |
| NOTE 5: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

[Unchanged Parts Skipped]