3GPP TSG-RAN WG4 Meeting #102-e	R4-2204573

# Electronic Meeting, February 21 – March 3, 2022

Title:	Discussion on ENDC combination and measurement parameters

Source:	Samsung

Agenda item:	10.2.2.2

Document for:	Discussion

1.	Introduction

In RAN4#101-bis-e meeting, the LTE anchor band for NR n28 and n41 has been agreed, down selection of LTE anchor band for NR n78 and n79 is expected this meeting, as indicated in the approved WF [1]

Issue 1-2-3: Example EN-DC combination

Agreement:

- Capture the table below in the TP R4-2203073

Table 4.3-3: Measurement parameters for example inter-band EN-DC band combinations (two bands)

| EN-DC configuration | E-UTRA configurations | NR configurations |
| --- | --- | --- |
| DC_3A_n28A | Note1 | Note2 |
| DC_2A_n41A | Note1 | Note2 |
| Note 1: As per TR 37.902 [10], Section 6.4 (Measurement frequencies). Note 2: As per Table 4.3-1 and Table 4.3-2 in this technical report. |  |  |

- RAN4 further discussion the down selection of the following EN-DC combinations next meeting.

| EN-DC configuration | E-UTRA configurations | NR configurations |
| --- | --- | --- |
| DC_13A_n78A or DC_1A_n78A | Note1 | Note2 |
| DC_5A_n79A or DC_1A_n79A | Note1 | Note2 |
| Note 1: As per TR 37.902 [10], Section 6.4 (Measurement frequencies). Note 2: As per Table 4.3-1 and Table 4.3-2 in this technical report. |  |  |

- RAN4 should further discuss other EN-DC combinations in future meetings.

- It is encouraged to define a decision tree / flow chart to ultimately determine the EN-DC combination of a UE based on the agreed EN-DC combination selection principle.

In this contribution, our view on this issue is provided.

2. 	Discussion

The agreed ENDC combination selection principle is as following [1]:

Agreement:

- Principle of EN-DC combinations selection for TRP TRS OTA testing:

1)	Focus on the performance of the NR carrier and do not consider multiple permutations between different LTE bands and NR band under test, i.e., for each NR band, only select one EN-DC band combination.

2)	For UE supporting multiple EN-DC band combinations for the same NR band, consider only those EN-DC configurations which have no MSD impact on either LTE or NR, i.e., the selected EN-DC combination should be no MSD issue identified in TS 38.101-3 Section 7.3B.2.3 (Inter-band EN-DC within FR1).

The ENDC configurations under discussion all comply with above principle. So the down-selection could be based on which one is typical.

LTE bands B1/B2/B3 are usually treated as middle frequency bands and share the same transmitting and receiving path in UE design. Compared with 700/800/900MHz bands, B1/B2/B3 are more widely supported at the same time for the same UE. Now that B2 and B3 have already been selected as LTE anchor bands for n28 and n41, due to MSD issue neither B2 nor B3 is suitable for n78 and n79. It is better to consider B1 so that the UE widely supported middle frequency bands are used as the anchor.

Proposal 1:	Adopt E-UTRA band B1 as the anchor for NR band n78 and n79 in ENDC configuration.

The test parameters for each bands include 3 ranges (Low, Mid, High) for E-UTRA and NR respectively. There would be 3x3=9 permutations for one ENDC combination. We have agreed the principle of EN-DC combinations selection: “Focus on the performance of the NR carrier and do not consider multiple permutations between different LTE bands and NR band under test, i.e., for each NR band, only select one EN-DC band combination”.  Similarly, it is not necessary to consider all permutations of Low Mid High ranges between E-UTRA and NR respectively. So we propose to only consider the following mapping for one ENDC combination:

- E-UTRA Low range + NR Low range

- E-UTRA Mid range + NR Mid range

- E-UTRA High range + NR High range

Proposal 2:	For one ENDC combination, the measurement parameters for NR Low Mid High ranges correspond to E-UTRA Low Mid High ranges respectively.

Based on proposal 1 and 2, the measurement parameter table for ENDC could be as following:

| EN-DC configuration | E-UTRA configurations | NR configurations |
| --- | --- | --- |
| DC_3A_n28A | Note1 | Note2 |
| DC_2A_n41A | Note1 | Note2 |
| DC_1A_n78A | Note1 | Note2 |
| DC_1A_n79A | Note1 | Note2 |
| Note 1: As per TR 37.902 [10], Section 6.4 (Measurement frequencies). Note 2: As per Table 4.3-1 and Table 4.3-2 in this technical report. The measurement parameters for NR Low Mid High ranges correspond to E-UTRA Low Mid High ranges respectively. |  |  |

3. 	Conclusion

Proposal 1:	Adopt E-UTRA band B1 as the anchor for NR band n78 and n79 in ENDC configuration.

Proposal 2:	For one ENDC combination, the measurement parameters for NR Low Mid High ranges correspond to E-UTRA Low Mid High ranges respectively.

Based on proposal 1 and 2, the measurement parameter table for ENDC could be as following:

| EN-DC configuration | E-UTRA configurations | NR configurations |
| --- | --- | --- |
| DC_3A_n28A | Note1 | Note2 |
| DC_2A_n41A | Note1 | Note2 |
| DC_1A_n78A | Note1 | Note2 |
| DC_1A_n79A | Note1 | Note2 |
| Note 1: As per TR 37.902 [10], Section 6.4 (Measurement frequencies). Note 2: As per Table 4.3-1 and Table 4.3-2 in this technical report. The measurement parameters for NR Low Mid High ranges correspond to E-UTRA Low Mid High ranges respectively. |  |  |

4. 	References

- R4-2203076, “WF on FR1 TRP TRS,” vivo, 3GPP RAN4#101-bis-e, Jan 2022