3GPP TSG-RAN WG4 Meeting #102                 	R4-2204588

Electronic Meeting, Feb 21- Mar 3, 2022

Source:	Nokia, Nokia Shanghai Bell

Title: 	Discussion on PMI/RI requirements

Agenda item:	10.20.4.3.2

Document for:	Discussion

Introduction

The last eMeeting’s discussion and WF of the UE demodulation and CSI requirements for RedCap topic are captured in the summary [1] and WF [2].

In this contribution we will express our views on the open issues related to PMI/RI requirements and open new discussions, if necessary.

Discussion

PMI reporting requirements

In WF ‎[2], whether to define PMI reporting tests or not was discussed

- RAN4 discuss further whether to define PMI reporting tests for RedCap UE. - Option 1: Define for both 1Rx UE and 2Rx UE - Option 2: Not define for both 1Rx UE and 2Rx UE - Option 3 Define only for 2Rx UE - If PMI reporting test is defined, discuss further applied test cases

Not supporting MIMO operation is not one of the reduced capabilities of the RedCap. Hence, we think that option 3 is a good compromise to reduce the specification and test load. PMI is still part of Redcap UE mandatory capabilities hence they should be tested.

MIMO operation is not one of the reduced capabilities of the RedCap.

Use Option 3 Define only for 2Rx UE with the supported RedCap BW.

RI reporting requirements for 2Rx UE

In WF ‎[2], two options were discussed

- RAN4 discuss further whether to define RI reporting tests for RedCap 2Rx UE. - Option 1: Define RI tests. It requires to rerun the simulation with CQI table 1 for FR1 FDD and TDD. - Option 2: Not define RI tests.

The same reasons as for PMI apply (RI is not optional, required for normal operation, and has heavy performance impact), hence we propose:

Use Option 1 Define only for 2Rx UE with the supported RedCap BW.

Conclusion

In this contribution we have provided our views on RedCap UE demodulation and CSI requirements. In particular, we have discussed the PMI/RI requirements.

We have made the following observations and proposals:

Concerning PMI reporting requirements

MIMO operation is not one of the reduced capabilities of the RedCap.

- Use Option 3 Define only for 2Rx UE with the supported RedCap BW.

Concerning PMI reporting requirements

Use Option 1 Define only for 2Rx UE with the supported RedCap BW.

- References

- R4-2203119 Email discussion summary for [101-bis-e][321] NR_RedCap_Demod_NWM, Ericsson

- R4-2203032 WF on RedCap demodulation and CQI reporting requirements, Ericsson

- RP-211574, “Revised WID on support of reduced capability NR devices”.

- 3GPP TS 38.101-4 “User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 16)”.

- 3GPP TR 38.875, “Study on support of reduced capability NR devices”, V17.0.0 (2021-03).