3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204633

# Electronic Meeting, February 21- March 3, 2022

Agenda item:	10.22.2.2

Source: 	vivo

Title: 	Further discussion on efficient activation/de-activation mechanism for one SCG

Document for:	Discussion

- Introduction

In the RAN4 #101bis-e meeting, the general aspects of activation/deactivation mechanism for SCG requirements were discussed. The agreements and open issues are captured in the WF [1].

In this paper we provide our views on the rest issues.

- Discussion

The following was captured in the WF [1] on activation/deactivation mechanism for SCG.

- RAN4 are to discuss the potential RRM impacts due to efficient activation/de-activation mechanism for one SCG - Measurement requirements for deactivated SCG (Issue 2-1) - DRX configuration for deactivated SCG measurement requirements (Issue 2-1-1) - SCG Activation/deactivation delay (Issue 2-2) - UE processing time (Tprocessing) in PSCell activation delay (Issue 2-2-2) - time/frequency tracking time (T∆) in PSCell activation delay (Issue 2-2-3) - Requirements for RACH based PSCell activation delay (Issue 2-2-5) - Conditions for RACH-less based SCG activation (Issue 2-2-6) - Interruption requirements (Issue 2-3) - Baseline for interruption due to PSCell activation/deactivation (Issue 2-3-1) - Whether to define interruption due to PSCell activation/deactivation in asynchronous deployment (Issue 2-3-2) - Interruption due to L3 measurement on deactivated SCG (Issue 2-3-3) - RLM/BFD/BFR/Beam management on deactivated PSCell (Issue 2-4) - Whether RLM/BFD requirements on deactivated PSCell can be relaxed (Issue 2-4-1) - Interruption requirement due to RLM and BFD on deactivated PSCell (Issue 2-4-2) - Others (Issue 2-5) - whether UE shall meet the existing Te and Tq when PSCell is deactivated (Issue 2-5-1) - if the answer of issue 2-5-1 is yes, SSB monitoring rate (Issue 2-5-2) - Known and Unknown TCI state for deactivated PSCell (Issue 2-5-3) - UE behaviour upon RLF and BFD on deactivated PSCell (Issue 2-5-4)

2.1 Measurement requirements for deactivated SCG

- DRX configuration for deactivated SCG measurement requirements (Issue 2-1-1) - Agreements - Introduce a parameter measCyclePSCell for relaxation of RRM measurement on deactivated PSCell - FFS: min value and range of measCyclePSCell

In #101-bis-e meeting, RAN4 has sent a LS [2] to ask RAN2 to introduce the parameter measCyclePSCell for deactivated PSCell measurement. Specific to min value and range of measCyclePSCell, they are within the scope of the discussion in RAN4. Compared with the PSCell in the activated state, the measurement frequency of deactivated PSCell would be lower, so we consider that the min value could be a larger value e.g. 640ms. As for the range of measCyclePSCell, it can be reused the range value of deactivated SCell. LS to RAN2 on RAN4 decision on min value and the range value of measCyclePSCell.

Proposal 1: The min value of measCyclePSCell could be a larger value e.g. 640ms. And the range of measCyclePSCell could be reused the range value of measCycleSCell. LS to RAN2 on RAN4 decision on min value and range of measCyclePSCell.

2.2 SCG Activation/deactivation delay

- UE processing time (Tprocessing) in PSCell activation delay (Issue 2-2-2) - Option 1: - Tprocessing = 20ms NR PSCell is in FR1 in EN-DC. - Tprocessing = 40 ms if NR PSCell is in FR2 in EN-DC or NR-DC - Revised Option 2: Tprocessing can be 20ms if any PSCell parameter is modified, and FFS otherwise - Option 3: Tprocssing is 10ms when a deactivated PSCell is being activated. - Option 3a: Tprocssing is X ms when a deactivated PSCell is being activated, X is FFS. - Option 4: - When a deactivated PSCell is being activated, - if PSCell parameter is not modified, Tprocessing can be 1ms - if any PSCell parameter is modified, Tprocessing can be 10ms. - When PSCell change event trigger and PSCell activation command happened at the same time (if we assume the case is supported that there is a PSCell change when SCG state is configured as deactivated), - Tprocessing = 20ms NR PSCell is in FR1 in EN-DC. - Tprocessing = 40 ms if NR PSCell is in FR2 in EN-DC or NR-DC - Option 5: - PSCell activation delay shall not include Tprocessing when PSCell is activated from deactivated state, i.e., Tprocessing = 0ms. However, PSCell activation delay should allow UE RF warm up delay. - RAN4 to define separate parameter to account for the RF warm up delay during PSCell activation – TRF_warmup. - RAN4 need to discuss and agree on a suitable RF warm delay for PSCell activation.

In our understanding, whether the parameters for PSCell are modified or not when the activation command is indicated in RRC has a certain impact on the value of Tprocessing. According to this influence, we think it will be divided into following two scenarios:

If PSCell parameters is not modified, it will need some time to finish some RRC configurations and 0ms cannot reach for this scenario. Besides, the RF warm up is also a part of processing time, we think 1ms could be enough for Tprocessing.

And if any PSCell parameters are modified, it will take some extra time to reconfigure these parameters. It is similar to PSCell change to a certain extent. Therefore, we prefer to use the principle in Option 1. When NR PSCell is in FR1 in EN-DC, Tprocessing could be 20ms. And when NR PSCell is in FR2 in EN-DC or NR-DC, Tprocessing could be 40 ms.

Proposal 2:

- If PSCell parameters is not modified, Tprocessing could be 1ms.

- If any PSCell parameter is modified,

  - Tprocessing = 20ms NR PSCell is in FR1 in EN-DC.

  - Tprocessing = 40 ms if NR PSCell is in FR2 in EN-DC or NR-DC

- time/frequency tracking time (T∆) in PSCell activation delay (Issue 2-2-3) - Option 1: time/frequency tracking time (T∆) in PSCell activation delay is needed - Option 2: - T∆ =1*Trs ms for a known or unknown PSCell. T∆ = 0 when UE is configured with RLM/BFD measurements in PSCell. - Option 2a - When the previous RS for RLM/BFD was received within [1280ms], UE can obtain fine timing information from the RLM/BFD measurements and T∆ could be 0, - otherwise, UE needs to do the fine T/F tracking again and T∆ could be 1*Trs ms.

We see that T∆ may be different for the different scenarios:

When the UE not configured to perform RLM/BFD on deactivated PSCell, UE needs to do the fine T/F tracking and we think T∆ could be 1*Trs ms.

When the UE configured to perform RLM/BFD on deactivated PSCell, whether we can acquire the valid timing information from RLM/BFD depends on how long the previous RS for RLM/BFD is received. If the RS has been received for a long time, UE also needs to perform fine T/F tracking again. Therefore, we should ensure that the previous RS for RLM/BFD was received within a short time [X ms] e.g.1280ms so that we can acquire the valid fining tracking information from RLM/BFD and T∆ could be 0. Otherwise, UE still needs to perform fine T/F tracking and T∆ could be 1*Trs ms.

Proposal 3: T∆ could be different in the following scenarios:

o	When the previous RS for RLM/BFD was received within [1280ms], UE can obtain fine timing information from the RLM/BFD measurements and T∆ could be 0,

o	otherwise, UE needs to do the fine T/F tracking again and T∆ could be 1*Trs ms.

- Requirements for RACH-less based PSCell activation delay (Issue 2-2-5) - Option 1: RACH-less PSCell activation delay can be defined as Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆+ TIU + 2 ms where TIU is the interruption uncertainty in acquiring the first PUSCH transmission occasion/[SR on PUCCH] when UE is configured with RACH-less SCG. - Option 2: RACH-less PSCell activation delay can be defined as Tconfig_PSCell =TRRC_delay + Tprocessing + T∆+TIU + 2 ms where TIU is the interruption uncertainty in acquiring the first [PUSCH transmission]/[SR on PUCCH] occasion when UE is configured with RACH-less SCG.

Firstly, we think the requirement for the RACH-less handover in LTE could not be the reference to RACH-less based PSCell activation delay. Compared with the RACH-less handover in LTE, when PSCell being activated is same cell as previously being deactivated, the target PSCell is always known. It is unnecessary to rely on whether the TCI state is known to remove Tsearch. Besides, we think whether the UE is configured BFD or not and the value of Tsearch should be decoupled. The former is related to T∆ discussed in Issue 2-2-3 rather than here.

Proposal 4: When PSCell being activated is same cell as previously being deactivated, the target PSCell is always known and the Tsearch could be removed. RACH-less PSCell activation delay can be defined as

Tconfig_PSCell =TRRC_delay + Tprocessing + T∆+TIU + 2 ms

- Conditions for RACH-less based SCG activation (Issue 2-2-6) - RACH-less based SCG activation delay requirements shall meet the following conditions: - TAT is running and TA is regarded as valid, - FFS: TCI state is known for both UE and network, - FFS: BFD should be configured and no BF is detected.

According to the LS that sent from RAN2 in last meeting [3], they discussed the UE behaviour when UE is configured to perform RLM/BFD while SCG is deactivated. When BFD is configured and no BF is detected, the TCI state can always be ensured to be valid. From this point, we think that the third bullet should be the condition for RACH-less based SCG activation when BFD is configured. Regarding the second bullet, how to implement TCI is known for both UE and network when the SCG is in deactivated state is a hard work. This bullet cannot be a necessary condition for RACH-less based SCG activation. Therefore, we only suggest to list the third bluet as the condition when the UE is configured to perform BFD.

Proposal 5: For RACH-less based SCG activation, the condition that BFD should be configured and no BF is detected needs to be satisfied (when the UE is configured to perform BFD).

2.3 Interruption requirements

- Baseline for interruption due to PSCell activation/deactivation (Issue 2-3-1) If PSCell is activated from a deactivated status - Option 1: Existing requirements for interruption due to SCell activation/deactivation can be used as a baseline. - Option 2: Existing requirements for interruption due to PSCell addition/release can be used as baseline, i.e., 1ms interruption length.

When PSCell is activated from a deactivated status, the interruption procedure is more similar to interruption due to SCell activation/deactivation and UE do not need to perform additional parameter loading which is needed for PSCell addition/release. From this point, it is reasonable to reuse existing interruption requirement due to activation/deactivation for SCell activation.

Proposal 6: When PSCell is activated from a deactivated status, existing requirements for interruption due to SCell activation/deactivation can be used as a baseline

- Whether to define interruption due to PSCell activation/deactivation in asynchronous deployment (Issue 2-3-2) - Option 1: - For SCG activation/deactivation in ENDC, - -When SCG is activated/deactivated, there are no active serving cells in the SCG. The interruption on LTE MCG can refer to clause 7.32.2.4 (Interruptions at SCell addition/release) in TS 36.133. - For SCG activation/deactivation in NR-DC, the interruption requirements can refer to existing interruptions at PSCell addition/release specified in clause 8.2.4.2.1 in TS38.133. - Option 2: Depends on issue 2-3-1.

According to the description about asynchronous deployment in TS38.133 8.2.4.2.2: “the requirements for Async apply for asynchronous NR-DC if the active serving cell is not in the same CG as any of the SCell or PSCells being added or released”, the additional interruption shall be allowed due to PSCell activation/deactivation in asynchronous deployment for PSCell activation/deactivation. For SCG activation/deactivation in EN-DC, there is only the interruption on LTE MCG needs to be considered and it can refer to clause 7.32.2.4 (Interruptions at SCell addition/release) in TS 36.133. For SCG activation/deactivation in NR-DC, the interruption requirement can refer to existing interruption at PSCell addition/release specified in clause 8.2.4.2.1 in TS38.133.

Proposal 7:

- For SCG activation/deactivation in EN-DC,

when SCG is activated/deactivated, there are no active serving cells in the SCG. The interruption on LTE MCG can refer to clause 7.32.2.4 (Interruptions at SCell addition/release) in TS 36.133.

- For SCG activation/deactivation in NR-DC,

the interruption requirements can refer to existing interruptions at PSCell addition/release specified in clause 8.2.4.2.1 in TS38.133.

- Interruption due to L3 measurement on deactivated PSCell (Issue 2-3-3) - Option 1: - If RLM/BFD is not configured, the current interruption requirement during measurements on deactivated inter-band SCC applies. - If RLM/BFD is configured, the current interruption requirement during Scell dormancy applies([X]%). - Option 2: The current interruption requirement during Scell dormancy applies ([1]%). - Option 3: The current interruption requirement on deactivated inter-band SCC can be reused for L3 measurement for deactivated SCG;

According to current requirement for interruption during measurements on deactivated SCC, the measCycleSCell which is configured to 640ms or longer is a pre-condition for interruption on PCell or activated SCell(s) due to measurements. As for deactivated PSCell, RAN4 has send a LS to RAN2 to introduce the parameter measCyclePSCell. From this point, the current interruption requirement on deactivated inter-band SCC can be reused for interruption due to L3 measurement on deactivated PSCell directly. In our perspective, interruption requirement due to RLM/BFD on deactivated PSCell is another separated issue discussed in Issue 2-4-2 and the interruption requirement due to SCell dormancy can be used as baseline.

Proposal 8: Whether RLM/BFD is configured or not, the current interruption requirement on deactivated inter-band SCC can be reused directly for L3 measurement for deactivated SCG.

2.4 RLM/BFD/BFR/Beam management on deactivated PSCell

- Whether RLM/BFD delay requirements on deactivated PSCell can be relaxed (Issue 2-4-1) - Option 1: Yes. RLM/BFD measurements and requirements would follow the measurement cycle of the deactivated PSCell. - Option 3: no RLM/BFD relaxation, and revisit the details if needed after RAN2 conclusions.

There are no RLM/BFD delay requirements on deactivated PSCell in the existing spec. In our understanding, the original intention of discussion on this issue is only to define RLM/BFD delay requirements for deactivated PSCell, instead of specifically discussing whether RLM/BFD delay requirements on deactivated PSCell could be relaxed.

Further, since the measCyclePSCell configuration is the main factor that affects the measurement period, compared with the PSCell in the activated state, whether there is a need to have further relaxation on deactivated PSCell when defining RLM/BFD delay requirements is affected by the value of measCyclePSCell. Related to the min value of measCyclePSCell discussed in Issue 2-1-1, if the min value of measCyclePSCell can be to 640ms or a larger value, there is no need to have further relaxation here. However, if the min value of measCyclePSCell less than 640ms, we propose to do 1.5 times or 2 times relaxation on RLM/BFD measurement of the deactivated PSCell.

Proposal 9: Related to the min value of measCyclePSCell discussed in Issue 2-1-1:

- If the min value of measCyclePSCell can be to 640ms or a larger value, there is no need to have further relaxation here.

- If the min value of measCyclePSCell less than 640ms, we propose to do 1.5 times or 2 times relaxation on RLM/BFD measurement of the deactivated PSCell.

- Interruption requirement due to RLM and BFD on deactivated PSCell (Issue 2-4-2) - Option 1: The same principle as the interruption due to SCell dormancy is applied ([X]%). (further check if 0.5% is agreeable) - Option 2: - - RLM and BFD based on SSBs: Take the TS38.133 8.2.3.2.5.2 Interruptions during measurements on deactivated E-UTRAN SCC as baseline which both take in the x% probability as well as the interruption window.

The interruption requirement due to SCell dormancy can be used as baseline. Regarding the percentage of CQI measurements during SCell dormancy, it is reasonable to use existing 0.5%.

Proposal 10: Interruption requirement due to RLM and BFD on deactivated PSCell could reused the same principle as the interruption due to SCell dormancy is applied ([0.5]%).

2.5 Others

- whether UE shall meet the existing Te and Tq when PSCell is deactivated (Issue 2-5-1) - Option 1b: Timing requirements including Te and Tq don’t need to be specified when PSCell is deactivated. - Option 3: The existing UE initial transmit timing error (Te) and gradual timing adjustment requirements are met at least until the TAT is running. - If the answer of issue 2-5-1 is yes, conditions for meeting Te requirements (Issue 2-5-2) - Option 1: RAN4 further discusses whether to meet Te requirements the SSB should be available at the UE once every 160 ms or it should be available at the UE with the same rate with which the UE performs RRM requirements on PSCell e.g. once every measCyclePscell if measCyclePscell is introduced.

When PSCell is deactivated, there is no UL transmission from UE. The timing requirement including Te and Tq don’t need to meet all the time before PSCell activation. As for the accuracy of UE timing, it could be guaranteed by fine time tracking discussed in Issue 2-2-3.

Regarding Issue 2-5-2, it’s up to the conclusion of Issue 2-5-1.

Proposal 11: Timing requirement including Te and Tq don’t need to meet when PSCell is activated. As for the accuracy of UE timing, it could be guaranteed by fine time tracking discussed in Issue 2-2-3.

- Known and Unknown TCI state for deactivated PSCell (Issue 2-5-3) - Option 1: Existing known conditions for a TCI state as defined in section 8.10.2 can apply also for a deactivated PSCell. - Option 2: the known conditions for a TCI state at RACH-less SCG activation can be “BFD for the TCI state is configured and no BF is detected”

In existing known conditions for a TCI state as defined in 8.10.2, L1-RSRP report needs to be sent. However, L1-RSRP is not supported on deactivated PSCell according to the discussion in last meeting. This leads the conditions cannot be met and it is not applicable for known and unknown TCI state. From this perspective, we think the existing TCI state switch requirements cannot be reused and it is necessary to additionally specify the conditions for known and unknown state when PSCell is deactivated. Combined with the statement in Issue 2-2-6, we supposed to define the condition for known and unknown TCI state only in the case which the UE is configured to perform BFD for deactivated PSCell. In this case, when the BFD is configured and no BF is detected, the TCI state could be known. Otherwise, the TCI state is regard as unknown.

Proposal 12: Defining the condition for known and unknown TCI state only when the UE is configured to perform BFD for deactivated PSCell. When the BFD is configured and no BF is detected, the TCI state could be known. Otherwise, the TCI state is regard as unknown.

- UE behaviour upon RLF and BFD on deactivated PSCell (Issue 2-5-4) - Option 1: - The UE shall not perform RRC connection re-establishment upon detecting RLF on the deactivated PSCell. - The UE shall stop performing RLM on the deactivated PSCell upon detecting RLF on the deactivated PSCell. - The UE shall not beam failure recovery or candidate beam detection upon beam failure detection on the deactivated PSCell. - The UE shall stop performing beam failure detection if the UE has detected beam failure on the deactivated PSCell. - Inform RAN2 about the observations and proposals related to impact of RLF and BFD on the deactivated PSCell. - Option2: leave the discussion to RAN2.

We think The UE behaviours upon RLF and BFD on deactivated PSCell belongs to the scope of RAN2. These issues should be discussed in RAN2 rather than RAN4.

Proposal 13: The UE behaviours upon RLF and BFD on deactivated PSCell belongs to the scope of RAN2.

- Conclusions

Proposal 1: The min value of measCyclePSCell could be a larger value e.g. 640ms. And the range of measCyclePSCell could be reused the range value of measCycleSCell. LS to RAN2 on RAN4 decision on min value and range of measCyclePSCell.

Proposal 2:

- If PSCell parameters is not modified, Tprocessing could be 1ms.

- If any PSCell parameter is modified,

  - Tprocessing = 20ms NR PSCell is in FR1 in EN-DC.

  - Tprocessing = 40 ms if NR PSCell is in FR2 in EN-DC or NR-DC

Proposal 3: T∆ could be different in the following scenarios:

o	When the previous RS for RLM/BFD was received within [1280ms], UE can obtain fine timing information from the RLM/BFD measurements and T∆ could be 0,

o	otherwise, UE needs to do the fine T/F tracking again and T∆ could be 1*Trs ms.

Proposal 4: When PSCell being activated is same cell as previously being deactivated, the target PSCell is always known and the Tsearch could be removed. RACH-less PSCell activation delay can be defined as

Tconfig_PSCell =TRRC_delay + Tprocessing + T∆+TIU + 2 ms

Proposal 5: For RACH-less based SCG activation, the condition that BFD should be configured and no BF is detected needs to be satisfied (when the UE is configured to perform BFD).

Proposal 6: When PSCell is activated from a deactivated status, existing requirements for interruption due to SCell activation/deactivation can be used as a baseline

Proposal 7:

- For SCG activation/deactivation in EN-DC,

when SCG is activated/deactivated, there are no active serving cells in the SCG. The interruption on LTE MCG can refer to clause 7.32.2.4 (Interruptions at SCell addition/release) in TS 36.133.

- For SCG activation/deactivation in NR-DC,

the interruption requirements can refer to existing interruptions at PSCell addition/release specified in clause 8.2.4.2.1 in TS38.133.

Proposal 8: Whether RLM/BFD is configured or not, the current interruption requirement on deactivated inter-band SCC can be reused directly for L3 measurement for deactivated SCG.

Proposal 9: Related to the min value of measCyclePSCell discussed in Issue 2-1-1:

- If the min value of measCyclePSCell can be to 640ms or a larger value, there is no need to have further relaxation here.

- If the min value of measCyclePSCell less than 640ms, we propose to do 1.5 times or 2 times relaxation on RLM/BFD measurement of the deactivated PSCell.

Proposal 10: Interruption requirement due to RLM and BFD on deactivated PSCell could reused the same principle as the interruption due to SCell dormancy is applied ([0.5]%).

Proposal 11: Timing requirement including Te and Tq don’t need to meet when PSCell is activated. As for the accuracy of UE timing, it could be guaranteed by fine time tracking discussed in Issue 2-2-3.

Proposal 12: Defining the condition for known and unknown TCI state only when the UE is configured to perform BFD for deactivated PSCell. When the BFD is configured and no BF is detected, the TCI state could be known. Otherwise, the TCI state is regard as unknown.

Proposal 13: The UE behaviours upon RLF and BFD on deactivated PSCell belongs to the scope of RAN2.

References

[1] R4-2202688 WF on R17 further Multi-RAT Dual-Connectivity enhancements, Huawei

[2] R4-2202777 LS on efficient activation/de-activation mechanism for one SCG, Ericsson

[3] R2-2201711 LS on efficient activation/de-activation mechanism for one SCG, Huawei

Appendix

3GPP TSG-RAN4 WG4 Meeting #102-e	R4-22xxxxx

# Electronic meeting, February 21 – March 3, 2022

Title:	Reply LS on efficient activation/de-activation mechanism for one SCG

Response to:	R2-2201711 “LS on efficient activation/de-activation mechanism for one SCG”

Release:	Rel-17

Work Item:	LTE_NR_DC_enh2

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name:

E-mail Address:

Attachments:

1. Overall Description:

In RAN4#101bis-e, RAN4 sent a LS (R4-2202777) to RAN2 to introduce a similar parameter as measCycleSCell for deactivated PSCell. The min value and range value was still discussing.

RAN4 has discussed and agreed the min value and range of the similar parameter as measCycleSCell for deactivated PSCell:

The min value is 640ms. And the range value is the same as the range value of measCycleSCell.

2. Actions:

To RAN WG2 group.

ACTION: 	RAN4 kindly asks RAN2 to take the above information into account in the further specification work.

3. References:

4. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #103-e		May 16 – May 27, 2022				Electronic Meeting