3GPP TSG-RAN WG4 Meeting #102-e                	R4-2204708

Electronic Meeting, Feb. 21- Mar.3, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Open issues on the CSI-RS based measurement requirements

Agenda item:	5.1.4

Document for:	Discussion

Introduction

In RAN4#101-e meeting, the UE behavior when the timing offset exceeds the threshold was discussed leaving the following options unsolved [1].

- Candidate agreements

  - UE behavior when the timing offset exceeds the threshold

    - Option 1:

      - UE is not required to report CSI-RS based L3 measurements. If UE reports CSI-RS based L3 measurement, then the UE shall meet CSI-RS based L3 measurement reporting requirements in TS 38.133 section 9.10.2.4 and 9.10.3.4 based on the accuracy requirements for the case when the timing offset is below the threshold

      - Do not define test cases to verify UE behavior for the case when the timing offset is above the threshold

      - Support: CATT, Apple, CMCC, Nokia

      - Object: MTK, Huawei, QC, vivo, Xiaomi

    - Option 2:

      - UE is not required to report CSI-RS based L3 measurements. If UE reports CSI-RS based L3 measurement, then the UE may not meet CSI-RS based L3 measurement reporting requirements in TS 38.133 section 9.10.2.4 and 9.10.3.4 based on the accuracy requirements for the case when the timing offset is below the threshold

      - Do not define test cases to verify UE behavior for the case when the timing offset is above the threshold

      - Support: MTK, Huawei, QC, vivo, Xiaomi

      - Object: CATT, Apple, CMCC, Nokia

    - Option 3:

      - UE is not required to report CSI-RS based L3 measurements.

      - Support: MTK, vivo, Huawei, Xiaomi, OPPO, QC

      - Object: Nokia, Apple, CATT, CMCC

    - Option 4:

      - When the timing offset exceeds the threshold X

        - X = CP + Y, where Y is the extra margin. Y is FFS

        - UE is not required to report CSI-RS based L3 measurements.

        - If UE reports CSI-RS based L3 measurement, then the UE shall meet CSI-RS based L3 measurement reporting requirements in TS 38.133 section 9.10.2.4 and 9.10.3.4 based on the accuracy requirements for the case when the timing offset is below the threshold

        - Do not define test cases to verify UE behavior for the case when the timing offset is above the threshold

We have been supporting Option 1 to ensure the network always receives a measurement report fulfilling the accuracy requirement. From the discussion, it is understood the UE may not be able to identify the timing difference to determine if the measurement reports shall be sent or not.  Considering the UE implementation, we can compromise to not requiring the CSI-RS based L3 measurements reporting fulfilling the performance requirements in section 10. But the UE behavior as discussed shall be clarified in the spec as apparently companies did not have common understanding on existing specification. To minimize the specification impact, this can be added as a note in TS 38.133 section 9.10.2.4 and 9.10.3.4.

Proposal 1: We support Option 1 but can compromise to Option 2.

Proposal 2: Clarify the UE behavior in the specification e.g. capturing the agreeable option as a note in TS 38.133 section 9.10.2.4 and 9.10.3.4.

Conclusion

This contribution discusses the UE behavior when timing offset exceeds the threshold. The proposal is summarized as below:

Proposal 1: We support Option 1 but can compromise to Option 2.

Proposal 2: Clarify the UE behavior in the specification e.g. capturing the agreeable option as a note in TS 38.133 section 9.10.2.4 and 9.10.3.4.

References

- RAN4#101-e RRM session report