3GPP TR 37.717-11-31 V0.7.0 (2022-03)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Networks;

Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL)

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Contents	3

Foreword	20

1	Scope		21

2	References		21

3	Definitions, symbols and abbreviations	21

3.1	Definitions		21

3.2	Symbols		22

3.3	Abbreviations		22

4	Background		22

4.1	TR Maintenance	22

5	DC band combinations of LTE x bands DL/1UL(x=1,2,3) + NR 3 bands DL/1UL: General Part	22

5.1	General		22

5.2 	General treatment of ∆TIB and ∆RIB values	22

6 DC band combinations of LTE 1 band DL/1UL + NR 3 bands DL/1UL: Specific Band Combination Part	22

6.1	DC_1_n3-n28-n77	22

6.1.1	Operating bands for DC	23

6.1.2	Inter-band DC Configurations	23

6.1.3	Co-existence studies	23

6.1.4	∆TIB and ∆RIB values	23

6.1.5	MSD		23

6.2	DC_8_n3-n28-n77	24

6.2.1	Operating bands for DC	24

6.2.2	Inter-band DC Configurations	24

6.2.3	Co-existence studies	24

6.2.4	∆TIB and ∆RIB values	24

6.2.5	MSD		25

6.3	DC_8A_n40A-n41A-n79A	25

6.3.1	Operating bands for DC	25

6.3.3	Co-existence studies	25

6.3.4	∆TIB and ∆RIB values	25

6.3.5	MSD		25

6.4	DC_3_n1-n78-n257, DC_3-3_n1-n78-n257	26

6.4.1	Operating bands for DC	26

6.4.2	Inter-band DC Configurations	26

6.4.3	Co-existence studies	26

6.4.4	∆TIB and ∆RIB values	26

6.4.5	MSD		26

6.5	DC_7_n1-n78-n257, DC_7-7_n1-n78-n257	26

6.5.1	Operating bands for DC	27

6.5.2	Inter-band DC Configurations	27

6.5.3	Co-existence studies	27

6.5.4	∆TIB and ∆RIB values	27

6.5.5	MSD		27

6.6	DC_11_n3-n28-n77	28

6.6.1	Operating bands for DC	28

6.6.2	Inter-band DC Configurations	28

6.6.3	Co-existence studies	28

6.6.4	∆TIB and ∆RIB values	28

6.6.5	MSD		29

6.7	DC_42_n3-n28-n77	29

6.7.1	Operating bands for DC	29

6.7.2	Inter-band DC Configurations	29

6.7.3	Co-existence studies	29

6.7.4	∆TIB and ∆RIB values	29

6.7.5	MSD		30

6.8	 DC_1A_n28A-n77A-n79A	30

6.8.1	Operating bands for DC	30

6.8.2	Inter-band DC Configurations	30

6.8.3	Co-existence studies	30

6.8.4	∆TIB an	d ∆RIB values	30

6.8.5	MSD		31

6.9	 DC_1A_n28A-n78A-n79A	31

6.9.1	Operating bands for DC	31

6.9.2	Inter-band DC Configurations	31

6.9.3	Co-existence studies	31

6.9.4	∆TIB and ∆RIB values	31

6.9.5	MSD		32

6.10	 DC_3A_n1A-n77A-n79A	32

6.10.1	Operating bands for DC	32

6.10.2	Inter-band DC Configurations	32

6.10.3	Co-existence studies	32

6.10.4	∆TIB and ∆RIB values	32

6.10.5	MSD		33

6.11	 DC_3A_n1A-n78A-n79A	33

6.11.1	Operating bands for DC	33

6.11.2	Inter-band DC Configurations	33

6.11.3	Co-existence studies	33

6.11.4	∆TIB and ∆RIB values	33

6.11.5	MSD		34

6.12	 DC_3A_n28A-n77A-n79A	34

6.12.1	Operating bands for DC	34

6.12.2	Inter-band DC Configurations	34

6.12.3	Co-existence studies	34

6.12.4	∆TIB and ∆RIB values	34

6.12.5	MSD		35

6.13	 DC_3A_n28A-n78A-n79A	35

6.13.1	Operating bands for DC	35

6.13.2	Inter-band DC Configurations	35

6.13.3	Co-existence studies	35

6.13.4	∆TIB and ∆RIB values	35

6.13.5	MSD		36

6.14	 DC_19A_n1A-n77A-n79A	36

6.14.1	Operating bands for DC	36

6.14.2	Inter-band DC Configurations	36

6.14.3	Co-existence studies	36

6.14.4	∆TIB and ∆R	IB values	36

6.14.5	MSD		37

6.15	 DC_19A_n1A-n78A-n79A	37

6.15.1	Operating bands for DC	37

6.15.2	Inter-band DC Configurations	37

6.15.3	Co-existence studies	37

6.15.4	∆TIB and ∆RIB values	37

6.15.5	MSD		38

6.16	 DC_21A_n1A-n77A-n79A	38

6.16.1	Operating bands for DC	38

6.16.2	Inter-band DC Configurations	38

6.16.3	Co-existence studies	38

6.16.4	∆TIB and ∆RIB values	38

6.16.5	MSD		39

6.17	 DC_21A_n1A-n78A-n79A	39

6.17.1	Operating bands for DC	39

6.17.2	Inter-band DC Configurations	39

6.17.3	Co-existence studies	39

6.17.4	∆TIB and ∆RIB values	39

6.17.5	MSD		40

6.18	 DC_21A_n28A-n77A-n79A	40

6.18.1	Operating bands for DC	40

6.18.2	Inter-band DC Configurations	40

6.18.3	Co-existence studies	40

6.18.4	∆TIB and ∆RIB values	40

6.18.5	MSD		41

6.19	 DC_21A_n28A-n78A-n79A	41

6.19.1	Operating bands for DC	41

6.19.2	Inter-band DC Configurations	41

6.19.3	Co-existence studies	41

6.19.4	∆TIB and ∆RIB values	41

6.19.5	MSD		42

6.20	 DC_42A_n1A-n77A-n79A	42

6.20.1	Operating bands for DC	42

6.20.2	Inter-band DC Configurations	42

6.20.3	Co-existence studies	42

6.20.4	∆TIB and ∆RIB values	42

6.20.5	MSD		43

6.21	 DC_42A_n1A-n78A-n79A	43

6.21.1	Operating bands for DC	43

6.21.2	Inter-band DC Configurations	43

6.21.3	Co-existence studies	43

6.21.4	∆TIB and ∆RIB values	43

6.21.5	MSD		44

6.22	  DC_8A_n39-n40-n41	44

6.22.1	Operating bands for DC	44

6.22.3	Co-existence studies	44

6.22.4	∆TIB and ∆RIB values	44

6.22.5	MSD		45

6.23	  DC_8A_n39-n40-n79	45

6.23.1	Operating bands for DC	45

6.23.3	Co-existence studies	45

6.23.4	∆TIB and ∆RIB values	45

6.23.5	MSD		46

6.24	  DC_3_n41-n79-n258	46

6.24.1	Operating bands for DC	46

DC_3_n41-n79-n258	46

6.24.3	Co-existence studies	46

6.24.4	∆TIB and ∆RIB values	46

6.24.5	MSD		46

6.25	DC_1_n3-n28-n79	47

6.25.1	Operating bands for DC	47

6.25.2	Inter-band DC Configurations	47

6.25.3	Co-existence studies	47

6.25.4	∆TIB and ∆RIB values	47

6.25.5	MSD		47

6.26	DC_1_n3-n77-n79	48

6.26.1	Operating bands for DC	48

6.26.2	Inter-band DC Configurations	48

6.26.3	Co-existence studies	48

6.26.4	∆TIB and ∆RIB values	48

6.26.5	MSD		48

6.27	DC_8_n3-n77-n79	49

6.27.1	Operating bands for DC	49

6.27.2	Inter-band DC Configurations	49

6.27.3	Co-existence studies	49

6.27.4	∆TIB and ∆RIB values	49

6.27.5	MSD		49

6.28	DC_8_n3-n28-n79	50

6.28.1	Operating bands for DC	50

6.28.2	Inter-band DC Configurations	50

6.28.3	Co-existence studies	50

6.28.4	∆TIB and ∆RIB values	50

6.28.5	MSD		51

6.29	DC_8_n28-n77-n79	51

6.29.1	Operating bands for DC	51

6.29.2	Inter-band DC Configurations	51

6.29.3	Co-existence studies	51

6.29.4	∆TIB and ∆RIB values	51

6.29.5	MSD		52

6.30	DC_3_n1-n8-n78, DC_3-3_n1-n8-n78	52

6.30.1	Operating bands for DC	52

6.30.2	Inter-band DC Configurations	52

6.30.3	Co-existence studies	52

6.30.4	∆TIB and ∆RIB values	52

6.30.5	MSD		53

6.31	DC_7_n1-n8-n78, DC_7-7_n1-n8-n78	53

6.31.1	Operating bands for DC	53

6.31.2	Inter-band DC Configurations	53

6.31.3	Co-existence studies	53

6.31.4	∆TIB and ∆RIB values	53

6.31.5	MSD		54

6.32	DC_8_n1-n3-n77	54

6.32.1	Operating bands for DC	54

6.32.2	Inter-band DC Configurations	54

6.32.3	Co-existence studies	54

6.32.4	∆TIB and ∆RIB values	54

6.32.5	MSD		55

6.33	DC_11_n3-n77-n79	55

6.33.1	Operating bands for DC	55

6.33.2	Inter-band DC Configurations	55

6.33.3	Co-existence studies	55

6.33.4	∆TIB and ∆RIB values	56

6.33.5	MSD		56

7	DC band combinations of LTE 2 bands DL/1UL + NR 3 bands DL/1UL: Specific Band Combination Part	57

7.1	DC_3-7_n1-n78-n257, DC_3-3-7_n1-n78-n257, DC_3-7-7_n1-n78-n257, DC_3-3-7-7_n1-n78-n257	57

7.1.1	Operating bands for DC	57

7.1.2	Inter-band DC Configurations	57

7.1.3	Co-existence studies	58

7.1.4	∆TIB and ∆RIB values	58

7.1.5	MSD		58

7.2	DC_1-8_n3-n28-n77	58

7.2.1	Operating bands for DC	58

7.2.2	Inter-band DC Configurations	59

7.2.3	Co-existence studies	59

7.2.4	∆TIB and ∆RIB values	59

7.2.5	MSD		59

7.3	DC_1-11_n3-n28-n77	60

7.3.1	Operating bands for DC	60

7.3.2	Inter-band DC Configurations	60

7.3.3	Co-existence studies	60

7.3.4	∆TIB and ∆RIB values	60

7.3.5	MSD		61

7.4	DC_1-42_n3-n28-n77	61

7.4.1	Operating bands for DC	61

7.4.2	Inter-band DC Configurations	62

7.4.3	Co-existence studies	62

7.4.4	∆TIB and ∆RIB values	62

7.4.5	MSD		63

7.5	DC_8-11_n3-n28-n77	63

7.5.1	Operating bands for DC	63

7.5.2	Inter-band DC Configurations	63

7.5.3	Co-existence studies	63

7.5.4	∆TIB and ∆RIB values	63

7.5.5	MSD		64

7.6	DC_8-42_n3-n28-n77	64

7.6.1	Operating bands for DC	64

7.6.2	Inter-band DC Configurations	65

7.6.3	Co-existence studies	65

7.6.4	∆TIB and ∆RIB values	65

7.6.5	MSD		66

7.7	 DC_1A-3A_n28A-n77A-n79A	67

7.7.1	Operating bands for DC	67

7.7.2	Inter-band DC Configurations	67

7.7.3	Co-existence studies	67

7.7.4	∆TIB and ∆RIB values	67

7.7.5	MSD		68

7.8	 DC_1A-3A_n28A-n78A-n79A	68

7.8.1	Operating bands for DC	68

7.8.2	Inter-band DC Configurations	68

7.8.3	Co-existence studies	68

7.8.4	∆TIB and ∆RIB values	68

7.8.5	MSD		69

7.9	 DC_1A-21A_n28A-n77A-n79A	69

7.9.1	Operating bands for DC	69

7.9.2	Inter-band DC Configurations	69

7.9.3	Co-existence studies	69

7.9.4	∆TIB and ∆RIB values	69

7.9.5	MSD		70

7.10	 DC_1A-21A_n28A-n78A-n79A	70

7.10.1	Operating bands for DC	70

7.10.2	Inter-band DC Configurations	70

7.10.3	Co-existence studies	70

7.10.4	∆TIB and ∆RIB values	70

7.10.5	MSD		71

7.11	 DC_3A-21A_n1A-n77A-n79A	71

7.11.1	Operating bands for DC	71

7.11.2	Inter-band DC Configurations	71

7.11.3	Co-existence studies	71

7.11.4	∆TIB and ∆RIB values	71

7.11.5	MSD		72

7.12	 DC_3A-21A_n1A-n78A-n79A	72

7.12.1	Operating bands for DC	72

7.12.2	Inter-band DC Configurations	72

7.12.3	Co-existence studies	72

7.12.4	∆TIB and ∆RIB values	72

7.12.5	MSD		73

7.13	 DC_3A-21A_n28A-n77A-n79A	73

7.13.1	Operating bands for DC	73

7.13.2	Inter-band DC Configurations	73

7.13.3	Co-existence studies	73

7.13.4	∆TIB and ∆RIB values	73

7.13.5	MSD		74

7.14	 DC_3A-21A_n28A-n78A-n79A	74

7.14.1	Operating bands for DC	74

7.14.2	Inter-band DC Configurations	74

7.14.3	Co-existence studies	74

7.14.4	∆TIB and ∆RIB values	74

7.14.5	MSD		75

7.15	 DC_19A-42A_n1A-n77A-n79A	75

7.15.1	Operating bands for DC	75

7.15.2	Inter-band DC Configurations	75

7.15.3	Co-existence studies	75

7.15.4	∆TIB and ∆RIB values	75

7.15.5	MSD		76

7.16	 DC_19A-42A_n1A-n78A-n79A	76

7.16.1	Operating bands for DC	76

7.16.2	Inter-band DC Configurations	76

7.16.3	Co-existence studies	76

7.16.4	∆TIB and ∆RIB values	76

7.16.5	MSD		77

7.17	DC_1-8_n3-n77-n79	77

7.17.1	Operating bands for DC	77

7.17.2	Inter-band DC Configurations	77

7.17.3	Co-existence studies	77

7.17.4	∆TIB and ∆RIB values	77

7.17.5	MSD		78

7.18	DC_3-7_n1-n8-n78, DC_3-3-7_n1-n8-n78, DC_3-7-7_n1-n8-n78, DC_3-3-7-7_n1-n8-n78	78

7.18.1	Operating bands for DC	78

7.18.2	Inter-band DC Configurations	78

7.18.3	Co-existence studies	79

7.18.4	∆TIB and ∆RIB values	79

7.18.5	MSD		79

7.18	DC_1-8_n3-n28-n79	79

7.18.1	Operating bands for DC	79

7.18.2	Inter-band DC Configurations	80

7.18.3	Co-existence studies	80

7.18.4	∆TIB and ∆RIB values	80

7.18.5	MSD		80

7.19	DC_1-8_n28-n77-n79	81

7.19.1	Operating bands for DC	81

7.19.2	Inter-band DC Configurations	81

7.19.3	Co-existence studies	81

7.19.4	∆TIB and ∆RIB values	81

7.19.5	MSD		82

8	DC band combinations of LTE 3 bands DL/1UL + NR 3 bands DL/1UL: Specific Band Combination Part	82

8.1	DC_1-8-11_n3-n28-n77	82

8.1.1	Operating bands for DC	82

8.1.2	Inter-band DC Configurations	82

8.1.3	Co-existence studies	82

8.1.4	∆TIB and ∆RIB values	83

8.1.5	MSD		83

8.2	DC_1-8-42_n3-n28-n77	83

8.2.1	Operating bands for DC	83

8.2.2	Inter-band DC Configurations	84

8.2.3	Co-existence studies	84

8.2.4	∆TIB and ∆RIB values	84

8.2.5	MSD		85

Annex A: Change history	86

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for DC of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL)  under Rel-17 time frame, including EN-DC and NE-DC. The purpose is to gather the relevant background information and studies in order to address DC of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL)  in Rel-17 as .

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]		3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: “Guideline on WI/SI for new Operating Bands”

[3]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[4]	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

[5]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for CA or DC operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for CA or DC operation, for serving cell c.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

4	Background

The present document is a technical report for  DC of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL)  under Rel-17 time frame, including EN-DC and NE-DC.

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

5	DC band combinations of LTE x bands DL/1UL(x=1,2,3) + NR 3 bands DL/1UL: General Part

5.1	General

In order to support DC of LTE CA for up to 3 different bands DL with 1 band UL + NR CA for 3 different bands DL with 1 band UL in rel-17,  the corresponding fallback modes shall be shall be completed and specified in advance.

Unless otherwise stated, the requirements for EN-DC and NE-DC for the same band combination are the same.

5.2 	General treatment of ∆TIB and ∆RIB values

For DC of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL) band combination,  RAN4 should consider to reuse agreed additional insertion losses for all EN-DC band combinations when new RF components are not introduced to support this basket WI. If the new RF components are introduced, then more detail description will be captured in some specific EN-DC band combinations.

- DC band combinations of LTE 1 band DL/1UL + NR 3 bands DL/1UL: Specific Band Combination Part

6.1	DC_1_n3-n28-n77

6.1.1	Operating bands for DC

Table 6.1.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_1_n3-n28-n77 | 1 | CA_n3-n28-n77 |

6.1.2	Inter-band DC Configurations

Table 6.1.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A_n3A-n28A-n77A | DC_1A_n3A DC_1A_n28A DC_1A_n77A |
| DC_1A_n3A-n28A-n77(2A) | DC_1A_n3A DC_1A_n28A DC_1A_n77A |

6.1.3	Co-existence studies

Co-existence study for DC_1_n3-n28-n77 was covered by the studies for the fallback modes of DC_1_n3-n28, DC_1_n3-n77 and DC_1_n28-n77.

No additional MSD requirement needs to be defined for this dual connectivity configuration.

6.1.4	∆TIB and ∆RIB values

For DC_1_n3-n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.1.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n3-n28-n77 | 1 | 0.6 |
|  | n3 | 0.8 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 6.1.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n3-n28-n77 | 1 | 0.2 |
|  | n3 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

6.1.5	MSD

As mentioned in 6.1.3, there is no need to specify additional MSD requirement for this UL DC configuration..

6.2	DC_8_n3-n28-n77

6.2.1	Operating bands for DC

Table 6.2.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_8_n3-n28-n77 | 8 | CA_n3-n28-n77 |

6.2.2	Inter-band DC Configurations

Table 6.2.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8A_n3A-n28A-n77A | DC_8A_n3A DC_8A_n28A DC_8A_n77A |
| DC_8A_n3A-n28A-n77(2A) | DC_8A_n3A DC_8A_n28A DC_8A_n77A |

6.2.3	Co-existence studies

Co-existence study for DC_8_n3-n28-n77 was covered by the studies for the fallback modes of DC_8_n3-n28, DC_8_n3-n77 and DC_8_n28-n77.

No additional MSD requirement needs to be defined for this dual connectivity configuration.

6.2.4	∆TIB and ∆RIB values

For DC_8_n3-n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n3-n28-n77 | 8 | 0.6 |
|  | n3 | 0.6 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |

Table 6.2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n3-n28-n77 | 8 | 0.2 |
|  | n3 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

6.2.5	MSD

As mentioned in 6.2.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.3	DC_8A_n40A-n41A-n79A

6.3.1	Operating bands for DC

Table 6.3.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_8_n40-n41-n79 | 8 | CA_n40-n41-n79 |

### 6.3.2	Inter-band DC Configurations

Table 6.3.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8A_n40A-n41A-n79A | DC_8A_n40A DC_8A_n41A DC_8A_n79A |

6.3.3	Co-existence studies

For DC_8A_n40A-n41A-n79A co-existence studies, the lower order DC_8A_n40A-n41A, DC_8A_n40A-n79A and DC_8A_n41A-n79A can be applied.

6.3.4	∆TIB and ∆RIB values

For DC_8A_n40A-n41A-n79A , the TIB,c and RIB,c values are given in the tables below.

Table 6.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n40-n41-n79 | 8 | 0.3 |
|  | n40 | 0.3 |
|  | n41 | 0.3 |
|  | n79 | 0 |

Table 6.3.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n40-n41-n79 | 8 | 0 |
|  | n40 | 0 |
|  | n41 | 0 |
|  | n79 | 0 |

6.3.5	MSD

No additional MSD requirement is needed.

6.4	DC_3_n1-n78-n257, DC_3-3_n1-n78-n257

6.4.1	Operating bands for DC

Table 6.4.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_3_n1-n78-n257 DC_3-3_n1-n78-n257 | 3 CA_3-3 | CA_n1-n78-n257 |

6.4.2	Inter-band DC Configurations

Table 6.4.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A_n1A-n78A-n257A DC_3A_n1A-n78A-n257D DC_3A_n1A-n78A-n257E DC_3A_n1A-n78A-n257F DC_3A_n1A-n78A-n257G DC_3A_n1A-n78A-n257H DC_3A_n1A-n78A-n257I DC_3A_n1A-n78A-n257J DC_3A_n1A-n78A-n257K DC_3A_n1A-n78A-n257L DC_3A_n1A-n78A-n257M | DC_3A_n1A DC_3A_n78A DC_3A_n257A |
| DC_3A-3A_n1A-n78A-n257A DC_3A-3A_n1A-n78A-n257D DC_3A-3A_n1A-n78A-n257E DC_3A-3A_n1A-n78A-n257F DC_3A-3A_n1A-n78A-n257G DC_3A-3A_n1A-n78A-n257H DC_3A-3A_n1A-n78A-n257I DC_3A-3A_n1A-n78A-n257J DC_3A-3A_n1A-n78A-n257K DC_3A-3A_n1A-n78A-n257L DC_3A-3A_n1A-n78A-n257M | DC_3A_n1A DC_3A_n78A DC_3A_n257A |

6.4.3	Co-existence studies

The co-existence studies can be covered by the studies of the constituent combinations mentioned in 37.716-21-21.

6.4.4	∆TIB and ∆RIB values

For DC_3_n1-n78-n257, DC_3-3_n1-n78-n257, TIB,c and RIB,c values are set to zero for n257, and the values for constituent E-UTRA and FR1 NR bands are same as those for the corresponding inter band EN-DC configurations which are defined in the TS 38.101-3 already.

6.4.5	MSD

No additional MSD requirement is needed.

<Next session>

6.5	DC_7_n1-n78-n257, DC_7-7_n1-n78-n257

6.5.1	Operating bands for DC

Table 6.5.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_7_n1-n78-n257 DC_7-7_n1-n78-n257 | 7 CA_7-7 | CA_n1-n78-n257 |

6.5.2	Inter-band DC Configurations

Table 6.5.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A_n1A-n78A-n257A DC_7A_n1A-n78A-n257D DC_7A_n1A-n78A-n257E DC_7A_n1A-n78A-n257F DC_7A_n1A-n78A-n257G DC_7A_n1A-n78A-n257H DC_7A_n1A-n78A-n257I DC_7A_n1A-n78A-n257J DC_7A_n1A-n78A-n257K DC_7A_n1A-n78A-n257L DC_7A_n1A-n78A-n257M | DC_7A_n1A DC_7A_n78A DC_7A_n257A |
| DC_7A-7A_n1A-n78A-n257A DC_7A-7A_n1A-n78A-n257D DC_7A-7A_n1A-n78A-n257E DC_7A-7A_n1A-n78A-n257F DC_7A-7A_n1A-n78A-n257G DC_7A-7A_n1A-n78A-n257H DC_7A-7A_n1A-n78A-n257I DC_7A-7A_n1A-n78A-n257J DC_7A-7A_n1A-n78A-n257K DC_7A-7A_n1A-n78A-n257L DC_7A-7A_n1A-n78A-n257M | DC_7A_n1A DC_7A_n78A DC_7A_n257A |

6.5.3	Co-existence studies

The co-existence studies can be covered by the studies for the constituent combinations mentioned in 37.716-21-21.

6.5.4	∆TIB and ∆RIB values

For DC_7_n1-n78-n257, DC_7-7_n1-n78-n257, TIB,c and RIB,c values are set to zero for n257, and the values for constituent E-UTRA and FR1 NR bands are same as those for the corresponding inter band EN-DC configurations which are defined in the TS 38.101-3 already.

6.5.5	MSD

No additional MSD requirement is needed.

6.6	DC_11_n3-n28-n77

6.6.1	Operating bands for DC

Table 6.6.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_11_n3-n28-n77 | 11 | CA_n3-n28-n77 |

6.6.2	Inter-band DC Configurations

Table 6.6.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_11A_n3A-n28A-n77A | DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_11A_n3A-n28A-n77(2A) | DC_11A_n3A DC_11A_n28A DC_11A_n77A |

6.6.3	Co-existence studies

Co-existence study for DC_11_n3-n28-n77 was covered by the studies for the fallback modes of DC_11_n3-n28, DC_11_n3-n77 and DC_11_n28-n77.

No additional MSD requirement needs to be defined for this dual connectivity configuration.

6.6.4	∆TIB and ∆RIB values

For DC_11_n3-n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11_n3-n28-n77 | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 6.6.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11_n3-n28-n77 | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

6.6.5	MSD

As mentioned in 6.6.3, there is no need to specify additional MSD requirement for this UL DC configuration..

6.7	DC_42_n3-n28-n77

6.7.1	Operating bands for DC

Table 6.7.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_42_n3-n28-n77 | 42 | CA_n3-n28-n77 |

6.7.2	Inter-band DC Configurations

Table 6.7.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_42A_n3A-n28A-n77A | DC_42A_n3A DC_42A_n28A |
| DC_42A_n3A-n28A-n77(2A) | DC_42A_n3A DC_42A_n28A |
| DC_42C_n3A-n28A-n77A | DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_42C_n3A-n28A-n77(2A) | DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |

6.7.3	Co-existence studies

Co-existence study for DC_42_n3-n28-n77 was covered by the studies for the fallback modes of DC_42_n3-n28, DC_42_n3-n77 and DC_42_n28-n77.

No additional MSD requirement needs to be defined for this dual connectivity configuration.

6.7.4	∆TIB and ∆RIB values

For DC_42_n3-n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.7.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42_n3-n28-n77 | 42 | 0.8 |
|  | n3 | 0.6 |
|  | n28 | 0.8 |
|  | n77 | 0.8 |

Table 6.7.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42_n3-n28-n77 | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n28 | 0.5 |
|  | n77 | 0.5 |

6.7.5	MSD

As mentioned in 6.7.3, there is no need to specify additional MSD requirement for this UL DC configuration..

6.8		DC_1A_n28A-n77A-n79A

6.8.1	Operating bands for DC

Table 6.8.1-1: EN-DC band combination (four bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_1_n28-n77-n79 | 1 | CA_n28-n77-n79 |

6.8.2	Inter-band DC Configurations

Table 6.8.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A |

6.8.3	Co-existence studies

Co-existence study for DC_1A_n28A-n77A-n79A was covered by the studies for the fallback modes of DC_1A_n28A-n77A, DC_1A_n28A-n79A and DC_1A_n77A-n79A.

6.8.4	∆TIB and ∆RIB values

For DC_1A_n28A-n77A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 6.8.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_1A_n28A-n77A-n79A | 1 | 0.6 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 6.8.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n28-n77-n79 | 1 | 0.3 |
|  | n28 | 0.3 |
|  | n77 | 0.5 |

6.8.5	MSD

No additional MSD requirement is needed.

6.9		DC_1A_n28A-n78A-n79A

6.9.1	Operating bands for DC

Table 6.9.1-1: EN-DC band combination (four bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_1_n28-n78-n79 | 1 | CA_n28-n78-n79 |

6.9.2	Inter-band DC Configurations

Table 6.9.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A_n28A-n78A-n79A | DC_1A_n28A DC_1A_n78A DC_1A_n79A |

6.9.3	Co-existence studies

Co-existence study for DC_1A_n28A-n78A-n79A was covered by the studies for the fallback modes of DC_1A_n28A-n78A, DC_1A_n28A-n79A and DC_1A_n78A-n79A.

6.9.4	∆TIB and ∆RIB values

For DC_1A_n28A-n78A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 6.9.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_1A_n28A-n78A-n79A | 1 | 0.3 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |

Table 6.9.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A_n28A-n78A-n79A | 1 | 0.3 |
|  | n28 | 0.3 |
|  | n78 | 0.5 |
|  | n79 | 0 |

6.9.5	MSD

No additional MSD requirement is needed.

6.10		DC_3A_n1A-n77A-n79A

6.10.1	Operating bands for DC

Table 6.10.1-1: EN-DC band combination (four bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_3_n1-n77-n79 | 3 | CA_n1-n77-n79 |

6.10.2	Inter-band DC Configurations

Table 6.10.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A_n1A-n77A-n79A | DC_3A_n1A DC_3A_n77A DC_3A_n79A |

6.10.3	Co-existence studies

Co-existence study for DC_3A_n1A-n77A-n79A was covered by the studies for the fallback modes of DC_3A_n1A-n77A, DC_3A_n1A-n79A and DC_3A_n77A-n79A.

6.10.4	∆TIB and ∆RIB values

For DC_3A_n1A-n77A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 6.10.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_3A_n1A-n77A-n79A | 3 | 0.6 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 6.10.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A_n1A-n77A-n79A | 3 | 0.2 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0 |

6.10.5	MSD

No additional MSD requirement is needed.

6.11		DC_3A_n1A-n78A-n79A

6.11.1	Operating bands for DC

Table 6.11.1-1: EN-DC band combination (four bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_3_n1-n78-n79 | 3 | CA_n1-n78-n79 |

6.11.2	Inter-band DC Configurations

Table 6.11.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A_n1A-n78A-n79A | DC_3A_n1A DC_3A_n78A DC_3A_n79A |

6.11.3	Co-existence studies

Co-existence study for DC_3A_n1A-n78A-n79A was covered by the studies for the fallback modes of DC_3A_n1A-n78A, DC_3A_n1A-n79A and DC_3A_n78A-n79A.

6.11.4	∆TIB and ∆RIB values

For DC_3A_n1A-n78A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 6.11.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_3A_n1A-n78A-n79A | 3 | 0.6 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |

Table 6.11.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A_n1A-n78A-n79A | 3 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |
|  | n79 | 0 |

6.11.5	MSD

No additional MSD requirement is needed.

6.12		DC_3A_n28A-n77A-n79A

6.12.1	Operating bands for DC

Table 6.12.1-1: EN-DC band combination (four bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_3_n28-n77-n79 | 3 | CA_n28-n77-n79 |

6.12.2	Inter-band DC Configurations

Table 6.12.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A_n28A-n77A-n79A | DC_3A_n28A DC_3A_n77A DC_3A_n79A |

6.12.3	Co-existence studies

Co-existence study for DC_3A_n28A-n77A-n79A was covered by the studies for the fallback modes of DC_3A_n28A-n77A, DC_3A_n28A-n79A and DC_3A_n77A-n79A.

6.12.4	∆TIB and ∆RIB values

For DC_3A_n28A-n77A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 6.12.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_3A_n28A-n77A-n79A | 3 | 0.6 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 6.12.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A_n28A-n77A-n79A | 3 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0 |

6.12.5	MSD

No additional MSD requirement is needed.

6.13		DC_3A_n28A-n78A-n79A

6.13.1	Operating bands for DC

Table 6.13.1-1: EN-DC band combination (four bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_3_n28-n78-n79 | 3 | CA_n28-n78-n79 |

6.13.2	Inter-band DC Configurations

Table 6.13.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A_n28A-n78A-n79A | DC_3A_n28A DC_3A_n78A DC_3A_n79A |

6.13.3	Co-existence studies

Co-existence study for DC_3A_n28A-n78A-n79A was covered by the studies for the fallback modes of DC_3A_n28A-n78A, DC_3A_n28A-n79A and DC_3A_n78A-n79A.

6.13.4	∆TIB and ∆RIB values

For DC_3A_n28A-n78A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 6.13.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_3A_n28A-n78A-n79A | 3 | 0.6 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |

Table 6.13.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A_n28A-n78A-n79A | 3 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
|  | n79 | 0 |

6.13.5	MSD

No additional MSD requirement is needed.

6.14		DC_19A_n1A-n77A-n79A

6.14.1	Operating bands for DC

Table 6.14.1-1: EN-DC band combination (four bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_19_n1-n77-n79 | 19 | CA_n1-n77-n79 |

6.14.2	Inter-band DC Configurations

Table 6.14.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_19A_n1A-n77A-n79A | DC_19A_n1A DC_19A_n77A DC_19A_n79A |

6.14.3	Co-existence studies

Co-existence study for DC_19A_n1A-n77A-n79A was covered by the studies for the fallback modes of DC_19A_n1A-n77A, DC_19A_n1A-n79A and DC_19A_n77A-n79A.

6.14.4	∆TIB and ∆RIB values

For DC_19A_n1A-n77A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 6.14.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_19A_n1A-n77A-n79A | 19 | 0.3 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 6.14.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19A_n1A-n77A-n79A | 19 | 0.3 |
|  | n1 | 0.3 |
|  | n77 | 0.5 |
|  | n79 | 0 |

6.14.5	MSD

No additional MSD requirement is needed.

6.15		DC_19A_n1A-n78A-n79A

6.15.1	Operating bands for DC

Table 6.15.1-1: EN-DC band combination (four bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_19_n1-n78-n79 | 19 | CA_n1-n78-n79 |

6.15.2	Inter-band DC Configurations

Table 6.15.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_19A_n1A-n78A-n79A | DC_19A_n1A DC_19A_n78A DC_19A_n79A |

6.15.3	Co-existence studies

Co-existence study for DC_19A_n1A-n78A-n79A was covered by the studies for the fallback modes of DC_19A_n1A-n78A, DC_19A_n1A-n79A and DC_19A_n78A-n79A.

6.15.4	∆TIB and ∆RIB values

For DC_19A_n1A-n78A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 6.15.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_19A_n1A-n78A-n79A | 19 | 0.3 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |

Table 6.15.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19A_n1A-n78A-n79A | 19 | 0.3 |
|  | n1 | 0.3 |
|  | n78 | 0.5 |
|  | n79 | 0 |

6.15.5	MSD

No additional MSD requirement is needed.

6.16		DC_21A_n1A-n77A-n79A

6.16.1	Operating bands for DC

Table 6.16.1-1: EN-DC band combination (four bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_21_n1-n77-n79 | 21 | CA_n1-n77-n79 |

6.16.2	Inter-band DC Configurations

Table 6.16.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_21A_n1A-n77A-n79A | DC_21A_n1A DC_21A_n77A DC_21A_n79A |

6.16.3	Co-existence studies

Co-existence study for DC_21A_n1A-n77A-n79A was covered by the studies for the fallback modes of DC_21A_n1A-n77A, DC_21A_n1A-n79A and DC_21A_n77A-n79A.

6.16.4	∆TIB and ∆RIB values

For DC_21A_n1A-n77A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 6.16.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_21_n1-n77-n79 | 21 | 0.4 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 6.16.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21_n1-n77-n79 | n1 | 0.2 |
|  | n77 | 0.5 |

6.16.5	MSD

No additional MSD requirement is needed.

## 6.17		DC_21A_n1A-n78A-n79A

6.17.1	Operating bands for DC

Table 6.17.1-1: EN-DC band combination (four bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_21_n1-n78-n79 | 21 | CA_n1-n78-n79 |

6.17.2	Inter-band DC Configurations

Table 6.17.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_21A_n1A-n78A-n79A | DC_21A_n1A DC_21A_n78A DC_21A_n79A |

6.17.3	Co-existence studies

Co-existence study for DC_21A_n1A-n78A-n79A was covered by the studies for the fallback modes of DC_21A_n1A-n78A, DC_21A_n1A-n79A and DC_21A_n78A-n79A.

6.17.4	∆TIB and ∆RIB values

For DC_21A_n1A-n78A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 6.17.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_21A_n1A-n78A-n79A | 21 | 0.4 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |

Table 6.17.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21A_n1A-n78A-n79A | 21 | 0 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |
|  | n79 | 0 |

6.17.5	MSD

No additional MSD requirement is needed.

6.18		DC_21A_n28A-n77A-n79A

6.18.1	Operating bands for DC

Table 6.18.1-1: EN-DC band combination (four bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_21_n28-n77-n79 | 21 | CA_n28-n77-n79 |

6.18.2	Inter-band DC Configurations

Table 6.18.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_21A_n28A-n77A-n79A | DC_21A_n28A DC_21A_n77A DC_21A_n79A |

6.18.3	Co-existence studies

Co-existence study for DC_21A_n28A-n77A-n79A was covered by the studies for the fallback modes of DC_21A_n28A-n77A, DC_21A_n28A-n79A and DC_21A_n77A-n79A.

6.18.4	∆TIB and ∆RIB values

For DC_21A_n28A-n77A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 6.18.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_21A_n28A-n77A-n79A | 21 | 0.4 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 6.18.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21A_n28A-n77A-n79A | 21 | 0 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0 |

6.18.5	MSD

No additional MSD requirement is needed.

6.19		DC_21A_n28A-n78A-n79A

6.19.1	Operating bands for DC

Table 6.19.1-1: EN-DC band combination (four bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_21_n28-n78-n79 | 21 | CA_n28-n78-n79 |

6.19.2	Inter-band DC Configurations

Table 6.19.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_21A_n28A-n78A-n79A | DC_21A_n28A DC_21A_n78A DC_21A_n79A |

6.19.3	Co-existence studies

Co-existence study for DC_21A_n28A-n78A-n79A was covered by the studies for the fallback modes of DC_21A_n28A-n78A, DC_21A_n28A-n79A and DC_21A_n78A-n79A.

6.19.4	∆TIB and ∆RIB values

For DC_21A_n28A-n78A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 6.19.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_21_n28-n78-n79 | 21 | 0.4 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 6.19.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21_n28-n78-n79 | 21 | 0 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
|  | n79 | 0 |

6.19.5	MSD

No additional MSD requirement is needed.

6.20		DC_42A_n1A-n77A-n79A

6.20.1	Operating bands for DC

Table 6.20.1-1: EN-DC band combination (four bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_42_n1-n77-n79 | 42 | CA_n1-n77-n79 |

6.20.2	Inter-band DC Configurations

Since Uplink EN-DC configuration is N/A, the combination is not used alone as described in NOTE X. This is the same NOTE with NOTE 9 specified in Table 5.5B.4.1-1 in TS 38.101-3 v17.0.0.

Table 6.20.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_42A_n1A-n77A-n79A | N/A |
| NOTE X: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used. |  |

6.20.3	Co-existence studies

Co-existence study for DC_42A_n1A-n77A-n79A was covered by the studies for the fallback modes of DC_42A_n1A-n77A, DC_42A_n1A-n79A and DC_42A_n77A-n79A.

6.20.4	∆TIB and ∆RIB values

For DC_42A_n1A-n77A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 6.20.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_42_n1-n77-n79 | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 6.20.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42_n1-n77-n79 | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

6.20.5	MSD

No additional MSD requirement is needed.

6.21		DC_42A_n1A-n78A-n79A

6.21.1	Operating bands for DC

Table 6.21.1-1: EN-DC band combination (four bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_42_n1-n78-n79 | 42 | CA_n1-n78-n79 |

6.21.2	Inter-band DC Configurations

Since Uplink EN-DC configuration is N/A, the combination is not used alone as described in NOTE X. This is the same NOTE with NOTE 9 specified in Table 5.5B.4.1-1 in TS 38.101-3 v17.0.0.

Table 6.21.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_42A_n1A-n78A-n79A | N/A |
| NOTE X: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used. |  |

6.21.3	Co-existence studies

Co-existence study for DC_42A_n1A-n78A-n79A was covered by the studies for the fallback modes of DC_42A_n1A-n78A, DC_42A_n1A-n79A and DC_42A_n78A-n79A.

6.21.4	∆TIB and ∆RIB values

For DC_42A_n1A-n78A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 6.21.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_42A_n1A-n78A-n79A | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |
|  | n79 | 0 |

Table 6.21.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42A_n1A-n78A-n79A | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |
|  | n79 | 0 |

6.21.5	MSD

No additional MSD requirement is needed.

6.22			DC_8A_n39-n40-n41

6.22.1	Operating bands for DC

Table 6.22.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_8_n39-n40-n41 | 8 | CA_n39-n40-n41 |

### 6.22.2	Inter-band DC Configurations

Table 6.22.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8A_n39A-n40A-n41A | DC_8A_n39A DC_8A_n40A DC_8A_n41A |

6.22.3	Co-existence studies

For DC_8A_n39A-n40A-n41A co-existence studies, the lower order DC_8A_n40A-n41A, DC_8A_n39A-n40A and DC_8A_n39A-n41A can be applied.

6.22.4	∆TIB and ∆RIB values

For DC_8A_n40A-n41A-n79A , the TIB,c and RIB,c values are given in the tables below.

Table 6.22.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n39-n40-n41 | 8 | 0.3 |
|  | n39 | 0.3 |
|  | n40 | 0.3 |
|  | n41 | 0.3 |

Table 6.22.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n39-n40-n41 | 8 | 0 |
|  | n39 | 0 |
|  | n40 | 0 |
|  | n41 | 0 |

6.22.5	MSD

No additional MSD requirement is needed.

6.23			DC_8A_n39-n40-n79

6.23.1	Operating bands for DC

Table 6.23.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_8_n39-n40-n79 | 8 | CA_n39-n40-n79 |

### 6.23.2	Inter-band DC Configurations

Table 6.23.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8A_n39A-n40A-n79A | DC_8A_n39A DC_8A_n40A DC_8A_n79A |

6.23.3	Co-existence studies

For DC_8A_n39A-n40A-n79A co-existence studies, the lower order DC_8A_n40A-n79A, DC_8A_n39A-n40A and DC_8A_n39A-n79A can be applied.

6.23.4	∆TIB and ∆RIB values

For DC_8A_n40A-n79A-n79A , the TIB,c and RIB,c values are given in the tables below.

Table 6.23.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n39-n40-n79 | 8 | 0.3 |
|  | n39 | 0.3 |
|  | n40 | 0.3 |
|  | n79 | 0.8 |

Table 6.23.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n39-n40-n79 | 8 | 0 |
|  | n39 | 0.3 |
|  | n40 | 0.3 |
|  | n79 | 0.5 |

6.23.5	MSD

No additional MSD requirement is needed.

- 6.24			DC_3_n41-n79-n258

- 6.24.1	Operating bands for DC

Table 6.24.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| - DC_3_n41-n79-n258 | 3 | CA_n41-n79-n258 |

### 6.24.2	Inter-band DC Configurations

Table 6.24.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A_n41A-n79A-n258A | DC_3A_n41A DC_3A_n79A DC_3A_n258A |

- 6.24.3	Co-existence studies

For DC_3A_n41A-n79A-n258A  co-existence studies, the lower order DC_3A_n41A-n79A, DC_3A_n41A-n258A and DC_3A_n79A-n258A can be applied.

- 6.24.4	∆TIB and ∆RIB values

For DC_3A_n41A-n79A-n258A, TIB,c and RIB,c values are set to zero for n258, and the values for constituent E-UTRA and FR1 NR bands are same as those for the corresponding inter band EN-DC configurations which are defined in the TS 38.101-3 already.

- 6.24.5	MSD

No additional MSD requirement is needed.

6.25	DC_1_n3-n28-n79

6.25.1	Operating bands for DC

Table 6.25.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_1_n3-n28-n79 | 1 | CA_n3-n28-n79 |

6.25.2	Inter-band DC Configurations

Table 6.25.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A_n3A-n28A-n79A | DC_1A_n3A DC_1A_n28A DC_1A_n79A |

6.25.3	Co-existence studies

Co-existence study for DC_1_n3-n28-n79 was covered by the studies for the fallback modes of DC_1_n3-n28, DC_1_n3-n79 and DC_1_n28-n79.

No additional MSD requirement needs to be defined for this dual connectivity configuration.

6.25.4	∆TIB and ∆RIB values

For DC_1_n3-n28-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.25.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n3-n28-n79 | 1 | 0.3 |
|  | n3 | 0.3 |
|  | n28 | 0.8 |
|  | n79 | 0.8 |

Table 6.25.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n3-n28-n79 | 1 | 0 |
|  | n3 | 0 |
|  | n28 | 0.5 |
|  | n79 | 0.5 |

6.25.5	MSD

As mentioned in 6.25.3, there is no need to specify additional MSD requirement for this UL DC configuration..

6.26	DC_1_n3-n77-n79

6.26.1	Operating bands for DC

Table 6.26.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_1_n3-n77-n79 | 1 | CA_n3-n77-n79 |

6.26.2	Inter-band DC Configurations

Table 6.26.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A_n3A-n77A-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A |

6.26.3	Co-existence studies

Co-existence study for DC_1_n3-n77-n79 was covered by the studies for the fallback modes of DC_1_n3-n77, DC_1_n3-n79 and DC_1_n77-n79.

No additional MSD requirement needs to be defined for this dual connectivity configuration.

6.26.4	∆TIB and ∆RIB values

For DC_1_n3-n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.26.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n3-n77-n79 | 1 | 0.6 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.8 |

Table 6.26.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n3-n77-n79 | 1 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0.5 |

6.26.5	MSD

As mentioned in 6.26.3, there is no need to specify additional MSD requirement for this UL DC configuration..

6.27	DC_8_n3-n77-n79

6.27.1	Operating bands for DC

Table 6.27.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_8_n3-n77-n79 | 8 | CA_n3-n77-n79 |

6.27.2	Inter-band DC Configurations

Table 6.27.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8A_n3A-n77A-n79A | DC_8A_n3A DC_8A_n77A DC_8A_n79A |

6.27.3	Co-existence studies

Co-existence study for DC_8_n3-n77-n79 was covered by the studies for the fallback modes of DC_8_n3-n77, DC_8_n3-n79 and DC_8_n77-n79.

No additional MSD requirement needs to be defined for this dual connectivity configuration.

6.27.4	∆TIB and ∆RIB values

For DC_8_n3-n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.27.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n3-n77-n79 | 8 | 0.6 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.8 |

Table 6.27.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n3-n77-n79 | 8 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0.5 |

6.27.5	MSD

As mentioned in 6.27.3, there is no need to specify additional MSD requirement for this UL DC configuration..

6.28	DC_8_n3-n28-n79

6.28.1	Operating bands for DC

Table 6.28.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_8_n3-n28-n79 | 8 | CA_n3-n28-n79 |

6.28.2	Inter-band DC Configurations

Table 6.28.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8A_n3A-n28A-n79A | DC_8A_n3A DC_8A_n28A DC_8A_n79A |

6.28.3	Co-existence studies

Co-existence study for DC_8_n3-n28-n79 was covered by the studies for the fallback modes of DC_8_n3-n28, DC_8_n3-n79 and DC_8_n28-n79.

No additional MSD requirement needs to be defined for this dual connectivity configuration.

6.28.4	∆TIB and ∆RIB values

For DC_8_n3-n28-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.28.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n3-n28-n79 | 8 | 0.6 |
|  | n3 | 0.5 |
|  | n28 | 0.3 |
|  | n79 | 0.8 |

Table 6.28.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n3-n28-n79 | 8 | 0.2 |
|  | n3 | 0 |
|  | n28 | 0.2 |
|  | n79 | 0.5 |

6.28.5	MSD

As mentioned in 6.28.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.29	DC_8_n28-n77-n79

6.29.1	Operating bands for DC

Table 6.29.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_8_n28-n77-n79 | 8 | CA_n28-n77-n79 |

6.29.2	Inter-band DC Configurations

Table 6.29.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8A_n28A-n77A-n79A | DC_8A_n28A DC_8A_n77A DC_8A_n79A |

6.29.3	Co-existence studies

Co-existence study for DC_8_n28-n77-n79 was covered by the studies for the fallback modes of DC_8_n28-n77, DC_8_n28-n79 and DC_8_n77-n79.

No additional MSD requirement needs to be defined for this dual connectivity configuration.

6.29.4	∆TIB and ∆RIB values

For DC_8_n28-n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.29.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n28-n77-n79 | 8 | 0.6 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
|  | n79 | 0.8 |

Table 6.29.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n28-n77-n79 | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0.5 |

6.29.5	MSD

As mentioned in 6.29.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.30	DC_3_n1-n8-n78, DC_3-3_n1-n8-n78

6.30.1	Operating bands for DC

Table 6.30.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_3_n1-n8-n78 DC_3-3_n1-n8-n78 | 3 CA_3-3 | CA_n1-n8-n78 |

6.30.2	Inter-band DC Configurations

Table 6.30.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A_n1A-n8A-n78A2 DC_3A-3A_n1A-n8A-n78A2 | DC_3A_n1A DC_3A_n8A DC_3A_n78A |
| NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

6.30.3	Co-existence studies

The co-existence studies can be covered by the studies of the constituent combinations mentioned in 37.716-21-21.

6.30.4	∆TIB and ∆RIB values

For DC_3_n1-n8-n78, DC_3-3_ n1-n8-n78, the TIB,c and RIB,c values are given in the tables below.

Table 6.30.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n1-n8-n78 | 3 | 0.6 |
|  | n1 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 6.30.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n1-n8-n78 | 3 | 0.2 |
|  | n1 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

6.30.5	MSD

No additional MSD requirement is needed.

<Next session>

6.31	DC_7_n1-n8-n78, DC_7-7_n1-n8-n78

6.31.1	Operating bands for DC

Table 6.30.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_7_n1-n8-n78 DC_7-7_n1-n8-n78 | 7 CA_7-7 | CA_n1-n8-n78 |

6.31.2	Inter-band DC Configurations

Table 6.30.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A_n1A-n8A-n78A2 DC_7A-7A_n1A-n8A-n78A2 | DC_7A_n1A DC_7A_n8A DC_7A_n78A |
| NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

6.31.3	Co-existence studies

The co-existence studies can be covered by the studies of the constituent combinations mentioned in 37.716-21-21.

6.31.4	∆TIB and ∆RIB values

For DC_7_n1-n8-n78, DC_7-7_ n1-n8-n78, the TIB,c and RIB,c values are given in the tables below.

Table 6.31.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n1-n8-n78 | 7 | 0.6 |
|  | n1 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 6.31.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n1-n8-n78 | 7 | 0.2 |
|  | n1 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

6.31.5	MSD

No additional MSD requirement is needed.

6.32	DC_8_n1-n3-n77

6.32.1	Operating bands for DC

Table 6.32.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_8_n1-n3-n77 | 8 | CA_n1-n3-n77 |

6.32.2	Inter-band DC Configurations

Table 6.32.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8A_n1A-n3A-n77A | DC_8A_n1A DC_8A_n3A DC_8A_n77A |

6.32.3	Co-existence studies

Co-existence study for DC_8_n1-n3-n77 was covered by the studies for the fallback modes of DC_8_n1-n3, DC_8_n1-n77 and DC_8_n3-n77.

No additional MSD requirement needs to be defined for this dual connectivity configuration.

6.32.4	∆TIB and ∆RIB values

For DC_8_n1-n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.32.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n1-n3-n77 | 8 | 0.6 |
|  | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 6.32.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n1-n3-n77 | 8 | 0.2 |
|  | n1 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

6.32.5	MSD

As mentioned in 6.32.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.33	DC_11_n3-n77-n79

6.33.1	Operating bands for DC

Table 6.33.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_11_n3-n77-n79 | 11 | CA_n3-n77-n79 |

6.33.2	Inter-band DC Configurations

Table 6.33.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_11A_n3A-n77A-n79A | DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_11A_n3A-n77(2A)-n79A | DC_11A_n3A DC_11A_n77A DC_11A_n79A |

6.33.3	Co-existence studies

Co-existence study for DC_11_n3-n77-n79 was covered by the studies for the fallback modes of DC_11_n3-n77, DC_11_n3-n79 and DC_11_n77-n79.

No additional MSD requirement needs to be defined for this dual connectivity configuration.

6.33.4	∆TIB and ∆RIB values

For DC_11_n3-n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.33.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11_n3-n77-n79 | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n77 | 0.8 |
|  | n79 | 0.8 |

Table 6.33.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11_n3-n77-n79 | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n77 | 0.5 |
|  | n79 | 0.5 |

6.33.5	MSD

As mentioned in 6.33.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7	DC band combinations of LTE 2 bands DL/1UL + NR 3 bands DL/1UL: Specific Band Combination Part

7.1	DC_3-7_n1-n78-n257, DC_3-3-7_n1-n78-n257, DC_3-7-7_n1-n78-n257, DC_3-3-7-7_n1-n78-n257

7.1.1	Operating bands for DC

Table 7.1.1-1: EN-DC band combination (five bands)

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_3-7_n1-n78-n257 DC_3-3-7_n1-n78-n257 DC_3-7-7_n1-n78-n257 DC_3-3-7-7_n1-n78-n257 | CA_3-7 CA_3-3-7 CA_3-7-7 CA_3-3-7-7 | CA_n1-n78-n257 |

7.1.2	Inter-band DC Configurations

Table 7.1.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A_n1A-n78A-n257A DC_3A-7A_n1A-n78A-n257D DC_3A-7A_n1A-n78A-n257E DC_3A-7A_n1A-n78A-n257F DC_3A-7A_n1A-n78A-n257G DC_3A-7A_n1A-n78A-n257H DC_3A-7A_n1A-n78A-n257I DC_3A-7A_n1A-n78A-n257J DC_3A-7A_n1A-n78A-n257K DC_3A-7A_n1A-n78A-n257L DC_3A-7A_n1A-n78A-n257M | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A |
| DC_3A-3A-7A_n1A-n78A-n257A DC_3A-3A-7A_n1A-n78A-n257D DC_3A-3A-7A_n1A-n78A-n257E DC_3A-3A-7A_n1A-n78A-n257F DC_3A-3A-7A_n1A-n78A-n257G DC_3A-3A-7A_n1A-n78A-n257H DC_3A-3A-7A_n1A-n78A-n257I DC_3A-3A-7A_n1A-n78A-n257J DC_3A-3A-7A_n1A-n78A-n257K DC_3A-3A-7A_n1A-n78A-n257L DC_3A-3A-7A_n1A-n78A-n257M | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A |
| DC_3A-7A-7A_n1A-n78A-n257A DC_3A-7A-7A_n1A-n78A-n257D DC_3A-7A-7A_n1A-n78A-n257E DC_3A-7A-7A_n1A-n78A-n257F DC_3A-7A-7A_n1A-n78A-n257G DC_3A-7A-7A_n1A-n78A-n257H DC_3A-7A-7A_n1A-n78A-n257I DC_3A-7A-7A_n1A-n78A-n257J DC_3A-7A-7A_n1A-n78A-n257K DC_3A-7A-7A_n1A-n78A-n257L DC_3A-7A-7A_n1A-n78A-n257M | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A |
| DC_3A-3A-7A-7A_n1A-n78A-n257A DC_3A-3A-7A-7A_n1A-n78A-n257D DC_3A-3A-7A-7A_n1A-n78A-n257E DC_3A-3A-7A-7A_n1A-n78A-n257F DC_3A-3A-7A-7A_n1A-n78A-n257G DC_3A-3A-7A-7A_n1A-n78A-n257H DC_3A-3A-7A-7A_n1A-n78A-n257I DC_3A-3A-7A-7A_n1A-n78A-n257J DC_3A-3A-7A-7A_n1A-n78A-n257K DC_3A-3A-7A-7A_n1A-n78A-n257L DC_3A-3A-7A-7A_n1A-n78A-n257M | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A |

7.1.3	Co-existence studies

The co-existence studies can be covered by the studies for the constituent combinations mentioned in 37.716-21-21.

7.1.4	∆TIB and ∆RIB values

For DC_3-7_n1-n78-n257, DC_3-3-7_n1-n78-n257, DC_3-7-7_n1-n78-n257, DC_3-3-7-7_n1-n78-n257, TIB,c and RIB,c values are set to zero for n257, and the values for constituent E-UTRA and FR1 NR bands are same as those for the corresponding inter band EN-DC configurations which are defined in the TS 38.101-3 already.

7.1.5	MSD

No additional MSD requirement is needed.

7.2	DC_1-8_n3-n28-n77

7.2.1	Operating bands for DC

Table 7.2.1-1: EN-DC band combination (five bands)

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_1-8_n3-n28-n77 | CA_1-8 | CA_n3-n28-n77 |

7.2.2	Inter-band DC Configurations

Table 7.2.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-8A_n3A-n28A-n77A | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A |
| DC_1A-8A_n3A-n28A-n77(2A) | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A |

7.2.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR3DL/1UL are already covered by the fallback configurations, DC_1-8_n3-n28, DC_1-8_n3-n77, DC_1-8_n28-n77, DC_1_n3_n28-n77 and DC_8_n3-n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.2.4	∆TIB and ∆RIB values

For DC_1-8_n3-n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n3-n28-n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | n3 | 0.8 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 8.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n3-n28-n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | n3 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

7.2.5	MSD

As mentioned in 7.2.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.3	DC_1-11_n3-n28-n77

7.3.1	Operating bands for DC

Table 7.3.1-1: EN-DC band combination (five bands)

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_1-11_n3-n28-n77 | CA_1-11 | CA_n3-n28-n77 |

7.3.2	Inter-band DC Configurations

Table 7.3.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-11A_n3A-n28A-n77A | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_1A-11A_n3A-n28A-n77(2A) | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |

7.3.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR3DL/1UL are already covered by the fallback configurations, DC_1-11_n3-n28, DC_1-11_n3-n77, DC_1-11_n28-n77, DC_1_n3_n28-n77 and DC_11_n3-n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.3.4	∆TIB and ∆RIB values

For DC_1-11_n3-n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11_n3-n28-n77 | 1 | 0.6 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 8.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-11_n3-n28-n77 | 1 | 0.2 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n28 | 0.2 |
|  | n77 | 0.3 |

7.3.5	MSD

As mentioned in 7.3.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.4	DC_1-42_n3-n28-n77

7.4.1	Operating bands for DC

Table 7.4.1-1: EN-DC band combination (five bands)

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_1-42_n3-n28-n77 | CA_1-42 | CA_n3-n28-n77 |

7.4.2	Inter-band DC Configurations

Table 7.4.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-42A_n3A-n28A-n77A | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42A_n28A |
| DC_1A-42A_n3A-n28A-n77(2A) | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42A_n28A |
| DC_1A-42C_n3A-n28A-n77A | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_1A-42C_n3A-n28A-n77(2A) | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |

7.4.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR3DL/1UL are already covered by the fallback configurations, DC_1-42_n3-n28, DC_1-42_n3-n77, DC_1-42_n28-n77, DC_1_n3_n28-n77 and DC_42_n3-n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.4.4	∆TIB and ∆RIB values

For DC_1-42_n3-n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-42_n3-n28-n77 | 1 | 0.6 |
|  | 42 | 0.8 |
|  | n3 | 0.8 |
|  | n28 | 0.8 |
|  | n77 | 0.8 |

Table 8.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-42_n3-n28-n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n28 | 0.5 |
|  | n77 | 0.5 |

7.4.5	MSD

As mentioned in 7.4.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.5	DC_8-11_n3-n28-n77

7.5.1	Operating bands for DC

Table 7.5.1-1: EN-DC band combination (five bands)

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_8-11_n3-n28-n77 | CA_8-11 | CA_n3-n28-n77 |

7.5.2	Inter-band DC Configurations

Table 7.5.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8A-11A_n3A-n28A-n77A | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_8A-11A_n3A-n28A-n77(2A) | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |

7.5.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR3DL/1UL are already covered by the fallback configurations, DC_8-11_n3-n28, DC_8-11_n3-n77, DC_8-11_n28-n77, DC_8_n3_n28-n77 and DC_11_n3-n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.5.4	∆TIB and ∆RIB values

For DC_8-11_n3-n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.5.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-11_n3-n28-n77 | 8 | 0.6 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 8.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-11_n3-n28-n77 | 8 | 0.2 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

7.5.5	MSD

As mentioned in 7.5.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.6	DC_8-42_n3-n28-n77

7.6.1	Operating bands for DC

Table 7.6.1-1: EN-DC band combination (five bands)

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_8-42_n3-n28-n77 | CA_8-42 | CA_n3-n28-n77 |

7.6.2	Inter-band DC Configurations

Table 7.6.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8A-42A_n3A-n28A-n77A | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |
| DC_8A-42A_n3A-n28A-n77(2A) | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |
| DC_8A-42C_n3A-n28A-n77A | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_8A-42C_n3A-n28A-n77(2A) | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |

7.6.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR3DL/1UL are already covered by the fallback configurations, DC_8-42_n3-n28, DC_8-42_n3-n77, DC_8-42_n28-n77, DC_8_n3_n28-n77 and DC_42_n3-n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.6.4	∆TIB and ∆RIB values

For DC_8-42_n3-n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-42_n3-n28-n77 | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n3 | 0.6 |
|  | n28 | 0.8 |
|  | n77 | 0.8 |

Table 8.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-42_n3-n28-n77 | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n28 | 0.5 |
|  | n77 | 0.5 |

7.6.5	MSD

As mentioned in 7.6.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.7		DC_1A-3A_n28A-n77A-n79A

7.7.1	Operating bands for DC

Table 7.7.1-1: EN-DC band combination (five bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_1-3_n28-n77-n79 | CA_1-3 | CA_n28-n77-n79 |

7.7.2	Inter-band DC Configurations

Table 7.7.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A DC_3A_n28A DC_3A_n77A DC_3A_n79A |

7.7.3	Co-existence studies

Co-existence study of this DL of LTE 2 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of DL of LTE 1 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.7.4	∆TIB and ∆RIB values

For DC_1A-3A_n28A-n77A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 7.7.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_1A-3A_n28A-n77A-n79A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 7.7.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A_n28A-n77A-n79A | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0 |

7.7.5	MSD

No additional MSD requirement is needed.

7.8		DC_1A-3A_n28A-n78A-n79A

7.8.1	Operating bands for DC

Table 7.8.1-1: EN-DC band combination (five bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_1-3_n28-n78-n79 | CA_1-3 | CA_n28-n78-n79 |

7.8.2	Inter-band DC Configurations

Table 7.8.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A_n28A-n78A-n79A | DC_1A_n28A DC_1A_n78A DC_1A_n79A DC_3A_n28A DC_3A_n78A DC_3A_n79A |

7.8.3	Co-existence studies

Co-existence study of this DL of LTE 2 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of DL of LTE 1 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.8.4	∆TIB and ∆RIB values

For DC_1A-3A_n28A-n78A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 7.8.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_1A-3A_n28A-n78A-n79A | 1 | 0.3 |
|  | 3 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |

Table 7.8.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A_n28A-n78A-n79A | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
|  | n79 | 0 |

7.8.5	MSD

No additional MSD requirement is needed.

7.9		DC_1A-21A_n28A-n77A-n79A

7.9.1	Operating bands for DC

Table 7.9.1-1: EN-DC band combination (five bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_1-21_n28-n77-n79 | CA_1-21 | CA_n28-n77-n79 |

7.9.2	Inter-band DC Configurations

Table 7.9.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-21A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A DC_21A_n28A DC_21A_n77A DC_21A_n79A |

7.9.3	Co-existence studies

Co-existence study of this DL of LTE 2 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of DL of LTE 1 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.9.4	∆TIB and ∆RIB values

For DC_1A-21A_n28A-n77A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 7.9.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_1A-21A_n28A-n77A-n79A | 1 | 0.6 |
|  | 21 | 0.4 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 7.9.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-21A_n28A-n77A-n79A | 1 | 0.3 |
|  | 21 | 0 |
|  | n28 | 0.3 |
|  | n77 | 0.5 |
|  | n79 | 0 |

7.9.5	MSD

No additional MSD requirement is needed.

7.10		DC_1A-21A_n28A-n78A-n79A

7.10.1	Operating bands for DC

Table 7.10.1-1: EN-DC band combination (five bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_1-21_n28-n78-n79 | CA_1-21 | CA_n28-n78-n79 |

7.10.2	Inter-band DC Configurations

Table 7.10.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-21A_n28A-n78A-n79A | DC_1A_n28A DC_1A_n78A DC_1A_n79A DC_21A_n28A DC_21A_n78A DC_21A_n79A |

7.10.3	Co-existence studies

Co-existence study of this DL of LTE 2 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of DL of LTE 1 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.10.4	∆TIB and ∆RIB values

For DC_1A-21A_n28A-n78A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 7.10.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_1A-21A_n28A-n78A-n79A | 1 | 0.6 |
|  | 21 | 0.4 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |

Table 7.10.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-21A_n28A-n78A-n79A | 1 | 0.3 |
|  | 21 | 0 |
|  | n28 | 0.3 |
|  | n78 | 0.5 |
|  | n79 | 0 |

7.10.5	MSD

No additional MSD requirement is needed.

7.11		DC_3A-21A_n1A-n77A-n79A

7.11.1	Operating bands for DC

Table 7.11.1-1: EN-DC band combination (five bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_3-21_n1-n77-n79 | CA_3-21 | CA_n1-n77-n79 |

7.11.2	Inter-band DC Configurations

Table 7.11.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-21A_n1A-n77A-n79A | DC_3A_n1A DC_3A_n77A DC_3A_n79A DC_21A_n1A DC_21A_n77A DC_21A_n79A |

7.11.3	Co-existence studies

Co-existence study of this DL of LTE 2 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of DL of LTE 1 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.11.4	∆TIB and ∆RIB values

For DC_3A-21A_n1A-n77A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 7.11.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_3A-21A_n1A-n77A-n79A | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 7.11.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-21A_n1A-n77A-n79A | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0 |

7.11.5	MSD

No additional MSD requirement is needed.

7.12		DC_3A-21A_n1A-n78A-n79A

7.12.1	Operating bands for DC

Table 7.12.1-1: EN-DC band combination (five bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_3-21_n1-n78-n79 | CA_3-21 | CA_n1-n78-n79 |

7.12.2	Inter-band DC Configurations

Table 7.12.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-21A_n1A-n78A-n79A | DC_3A_n1A DC_3A_n78A DC_3A_n79A DC_21A_n1A DC_21A_n78A DC_21A_n79A |

7.12.3	Co-existence studies

Co-existence study of this DL of LTE 2 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of DL of LTE 1 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.12.4	∆TIB and ∆RIB values

For DC_3A-21A_n1A-n78A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 7.12.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_3A-21A_n1A-n78A-n79A | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |

Table 7.12.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-21A_n1A-n78A-n79A | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |
|  | n79 | 0 |

7.12.5	MSD

No additional MSD requirement is needed.

7.13		DC_3A-21A_n28A-n77A-n79A

7.13.1	Operating bands for DC

Table 7.13.1-1: EN-DC band combination (five bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_3-21_n28-n77-n79 | CA_3-21 | CA_n28-n77-n79 |

7.13.2	Inter-band DC Configurations

Table 7.13.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-21A_n28A-n77A-n79A | DC_3A_n28A DC_3A_n77A DC_3A_n79A DC_21A_n28A DC_21A_n77A DC_21A_n79A |

7.13.3	Co-existence studies

Co-existence study of this DL of LTE 2 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of DL of LTE 1 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.13.4	∆TIB and ∆RIB values

For DC_3A-21A_n28A-n77A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 7.13.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_3A-21A_n28A-n77A-n79A | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 7.13.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-21A_n28A-n77A-n79A | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0 |

7.13.5	MSD

No additional MSD requirement is needed.

7.14		DC_3A-21A_n28A-n78A-n79A

7.14.1	Operating bands for DC

Table 7.14.1-1: EN-DC band combination (five bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_3-21_n28-n78-n79 | CA_3-21 | CA_n28-n78-n79 |

7.14.2	Inter-band DC Configurations

Table 7.14.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-21A_n28A-n78A-n79A | DC_3A_n28A DC_3A_n78A DC_3A_n79A DC_21A_n28A DC_21A_n78A DC_21A_n79A |

7.14.3	Co-existence studies

Co-existence study of this DL of LTE 2 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of DL of LTE 1 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.14.4	∆TIB and ∆RIB values

For DC_3A-21A_n28A-n78A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 7.14.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_3A-21A_n28A-n78A-n79A | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |

Table 7.14.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-21A_n28A-n78A-n79A | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
|  | n79 | 0 |

7.14.5	MSD

No additional MSD requirement is needed.

7.15		DC_19A-42A_n1A-n77A-n79A

7.15.1	Operating bands for DC

Table 7.15.1-1: EN-DC band combination (five bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_19-42_n1-n77-n79 | CA_19-42 | CA_n1-n77-n79 |

7.15.2	Inter-band DC Configurations

Table 7.15.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_19A-42A_n1A-n77A-n79A | DC_19A_n1A DC_19A_n77A DC_19A_n79A |

7.15.3	Co-existence studies

Co-existence study of this DL of LTE 2 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of DL of LTE 1 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.15.4	∆TIB and ∆RIB values

For DC_19A-42A_n1A-n77A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 7.15.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_19A-42A_n1A-n77A-n79A | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 7.15.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19A-42A_n1A-n77A-n79A | 19 | 0.3 |
|  | 42 | 0.5 |
|  | n1 | 0.3 |
|  | n77 | 0.5 |
|  | n79 | 0 |

7.15.5	MSD

No additional MSD requirement is needed.

7.16		DC_19A-42A_n1A-n78A-n79A

7.16.1	Operating bands for DC

Table 7.16.1-1: EN-DC band combination (five bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_19-42_n1-n78-n79 | CA_19-42 | CA_n1-n78-n79 |

7.16.2	Inter-band DC Configurations

Table 7.16.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_19A-42A_n1A-n78A-n79A | DC_19A_n1A DC_19A_n78A DC_19A_n79A |

7.16.3	Co-existence studies

Co-existence study of this DL of LTE 2 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of DL of LTE 1 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.16.4	∆TIB and ∆RIB values

For DC_19A-42A_n1A-n78A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 7.16.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_19A-42A_n1A-n78A-n79A | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |

Table 7.16.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19A-42A_n1A-n78A-n79A | 19 | 0.3 |
|  | 42 | 0.5 |
|  | n1 | 0.3 |
|  | n78 | 0.5 |
|  | n79 | 0 |

7.16.5	MSD

No additional MSD requirement is needed.

7.17	DC_1-8_n3-n77-n79

7.17.1	Operating bands for DC

Table 7.17.1-1: EN-DC band combination (five bands)

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_1-8_n3-n77-n79 | CA_1-8 | CA_n3-n77-n79 |

7.17.2	Inter-band DC Configurations

Table 7.17.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-8A_n3A-n77A-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A DC_8A_n3A DC_8A_n77A DC_8A_n79A |
| DC_1A-8A_n3A-n77(2A)-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A DC_8A_n3A DC_8A_n77A DC_8A_n79A |

7.17.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR3DL/1UL are already covered by the fallback configurations, DC_1-8_n3-n77, DC_1-8_n3-n79, DC_1-8_n77-n79, DC_1_n3_n77-n79 and DC_8_n3-n77-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.17.4	∆TIB and ∆RIB values

For DC_1-8_n3-n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.17.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n3-n77-n79 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | n3 | 0.8 |
|  | n77 | 0.8 |
|  | n79 | 0.8 |

Table 8.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n3-n77-n79 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0.5 |

7.17.5	MSD

As mentioned in 7.17.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.18	DC_3-7_n1-n8-n78, DC_3-3-7_n1-n8-n78, DC_3-7-7_n1-n8-n78, DC_3-3-7-7_n1-n8-n78

7.18.1	Operating bands for DC

Table 7.18.1-1: EN-DC band combination (five bands)

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_3-7_n1-n8-n78 DC_3-3-7_n1-n8-n78 DC_3-7-7_n1-n8-n78 DC_3-3-7-7_n1-n8-n78 | CA_3-7 CA_3-3-7 CA_3-7-7 CA_3-3-7-7 | CA_n1-n8-n78 |

7.18.2	Inter-band DC Configurations

Table 7.18.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A_n1A-n8A-n78A2 DC_3A-3A-7A_n1A-n8A-n78A2 DC_3A-7A-7A_n1A-n8A-n78A2 DC_3A-3A-7A-7A_n1A-n8A-n78A2 | DC_3A_n1A DC_3A_n8A DC_3A_n78A DC_7A_n1A DC_7A_n8A DC_7A_n78A |
| NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

7.18.3	Co-existence studies

The co-existence studies can be covered by the studies for the constituent combinations mentioned in 37.716-21-21.

7.18.4	∆TIB and ∆RIB values

For DC_3-7_n1-n8-n78, DC_3-3-7_n1-n8-n78, DC_3-7-7_n1-n8-n78, DC_3-3-7-7_n1-n8-n78, the TIB,c and RIB,c values which are reused from DC_3-3-7-7-8_n1-n78 are given in the tables below.

Table 7.18.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n1-n8-n78, DC_3-3-7_n1-n8-n78, DC_3-7-7_n1-n8-n78, DC_3-3-7-7_n1-n8-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n1 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 7.18.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n1-n8-n78, DC_3-3-7_n1-n8-n78, DC_3-7-7_n1-n8-n78, DC_3-3-7-7_n1-n8-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n1 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

7.18.5	MSD

No additional MSD requirement is needed.

# 7.18	DC_1-8_n3-n28-n79

7.18.1	Operating bands for DC

Table 7.18.1-1: EN-DC band combination (five bands)

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_1-8_n3-n28-n79 | CA_1-8 | CA_n3-n28-n79 |

7.18.2	Inter-band DC Configurations

Table 7.18.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-8A_n3A-n28A-n79A | DC_1A_n3A DC_1A_n28A DC_1A_n79A DC_8A_n3A DC_8A_n28A DC_8A_n79A |

7.18.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR3DL/1UL are already covered by the fallback configurations, DC_1-8_n3-n28, DC_1-8_n3-n79, DC_1-8_n28-n79, DC_1_n3_n28-n79 and DC_8_n3-n28-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.18.4	∆TIB and ∆RIB values

For DC_1-8_n3-n28-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.18.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n3-n28-n79 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | n3 | 0.6 |
|  | n28 | 0.6 |
|  | n79 | 0.8 |

Table 8.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n3-n28-n79 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | n3 | 0.2 |
|  | n28 | 0.2 |
|  | n79 | 0.5 |

7.18.5	MSD

As mentioned in 7.18.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.19	DC_1-8_n28-n77-n79

7.19.1	Operating bands for DC

Table 7.19.1-1: EN-DC band combination (five bands)

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_1-8_n28-n77-n79 | CA_1-8 | CA_n28-n77-n79 |

7.19.2	Inter-band DC Configurations

Table 7.19.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-8A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A DC_8A_n28A DC_8A_n77A DC_8A_n79A |

7.19.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR3DL/1UL are already covered by the fallback configurations, DC_1-8_n28-n77, DC_1-8_n28-n79, DC_1-8_n77-n79, DC_1_n28_n77-n79 and DC_8_n28-n77-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.19.4	∆TIB and ∆RIB values

For DC_1-8_n28-n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.19.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n28-n77-n79 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.8 |

Table 8.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n28-n77-n79 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | n28 | 0.3 |
|  | n77 | 0.5 |
|  | n79 | 0.5 |

7.19.5	MSD

As mentioned in 7.19.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8	DC band combinations of LTE 3 bands DL/1UL + NR 3 bands DL/1UL: Specific Band Combination Part

8.1	DC_1-8-11_n3-n28-n77

8.1.1	Operating bands for DC

Table 8.1.1-1: EN-DC band combination (six bands)

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_1-8-11_n3-n28-n77 | CA_1-8-11 | CA_n3-n28-n77 |

8.1.2	Inter-band DC Configurations

Table 8.1.2-1: Inter-band EN-DC configurations (six bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-8A-11A_n3A-n28A-n77A | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_1A-8A-11A_n3A-n28A-n77(2A) | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |

8.1.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR3DL/1UL are already covered by the fallback configurations, DC_1-8-11_n3-n28, DC_1-8-11_n3-n77, DC_1-8-11_n28-n77, DC_1-8_n3-n28-n77, DC_1-11_n3_n28-n77 and DC_8-11_n3-n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.1.4	∆TIB and ∆RIB values

For DC_1-8-11_n3-n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.1.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n3-n28-n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 8.1.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-11_n3-n28-n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

8.1.5	MSD

As mentioned in 8.1.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.2	DC_1-8-42_n3-n28-n77

8.2.1	Operating bands for DC

Table 8.2.1-1: EN-DC band combination (six bands)

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_1-8-42_n3-n28-n77 | CA_1-8-42 | CA_n3-n28-n77 |

8.2.2	Inter-band DC Configurations

Table 8.2.2-1: Inter-band EN-DC configurations (six bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-8A-42A_n3A-n28A-n77A | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |
| DC_1A-8A-42A_n3A-n28A-n77(2A) | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |
| DC_1A-8A-42C_n3A-n28A-n77A | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_1A-8A-42C_n3A-n28A-n77(2A) | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |

8.2.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR3DL/1UL are already covered by the fallback configurations, DC_1-8-42_n3-n28, DC_1-8-42_n3-n77, DC_1-8-42_n28-n77, DC_1-8_n3-n28-n77, DC_1-42_n3_n28-n77 and DC_8-42_n3-n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.2.4	∆TIB and ∆RIB values

For DC_1-8-42_n3-n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-42_n3-n28-n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n3 | 0.8 |
|  | n28 | 0.8 |
|  | n77 | 0.8 |

Table 8.2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-42_n3-n28-n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n28 | 0.5 |
|  | n77 | 0.5 |

8.2.5	MSD

As mentioned in 8.2.3, there is no need to specify additional MSD requirement for this UL DC configuration.

Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Subject/Comment | Old | New |
| 2020-08 | RAN4 #96-e | R4-2010661 |  |  | Skeleton | N/A | 0.0.1 |
| 2020-08 | RAN4 #96-e | R4-2010662 |  |  | Some Editor’s note are corrected |  | 0.1.0 |
| 2020-11 | RAN4 #97-e | R4-2015067 |  |  | Including the following approved TPs. - R4-2016765 TP for TR 37.716-11-31: EN-DC_1_n3-n28-n77 - R4-2016766 TP for TR 37.717-11-31: EN-DC_8_n3-n28-n77 - R4-2015050 TP for 37.717-11-31_ DC_8A_n40A-n41A-n79A - R4-2015802 TP for TR 37.717-11-31: support of DC_3_n1-n78-n257, DC_3-3_n1-n78-n257, DC_7_n1-n78-n257, DC_7-7_n1-n78-n257 - R4-2015806 TP for TR 37.717-11-31: support of DC_3-7_n1-n78-n257, DC_3-3-7_n1-n78-n257, DC_3-7-7_n1-n78-n257, DC_3-3-7-7_n1-n78-n257 |  | 0.2.0 |
| 2021-02 | RAN4 #98-e | R4-2102232 |  |  | Including the following approved TPs. - R4-2100669 TP for TR 37.717-11-31: EN-DC_11_n3-n28-n77 SoftBank Corp. - R4-2100670 TP for TR 37.717-11-31: EN-DC_42_n3-n28-n77 SoftBank Corp. - R4-2100681 TP for TR 37.717-11-31: EN-DC_1-8_n3-n28-n77 SoftBank Corp. - R4-2100682 TP for TR 37.717-11-31: EN-DC_1-11_n3-n28-n77 SoftBank Corp. - R4-2100685 TP for TR 37.717-11-31: EN-DC_1-42_n3-n28-n77 SoftBank Corp. - R4-2100686 TP for TR 37.717-11-31: EN-DC_8-11_n3-n28-n77 SoftBank Corp. - R4-2100687 TP for TR 37.717-11-31: EN-DC_8-42_n3-n28-n77 SoftBank Corp. - R4-2100690 TP for TR 37.717-11-31: EN-DC_1-8-11_n3-n28-n77 SoftBank Corp. - R4-2100691 TP for TR 37.717-11-31: EN-DC_1-8-42_n3-n28-n77 SoftBank Corp. - R4-2101019 TP for DC_1A_n28A-n77A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101020 TP for DC_1A_n28A-n78A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101021 TP for DC_3A_n1A-n77A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101022 TP for DC_3A_n1A-n78A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101023 TP for DC_3A_n28A-n77A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101024 TP for DC_3A_n28A-n78A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101025 TP for DC_19A_n1A-n77A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101026 TP for DC_19A_n1A-n78A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101027 TP for DC_21A_n1A-n77A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101028 TP for DC_21A_n1A-n78A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101029 TP for DC_21A_n28A-n77A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101030 TP for DC_21A_n28A-n78A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101031 TP for DC_1A-3A_n28A-n77A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101032 TP for DC_1A-3A_n28A-n78A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101033 TP for DC_1A-21A_n28A-n77A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101034 TP for DC_1A-21A_n28A-n78A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101035 TP for DC_3A-21A_n1A-n77A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101036 TP for DC_3A-21A_n1A-n78A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101037 TP for DC_3A-21A_n28A-n77A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101038 TP for DC_3A-21A_n28A-n78A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101039 TP for DC_19A-42A_n1A-n77A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101040 TP for DC_19A-42A_n1A-n78A-n79A for TR37.717-11-31 NTT DOCOMO, INC. - R4-2101186 TP for DC_42A_n1A-n77A-n79A for TR37.717-11-31 NTT DOCOMO INC. - R4-2101187 TP for DC_42A_n1A-n78A-n79A for TR37.717-11-31 NTT DOCOMO INC. R4-2102219 TP for 37.717-11-31_ DC_8A_n39A-n40A-n41A ZTE Corporation |  | 0.3.0 |
| 2021-05 | RAN4 #99-e | R4-2110473 |  |  | Including the following approved TPs. - R4-2110456 TP for 37.717-11-31_DC_8A_n39A-n40A-n79A,ZTE Corporation |  | 0.4.0 |
| 2021-08 | RAN4 #100-e | R4-2112952 |  |  | Including the following approved TPs. - R4-2112927, TP for 37.717-11-31_DC_3A_n41A-n79A-n258A,ZTE Corporation |  | 0.5.0 |
| 2021-11 | RAN4 #101-e | R4-2118217 |  |  | Including the following approved TPs. - R4-2117650 TP for TR 37.717-11-31: EN-DC_1_n3-n28-n79 - R4-2117651 TP for TR 37.717-11-31: EN-DC_1_n3-n77-n79 - R4-2117656 TP for TR 37.717-11-31: EN-DC_8_n3-n77-n79 |  | 0.6.0 |
| 2022-03 | RAN4 #102-e | R4-2204777 |  |  | Including the following approved TPs. - R4-2200187 Draft CR for 38.101-3: the support of n77(2A) in DC_8A_n3A-n77-n79A - R4-2200224 TP for TR 37.716-11-31: EN-DC_1-8_n3-n77-n79 - R4-2200229 TP for TR 37.717-11-31: EN-DC_8_n3-n28-n79 - R4-2200230 TP for TR 37.717-11-31: EN-DC_8_n28-n77-n79 - R4-2201438 TP for TR 37.717-11-31: support of DC_3_n1-n8-n78, DC_3-3_n1-n8-n78, DC_7_n1-n8-n78, DC_7-7_n1-n8-n78 - R4-2201439 TP for TR 37.717-11-31: support of DC_3-7_n1-n8-n78, DC_3-3-7_n1-n8-n78, DC_3-7-7_n1-n8-n78, DC_3-3-7-7_n1-n8-n78 - R4-2204064 draft CR for DC_8_n1-n78-n257, DC_3-8_n1-n78-n257, DC_3-3-8_n1-n78-n257, DC_7-8_n1-n78-n257, DC_7-7-8_n1-n78-n257, DC_3-7-8_n1-n78-n257, DC_3-3-7-8_n1-n78-n257, DC_3-7-7-8_n1-n78-n257, DC_3-3-7-7-8_n1-n78-n257 - R4-2204067 TP for TR 37.717-11-31: update support of DC_3_n1-n8-n78, DC_3-3_n1-n8-n78, DC_7_n1-n8-n78, DC_7-7_n1-n8-n78 - R4-2204068 TP for TR 37.717-11-31: update support of DC_3-7_n1-n8-n78, DC_3-3-7_n1-n8-n78, DC_3-7-7_n1-n8-n78, DC_3-3-7-7_n1-n8-n78 - R4-2204109 TP for TR 37.717-11-31: EN-DC_1-8_n3-n28-n79 - R4-2204111 TP for TR 37.717-11-31: EN-DC_1-8_n28-n77-n79 - R4-2204117 TP for TR 37.717-11-31: EN-DC_8_n1-n3-n77 - R4-2204133 TP for TR 37.717-11-31: EN-DC_11_n3-n77-n79 - R4-2204137 Draft CR for 38.101-3: support of DL n77(2A) in Inter-band EN-DC of DC_1A_n3A-n77-n79A |  | 0.7.0 |