3GPP TSG-RAN WG4 Meeting#102-e      	R4-2204786

E-meeting, 21 Feb – 3 Mar, 2022

Title:	TP to 37.828: FWA MPR

Source:	Nokia, Nokia Shanghai Bell

Agenda item:	5.29.3.2

Document for:	Approval

# 1	Introduction

This contribution discusses PC1 FWA MPR requirement.

# 2	Discussion

## 2.1	Background

Currently NR PC1 MPR is valid only for n14, reason for this is that when these agreements were done there was urgency to have requirements at least for n14 and due to limited time as a compromise all other bands were out scoped.

What this means for MPR is that PC3 MPR has been seen adequate for PC1 for 5 and 10 MHz channel bandwidths. As MPR is to meet general emission requirements such as SEM and ACLR if it works for n14 it works for all bands for 5 and 10 MHz. Actually, n14 has more stringent SEM compared to general SEM for first 5 MHz thus from SEM point of view MPR must be ok for all bands.

Table 1: n14 SEM

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 |  |
| ± 0 – 0.1 | -15 | -18 | -20 | 30 kHz |
| ± 0.1 – 1 | -13 | -13 | -13 | 100 kHz |
| ± 1 – 6 | -13 | -13 | -13 | 1 MHz |
| ± 6 – 10 | -25 |  |  |  |
| ± 10 – 15 |  | -25 |  |  |
| ± 15 – 20 |  |  | -25 |  |

Table 2: General SEM

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5 | 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 | -13 |  | 1 % of channel BW |
| ± 0-1 |  |  | -24 | 30 kHz |
| ± 1-5 | -10 | -10 |  | 1 MHz |
| ± 5-6 | -13 |  |  |  |
| ± 6-10 | -25 |  |  |  |
| ± 5-BWChannel |  | -13 |  |  |
| ± BWChannel-(BWChannel+5) |  | -25 |  |  |

## 2.2	Simulations

We made MPR simulation campaign for PC1 PA against PC1 general emission requirements SEM in table 2 and ACLR in Table 3 and spurious emissions Table 4.

Table 3: NR ACLR

|  | Power class 11 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB1 | 31 dB | 31 dB | 30 dB |
| NOTE 1: Applicable for power class 1 UE operating in Band n14. |  |  |  |  |

Table 4: Spurious emission requirements

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz ≤ f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz ≤ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz ≤ f < 12.75 GHz | -30 dBm | 1 MHz | 4 |
|  | -25 dBm | 1 MHz | 3 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm | 1 MHz | 2 |

PA was calibrated so that it just meets SEM, ACLR and Spurious emissions. MPR was studied for 5, 10, 20, 40 and 100 MHz channel bandwidths. This selection of CH BWs was justified by looking what CHBWs are most challenging in terms of SU, table 5. From table 5 we can see that 40 MHz is most challenging for 15 kHz and 100 MHz for 30 and 60 kHz.

Table 5: SU comparison.

| SCS (kHz) | 5000 | 10000 | 15000 | 20000 | 25000 | 30000 | 35000 | 40000 | 45000 | 50000 | 60000 | 70000 | 80000 | 90000 | 100000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 552.5 | 712.5 | 692.5 |  |  |  |  |  |
| GB size versus CH BW / % | 4.85 | 3.125 | 2.55 | 2.2625 | 2.09 | 1.975 | 1.635714 | 1.38125 | 1.583333 | 1.385 |  |  |  |  |  |
| 30 | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 905 | 1065 | 1045 | 825 | 965 | 925 | 885 | 845 |
| GB size versus CH BW / % | 10.1 | 6.65 | 4.3 | 4.025 | 3.14 | 3.15 | 2.642857 | 2.2625 | 2.366667 | 2.09 | 1.375 | 1.378571 | 1.15625 | 0.983333 | 0.845 |
| 60 |  | 1010 | 990 | 1330 | 1310 | 1290 | 1630 | 1610 | 1590 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |
| GB size versus CH BW / % |  | 10.1 | 6.6 | 6.65 | 5.24 | 4.3 | 4.657143 | 4.025 | 3.533333 | 3.14 | 2.55 | 2.128571 | 1.8125 | 1.566667 | 1.37 |

### 2.2.1	Results

Can be found from annex. Plots for MPR and gating factors are provided.

## 2.3	MPR

Based on the results presented in annex we propose that NR PC3 MPR is extended from being applicable to n14 only being applicable to all bands. This would be in line with LTE which has same MPR for all power classes, see table 6.

Table 6: LTE Maximum Power Reduction (MPR) for Power Class 1, 2 and 3

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK | > 5 | > 4 | > 8 | > 12 | > 16 | > 18 | ≤ 1 |
| 16 QAM | ≤ 5 | ≤ 4 | ≤ 8 | ≤ 12 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16 QAM | > 5 | > 4 | > 8 | > 12 | > 16 | > 18 | ≤ 2 |
| 64 QAM | ≤ 5 | ≤ 4 | ≤ 8 | ≤ 12 | ≤ 16 | ≤ 18 | ≤ 2 |
| 64 QAM | > 5 | > 4 | > 8 | > 12 | > 16 | > 18 | ≤ 3 |
| 256 QAM | ≥ 1 |  |  |  |  |  | ≤ 5 |

Proposal: NR PC3 MPR is valid also for NR PC1 operation for all NR bands

# 3	Conclusion

In this contribution we propose

Proposal: NR PC3 MPR is valid also for NR PC1 operation for all NR bands

# 4 	References

[1] R4-2200704, PC1 FWA MPR, Nokia, RAN4#101bis

# Annex: MPR Results

*************************************** Start of TP ************************************

#### 6.2.2.1	MPR studies general

Currently NR PC1 MPR is valid only for n14, reason for this is that when these agreements were done there was urgency to have requirements at least for n14 and due to limited time as a compromise all other bands were out scoped.

What this means for MPR is that PC3 MPR has been seen adequate for PC1 UE for 5 and 10 MHz channel bandwidths. As MPR is to meet general emission requirements such as SEM and ACLR if it works for n14 it works for all bands for 5 and 10 MHz. Actually, n14 has more stringent SEM compared to general SEM for first 5 MHz thus from SEM point of view MPR must be ok for all bands.

Table 6.2.2.1-1: n14 SEM

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 |  |
| ± 0 – 0.1 | -15 | -18 | -20 | 30 kHz |
| ± 0.1 – 1 | -13 | -13 | -13 | 100 kHz |
| ± 1 – 6 | -13 | -13 | -13 | 1 MHz |
| ± 6 – 10 | -25 |  |  |  |
| ± 10 – 15 |  | -25 |  |  |
| ± 15 – 20 |  |  | -25 |  |

Table 6.2.2.1-2: General SEM

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5 | 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 | -13 |  | 1 % of channel BW |
| ± 0-1 |  |  | -24 | 30 kHz |
| ± 1-5 | -10 | -10 |  | 1 MHz |
| ± 5-6 | -13 |  |  |  |
| ± 6-10 | -25 |  |  |  |
| ± 5-BWChannel |  | -13 |  |  |
| ± BWChannel-(BWChannel+5) |  | -25 |  |  |

##### 6.2.2.1.1	Simulations

MPR simulation campaign for PC1 PA against PC1 general emission requirements SEM in Table 6.2.2.1-2 and ACLR in Table 6.2.2.1-3 and spurious emissions Table 6.2.2.1-4 were done.

Table 6.2.2.1-3: NR ACLR

|  | Power class 11 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB1 | 31 dB | 31 dB | 30 dB |
| NOTE 1: Applicable for power class 1 UE operating in Band n14. |  |  |  |  |

Table 6.2.2.1-4: Spurious emission requirements

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz ≤ f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz ≤ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz ≤ f < 12.75 GHz | -30 dBm | 1 MHz | 4 |
|  | -25 dBm | 1 MHz | 3 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm | 1 MHz | 2 |

PA was calibrated so that it just meets SEM, ACLR and Spurious emissions. MPR was studied for 5, 10, 20, 40 and 100 MHz channel bandwidths. This selection of CH BWs was justified by looking what channel bandwidths are most challenging in terms of spectrum utilization, Table 6.2.2.1-5. From table 5 we can see that 40 MHz is most challenging for 15 kHz scs and 100 MHz for 30 and 60 kHz scs.

Table 6.2.2.1-5: SU comparison.

| SCS (kHz) | 5000 | 10000 | 15000 | 20000 | 25000 | 30000 | 35000 | 40000 | 45000 | 50000 | 60000 | 70000 | 80000 | 90000 | 100000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 552.5 | 712.5 | 692.5 |  |  |  |  |  |
| GB size versus CH BW / % | 4.85 | 3.125 | 2.55 | 2.2625 | 2.09 | 1.975 | 1.635714 | 1.38125 | 1.583333 | 1.385 |  |  |  |  |  |
| 30 | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 905 | 1065 | 1045 | 825 | 965 | 925 | 885 | 845 |
| GB size versus CH BW / % | 10.1 | 6.65 | 4.3 | 4.025 | 3.14 | 3.15 | 2.642857 | 2.2625 | 2.366667 | 2.09 | 1.375 | 1.378571 | 1.15625 | 0.983333 | 0.845 |
| 60 |  | 1010 | 990 | 1330 | 1310 | 1290 | 1630 | 1610 | 1590 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |
| GB size versus CH BW / % |  | 10.1 | 6.6 | 6.65 | 5.24 | 4.3 | 4.657143 | 4.025 | 3.533333 | 3.14 | 2.55 | 2.128571 | 1.8125 | 1.566667 | 1.37 |

##### 6.2.2.1.2	Results

Some back off results are provided, complete set is found in R4-2200704.

###### 6.2.2.1.2.1	MPR, 40MHz, 15kHz

Next we present gating factors for 40 MHz bandwidths.

###### 6.2.2.1.2.2	MPR, 100MHz, 30kHz

Next we present gating factors for 100 MHz bandwidths.

#### 6.2.2.1.3	MPR

Based on the results presented in annex we propose that NR PC3 MPR is extended from being applicable to n14 only being applicable to all bands. This would be in line with LTE which has same MPR for all power classes, see table 6.

Table 6: LTE Maximum Power Reduction (MPR) for Power Class 1, 2 and 3

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK | > 5 | > 4 | > 8 | > 12 | > 16 | > 18 | ≤ 1 |
| 16 QAM | ≤ 5 | ≤ 4 | ≤ 8 | ≤ 12 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16 QAM | > 5 | > 4 | > 8 | > 12 | > 16 | > 18 | ≤ 2 |
| 64 QAM | ≤ 5 | ≤ 4 | ≤ 8 | ≤ 12 | ≤ 16 | ≤ 18 | ≤ 2 |
| 64 QAM | > 5 | > 4 | > 8 | > 12 | > 16 | > 18 | ≤ 3 |
| 256 QAM | ≥ 1 |  |  |  |  |  | ≤ 5 |

Proposal: NR PC3 MPR is valid also for NR PC1 operation for all NR bands

*************************************** End of TP ************************************