# 3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204857

# Electronic Meeting, February 21 – March 3, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on maintenance of measurement requirements for NR-U R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Note: Resubmission of previously endorsed Draft CR R4-2115276 missing from BIG CR# 2209 1.The SSB symbols in scheduling restrictions shall considered the merged set configured by different MOs. The defination has been corrected in the requirements in other parts, which should be also fixed in NR-U section. 2. According to the defination of rmtc-SubframeOffset, when it is not configured for inter-frequency measurements, UE shall choose a random value to determine the RMTC window. According to RAN1 LS to RAN2 [R1-2106264], the generation method for the random offset value is up to UE’s implementation. Thus, for inter-frequency RSSI/CO measurement, the randomly generated window may not be covered by the measurement gap. rmtc-SubframeOffset Indicates the RSSI measurement timing configuration (RMTC) subframe offset for this frequency (see TS 38.215 [9], clause 5.1.21). For inter-frequency measurements, this field is optional present and if it is not configured, the UE chooses a random value as rmtc-SubframeOffset for measDurationSymbols which shall be selected to be between 0 and the configured rmtc-Periodicity with equal probability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Align the defination of SSB symbols with the existing requirements. - Clarify that there is no requirements for inter-frequency RSSI/CO measurement when rmtc-SubframeOffset is not configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2A.5.3, 9.3A.8 and 9.3A.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <End of Change 1>

#### 9.2A.5.3	Scheduling availability of UE during intra-frequency measurements

UE shall be capable of measuring without measurement gaps when the SSB is completely contained in the active bandwidth part of the UE. When any of the conditions in the following clauses is met, there are restrictions on the scheduling availability; otherwise, there is no scheduling restriction. Note that the SSB symbols indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged [2], if it is configured; otherwise, all L SSB symbols within SMTC window duration defined in clause 4.1 of TS 38.213 [3] are included.

### <End of Change 1>

### <Start of Change 2>

### 9.3A.8	Inter-frequency RSSI measurements

An RSSI measurement is defined as an inter-frequency measurement provided that the RSSI measurement bandwidth is not contained within the current carrier bandwidth of the UE.

The UE physical layer shall be capable of performing the RSSI measurements, defined in TS 38.215 [4] on one or more inter-frequency carriers operating with CCA, TS 37.213 [33], if the carrier(s) are indicated by higher layers [2], and report the RSSI measurements to higher layers. The UE physical layer shall provide to higher layers a single RSSI sample for each OFDM symbol within each configured RSSI measurement duration [2] occurring with a configured RSSI measurement timing configuration periodicity [2], rmtc-Periodicity. The requirements apply if rmtc-SubframeOffset [2] is configured.

Table 9.3A.8-1: Measurement period for inter-frequency RSSI measurements with gaps

| Condition NOTE1,2,3,4 | T RSSI_measurement_period_inter_cca |
| --- | --- |
| No DRX | max(reportInterval, max(rmtc-Periodicity, MGRP) x CSSFinter) |
| DRX | max(reportInterval, max(rmtc-Periodicity, MGRP,DRX cycle) x CSSFinter) |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: CSSFinter is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps. |  |

If the UE requires measurement gaps to perform inter-frequency measurements, a single measurement gap pattern is used for all concurrent inter-frequency measurements, including inter-frequency RSSI measurements. The RSSI measurement duration and the measurement gap should be aligned, and the following additional condition should be fulfilled:

-	Entire RSSI measurement duration should be contained in the measurement gap.

The RSSI measurement performed and reported according to this clause shall meet the RSSI measurement accuracy requirement in Clause 10.1.34.2. The reported RSSI measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in Clause 10.1.34.3.

### 9.3A.9	Inter-frequency channel occupancy measurements

The UE shall be capable of estimating the channel occupancy on one or more carrier frequencies indicated by higher layers [2], based on RSSI samples provided by the physical layer. The requirements apply if rmtc-SubframeOffset [2] is configured.

Table 9.3A.9-1: Measurement period for inter-frequency Channel Occupancy measurements with gaps

| Condition NOTE1,2,3,4 | T CO_measurement_period_inter_cca |
| --- | --- |
| No DRX | max(reportInterval, max(rmtc-Periodicity, MGRP) x CSSFinter) |
| DRX | max(reportInterval, max(rmtc-Periodicity, MGRP,DRX cycle) x CSSFinter) |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: CSSFinter is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps. |  |

If the UE requires measurement gaps to perform inter-frequency measurements, a single measurement gap pattern is used for all concurrent inter-frequency measurements, including inter-frequency channel occupancy measurements. The RSSI measurement duration used for channel occupancy measurement and the measurement gap should be aligned, and the following additional condition should be fulfilled:

-	Entire RSSI measurement duration should be contained in the measurement gap.

The channel occupancy measurement performed and reported according to this clause shall meet the channel occupancy measurement accuracy requirements in Clause 10.1.35.2. The reported channel occupancy measurement values contained in measurement reports shall be based on the measurement reporting range specified in TS 38.331 [2].

### <End of Change 2>