# 3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204873

# Electronic Meeting, February 21- March 3, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on interruption of PUCCH SCell activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Interruption requirements of PUCCH SCell activation should be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the interruption requirements for PUCCH SCell activaiton/deactivation for NR SA, EN-DC and NE-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for HO with PSCell are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2.x, 8.2.1.2.2 and 8.2.3.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

##### 8.2.1.2.x	Interruptions due to PUCCH SCell activation/deactivation

When one SCell in SCG configured with PUCCH is activated or deactivated,

- The UE is allowed an interruption on active serving cell in SCG as defined in clause 8.2.1.2.4.

- The starting time of interruption shall be within the delay as defined in clause 8.x.y.

Editor notes: FFS on whether to define interruptions due to PUCCH SCell RACH has different SCS from spCell data/control channel

### <End of Change 1>

### <Start of Change 2>

##### 8.2.2.2.x	Interruptions due to PUCCH SCell activation/deactivation

When one SCell in MCG configured with PUCCH is activated or deactivated,

- The UE is allowed an interruption on active serving cell in MCG as defined in clause 8.2.2.2.2.

- The starting time of interruption shall be within the delay as defined in clause 8.x.y.

Editor notes: FFS on whether to define interruptions due to PUCCH SCell RACH has different SCS from spCell data/control channel

### <End of Change 2>

### <Start of Change 3>

##### 8.2.3.2.x	Interruptions due to PUCCH SCell activation/deactivation

When one SCell in MCG configured with PUCCH is activated or deactivated,

- The UE is allowed an interruption on active serving cell in MCG as defined in clause 8.2.3.2.4.

- The starting time of interruption shall be within the delay as defined in clause 8.x.y.

Editor notes: FFS on whether to define interruptions due to PUCCH SCell RACH has different SCS from spCell data/control channel

### <End of Change 3>