3GPP TSG-RAN WG4 Meeting#102-e                                R4-2204919

E-meeting,21th Feb – 3rd Mar,2022

Source:	ZTE Corporation

Title:	Test setup for 15 kHz SCS scenario

Agenda Item:	10.12.2.3.3

Document for:	Discussion

- Introduction

- In RAN4#101-bits-e meeting, test setup for 15 kHz SCS scenario was discussed, and a way forward was agreed [1]. Regarding necessity of test setup for 15 kHz SCS scenario for CRS-IM receiver, there are still some issues open, we would like to share our views on the following three issues for further discussion:

- PDSCH loading level

- Interference power level

- Tx antenna and LTE CRS port number

- Discussion

PDSCH loading level

Discussion in last meeting from [1] is summarized below:

- Option 1: Add one of the combinations below that achieves about 1dB performance gain over the reference scheme - Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB - Loading 40 %, INR 1 = 8.79 dB, INR 2 = 2.7 dB - Loading 50 %, INR 1 = 8.36 dB, INR 2 = 1.7 dB - Option 2: Only consider 20% PDSCH loading level - Option 3: Consider 30% interference PDSCH loading for Scenario 1 and 20% interference PDSCH loading for Scenario 2 - Option 4: Also include 80% loading - Option 5: Further check 30% loading level - Option 6: Either 30% or 20%

Based on our simulation results, 20% PDSCH loading level is a good condition that has high performance for LLR weighting. On the other hand, it’s not only focus on 20% loading but also should consider other loadings with relative lower performance gain. So it should add more loadings such as 30%, 40% and 50% with lower performance gain.

Proposal 1: Add more loadings such as 30%, 40% and 50% with lower performance gain.

Interference power level

- Option 1: Add one of the combinations below that achieves about 1dB performance gain over the reference scheme - Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB - Loading 40 %, INR 1 = 8.79 dB, INR 2 = 2.7 dB - Loading 50 %, INR 1 = 8.36 dB, INR 2 = 1.7 dB - Option 2: Only consider INR1 = 10.45 dB and INR2 = 4.6 dB - Option 3: Only consider INR1 = 10.45 dB and INR2 = 4.6 dB in case of 20% interference PDSCH loading - Option 4: 30% PDSCH loading and INR 1 = 9.69 dB, INR 2 = 3.7 dB for Scenario 1 and 20% interference PDSCH loading and INR1 = 10.45 dB and INR2 = 4.6 dB for Scenario 2

The conclusion is same as PDSCH loading level.

Proposal 2: Add more interference power levels with lower performance gain.

Tx antenna and LTE CRS port number

- Option 1: Only cover 4 CRS ports - Option 2: Cover 2 and 4 for CRS ports - Option 2A: Consider 2 CRS ports for Scenario 1 and 4 CRS ports for Scenario 2 - Option 2B: Use different CRS port number in the tests with different INR levels - Option 3: Only cover 2 CRS ports (Nokia)

According the conclusion of last meeting[1], 4 CRS ports for Scenario 2 as baseline assumption. For scenario 1, our simulation was based on 4 CRS ports. It is necessary to verify the performance of 2 CRS ports for scenario 1. If there is no significant performance difference in 2 CRS ports. Only consider 4 ports for Scenario 1.

Proposal 3: Consider 2 CRS ports for Scenario 1 and 4 CRS ports for Scenario 2.

- Conclusion

In this contribution, we have the following proposals for test setup for 15 kHz SCS scenario:

Proposal 1: Add more loadings such as 30%, 40% and 50% with lower performance gain.

Proposal 2: Add more interference power levels with lower performance gain.

Proposal 3: Consider 2 CRS ports for Scenario 1 and 4 CRS ports for Scenario 2.

- Reference

- R4-2203028, “WF on general part and 15kHz NR SCS scenario for CRS-IM receiver”. China Telecom