3GPP TSG-RAN WG4 Meeting #102-e		R4-2204982
Electronic Meeting, 21st Feb. – 03rd Mar., 2022

Agenda item:	10.2.2.2

Source:	OPPO

Title:	On EN-DC combinations

Document for:	Approval

1	Introduction

In the last meeting of RAN4 #101-bis-e, the principle of EN-DC combinations selection for FR1 TRP TRS testing is agreed [1]. Only one combination will be selected for NR band under test.

And the examples of EN-DC combination for some bands are given to clearly identify the test band. However, there are concerns that the examples can not guarantee all test labs choosing the same test configuration because DUT may not support the example EN-DC combination.

This paper presents our views on EN-DC combination selection.

2	Discussion

Examples of EN-DC combination can illustrate clear instructions on EN-DC band combination selection. However, examples can not cover various possibilities, such as DUT not support the example EN-DC combination on the NR band to be tested, no example defined in the specification on the test NR band, and so on. So it is benefit to provide a decision procedure to ultimately determine the EN-DC combination to be tested, when the example EN-DC combination is not supported by DUT.

Observation 1: Examples of EN-DC combination can not cover various possibilities.

Proposal 1: It is proposed to provide a decision procedure to ultimately determine the EN-DC combination to be tested, when the example EN-DC combination is not supported by DUT.

Generally, UE will use its normal antenna(s) and tuning configuration for the operating NR band. The exception case happens when the anchor LTE band is near to the operating NR band in EN-DC mode. In this case, the antenna resonance worked for the NR band may be fine tuned to support both NR and LTE sharing the same antenna. Or, the NR antenna may switch to work on another antenna to leave the previous antenna for LTE band. Based on this understanding, one principle for determining the EN-DC combination can consider the frequency interval between the NR band and the LTE band in EN-DC mode. Following the agreed principle on MSD impact in the last meeting, the similar approach for the impact of antenna switching and tuning is proposed that selecting the EN-DC combination with the largest frequency interval between the NR band to be tested and the LTE band from the UE supported EN-DC combination list to avoid antenna switching and tuning as far as possible.

Proposal 2: It is proposed to select the EN-DC combination with the largest frequency interval between the NR band to be tested and the LTE band from the UE supported EN-DC combination list.

Proposal 3: The proposed decision procedure to determine the EN-DC combination to be tested is illustrated below.

Does DUT support the example EN-DC combination

Collect all the EN-DC combinations that DUT supported on the NR band to be tested

Identify the example EN-DC combination on the NR band to be tested in the specification

Identify the frequency ranges of the anchor LTE bands collected from previous step

Select the LTE band with the largest frequency interval between the NR band to be tested and the LTE band as the anchor band for EN-DC combination

Use the example EN-DC combination as the test configuration

Determine the EN-DC combination to be tested

Yes

No

3	Conclusion

Proposal 1: It is proposed to provide a decision procedure to ultimately determine the EN-DC combination to be tested, when the example EN-DC combination is not supported by DUT.

Proposal 2: It is proposed to select the EN-DC combination with the largest frequency interval between the NR band to be tested and the LTE band from the UE supported EN-DC combination list.

Proposal 3: The proposed decision procedure to determine the EN-DC combination to be tested is illustrated below.

Does DUT support the example EN-DC combination

Collect all the EN-DC combinations that DUT supported on the NR band to be tested

Identify the example EN-DC combination on the NR band to be tested in the specification

Identify the frequency ranges of the anchor LTE bands collected from previous step

Select the LTE band with the largest frequency interval between the NR band to be tested and the LTE band as the anchor band for EN-DC combination

Use the example EN-DC combination as the test configuration

Determine the EN-DC combination to be tested

Yes

No

References

[1] R4-2203076 WF on FR1 TRP TRS