3GPP TSG-RAN4 Meeting #102-e	R4-2205191

# Electronic Meeting, 21st Feb – 3rd March, 2022

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-17 feature list

Agenda Item:	8

Document for:	Approval

Introduction

In this contribution we would like to provide our view on Rel-17 UE feature list.

Discussion

Feature list is proposed for following features.

- NR_RF_FR1_enh

For NR_RF_FR1_enh, several feature items have already been approved in RAN4 101bis-e with FFS left for 16-7. Below table includes proposed update for item 16-7. The prerequisite feature groups are updated for a few feature items. In addition one new item 16-8 is added.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16. NR_RF_FR1_enh | 16-1 | Dynamic Tx switching between 2CC 2Tx-2Tx switching | Indicate the supported switching period for dynamic UL Tx switching between two uplink carriers with two transmit antenna connectors in inter-band UL CA or SUL |  | Yes | N/A | UE does not support 2CC 2Tx-2Tx switching for inter-band UL CA and SUL band combinations. | UE signals supported switching period per pair of UL bands per UL band combination | No need | Applicable only to FR1 | Support mixture of FDD/TDD | Candidate value set: {35us, 140 us, 210us} Detailed information can refer to the LS to RAN2 in R4-2103234 and R4-2107847. | Optional with capability signalling |
| 16. NR_RF_FR1_enh | 16-2 | Dynamic Tx switching between 3CC 1Tx-2Tx switching | Indicate the supported switching period for dynamic UL Tx switching between one band (with one carrier) capable of one transmit antenna connector and one band (with two carriers) capable of two transmit antenna connectors in inter-band UL CA or SUL |  | Yes | N/A | UE does not support Tx switching between 3CC 1Tx-2Tx switching for inter-band UL CA and SUL band combinations. | UE signals supported switching period per pair of UL bands per UL band combination | No need | Applicable only to FR1 | Support mixture of FDD/TDD | Candidate value set: {35us, 140 us, 210us} Detailed information can refer to the LS to RAN2 in R4-2103234 and R4-2107847. | Optional with capability signalling |
| 16. NR_RF_FR1_enh | 16-3 | Dynamic Tx switching between 3CC 2Tx-2Tx switching | Indicate the supported switching period for dynamic UL Tx switching between one band (with one carrier) capable of two transmit antenna connectors and one band (with two carriers) capable of two transmit antenna connectors in inter-band UL CA or SUL |  | Yes | N/A | UE does not support Tx switching between 3CC 2Tx-2Tx switching for inter-band UL CA and SUL band combinations. | UE signals supported switching period per pair of UL bands per UL band combination | No need | Applicable only to FR1 | Support mixture of FDD/TDD | Candidate value set: {35us, 140 us, 210us} Detailed information can refer to the LS to RAN2 in R4-2103234 and R4-2107847. | Optional with capability signalling |
| 16. NR_RF_FR1_enh | 16-4 | Application of DL interruptions due to dynamic UL Tx switching | Capability to indicate that for the band where DL interruption is needed, the RRM interruption requirements defined in RAN4 shall be applied for duplex mode combinations except the combinations - SUL+TDD - TDD+TDD CA with the same UL-DL pattern | 16-1, 16-2, or 16-3 | Yes | N/A | UE not reporting this capability means DL interruption is not required | UE capability is defined as per band per band combination for each band pair supporting UL Tx switching | No need | Applicable only to FR1 | Support mixture of FDD/TDD | The same capability for Rel-16 DL interruption due to Tx switching is reused. Detailed information can refer to the LS to RAN2 in R4-2103234. | Optional with capability signalling |
| 16. NR_RF_FR1_enh | 16-5 | UL-MIMO coherence capability for dynamic Tx switching between 3CC 1Tx-2Tx switching | Capability to indicate whether UL-MIMO coherence is supported when dynamic Tx switching between 3CC (within 2 bands) 1Tx-2Tx switching is conducted. | 16-2 | Yes | N/A | Rel-15 per band capability pusch-TransCoherence is applicable | Per BC | No need | Applicable only to FR1 | Support mixture of FDD/TDD | The Rel-16 UL-MIMO capability for 2CC 1Tx-2Tx switching is reused. Detailed information can refer to the LS to RAN2 in R4-2120039. | Optional with capability signalling |
| 16. NR_RF_FR1_enh | 16-6 | UL-MIMO coherence capability for dynamic Tx switching between 2Tx-2Tx switching | Capability to indicate whether UL-MIMO coherence is supported when dynamic Tx switching between 2CC or 3CC (within 2 bands) 2Tx-2Tx switching is conducted. | 16-1 or 16-3 | Yes | N/A | The per BC UL-MIMO coherence capability for 1Tx-2Tx switching or Rel-15 per band capability pusch-TransCoherence is applicable | Per band per BC | No need | Applicable only to FR1 | Support mixture of FDD/TDD | Detailed information can refer to the LS to RAN2 in R4-2120039. | Optional with capability signalling |
| 16. NR_RF_FR1_enh | 16-7 | [Support RRC configuration to prevent SCell dropping for CA] | UE capability to indicate whether to support the function. NW configure a relative parameter for serving cells which is a UE specific RRC signalling for a set of values based on possible proportion of channel BW or allocated RB resources among the CCs, and the appropriate parameter according to CBW ratio or dynamic RB allocation ratio can be fast activated/deactivated by MAC-CE or DCI for each scheduling. The parameter set includes values of 10log10{5%, 10%, 20%, 30%, 40%, 50%, 60%, 70%, 80%, 90%, 95%, 100%}. |  | Yes | N/A | [UE may drop SCell without enough transmission power according the current power control mechanism for CA] | Per BC | No | No | N/A |  | Optional with capability signalling |
| 16. NR_RF_FR1_enh | 16-8 | PHR reporting for CA | Support PCMAX,CA , and PHRCA reporting for CA |  | Yes | N/A | NW may not get the accurate information for the power head room for CA | Per BC | No | No | N/A |  | Optional with capability signalling |

- NR_RF_FR2_req_enh2

One more feature item is proposed for NR_RF_FR2_req_enh2.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 17. NR_RF_FR2_req_enh2 | 17-4 | Support of Fs_Inter for CBM inter-band CA | Support for frequency separation class for inter-band CA with CBM (Fs_Inter): Same values as intra-band non-contiguous CA (Fs) are supported, frequency separation for other value is not precluded. |  | Yes | N/A | UE does not support frequency separation class for inter-band CA with CBM. | Per BC | No | FR2 only | N/A |  | Optional with capability signalling |

- Power_Limit_CA_DC

For Power_Limin_CA_DC, following feature item is proposed.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22. Power_Limit_CA_DC | 22-1 | NR UL CA/DC | Indicates the power class per-band and the max total output power when operating according to this band combination. |  | Yes | No | UE cannot fully utilize its Tx power capability for a band combination | Per BC | No need | FR1 only | N/A |  | Optional with capability signalling |

- References

[1] R4-2202400	Rel-17 UE feature list, RAN4#101bis-e