3GPP TSG-RAN WG4 Meeting #102e	R4-2205225

Electronic Meeting, 21st Feb - 3rd Mar, 2022

Source:	ZTE Corporation

Title:	ULFPTx requirements for fallback and TxD

Agenda Item:	10.7.3.2

Document for:	Discussion

# Introduction

In RAN4#101b-e, there was an open issue left on ULFPTx requirements for fallback and TxD [1]:

- How ULFPTx capable UE’s fallback DCI requirement should be defined? - Alt1:… - Alt2:… - Alt3:… - Other alternatives, or combined one if identified.

The key point is to check either single Tx or Dual Tx requirements should be applied for each of the possible combination of UE capabilities in terms of TxD and ULFPTx.

# Discussion

Since UE reports its capabilities on TxD and ULFPTx with different modes independently, there could be different combinations. Of all three alternatives, these combinations are not aligned:

(1) No TxD + ULFPTx mode 1

(2) TxD + ULFPTx mode 2 (both Mechanisms)

(3) TxD + ULFPTx full power

For (1), if No TxD but ULFPTx mode 1 is reported, both PAs are active, then dual Tx requirements should apply.

For (2), since RAN4 agreed that UE reports TxD support for only symmetric PAs configurations (i.e., 23+23, 26+26), then this case is not supposed to be valid.

For (3), if PA configuration is 26+26, PC2 can be implemented either ULFPTx full mode (1Tx), or TxD (2Tx, 23+23) , therefore different requirements should apply. And a UE is not supposed to be in TxD and ULFPTx full mode at the same time.

The overall requirements are illustrated in the following table:

| Single antenna-port Requirements applicability | ULFPTx modes |  |  |  | No ULFPTx |
| --- | --- | --- | --- | --- | --- |
|  | ULFPTx mode 1 | ULFP Tx Mode-2 Mechanism 1-SRS virtualization | ULFP Tx Mode-2 Mechanism 2: TPMI | ULFPTx full mode |  |
| txDiversity-r16 | Dual Tx | Not applicable | Not applicable | If UE is running in TxD, Dual Tx applies If UE is running in ULFPTx full mode, Single Tx applies | Dual Tx |
| No TxD indication | Dual Tx | Single Tx | Single Tx | Single Tx | Single Tx |

Proposal: RAN4 to specify ULFPTx requirements for TxD as above table (None of the three listed alternatives).

# Conclusion

In this paper, we have the following proposal for ULFPTx requirements for TxD:

| Single antenna-port Requirements applicability | ULFPTx modes |  |  |  | No ULFPTx |
| --- | --- | --- | --- | --- | --- |
|  | ULFPTx mode 1 | ULFP Tx Mode-2 Mechanism 1-SRS virtualization | ULFP Tx Mode-2 Mechanism 2: TPMI | ULFPTx full mode |  |
| txDiversity-r16 | Dual Tx | Not applicable | Not applicable | If UE is running in TxD, Dual Tx applies If UE is running in ULFPTx full mode, Single Tx applies | Dual Tx |
| No TxD indication | Dual Tx | Single Tx | Single Tx | Single Tx | Single Tx |

Proposal: RAN4 to specify ULFPTx requirements for TxD as above table (None of the three listed alternatives).

# Reference

R4-2202354 ULFPTx requirements for fallback and SRS antenna switching, Samsung