3GPP TSG-RAN WG4 Meeting # 102-e	R4-2205227

# Electronic Meeting, February 21 – March 03, 2022

Source:	LG Electronics

Title:	Discussion on Tx RF requirements in FR2-2

Agenda item:	10.16.3.1

Document for:	Discussion

# Introduction

In this paper, we provide our view on Tx RF requirements in FR2-2.

# Discussion

In last RAN4 meeting, some WFs were made for NR_ext_to_71GHz. We would like to discuss UE transmit power for vehicular UE based on WF [1].

The related agreements in WF are captured.

1 Handheld UE number of antenna element assumption - Note: This is an assumption used to derive requirements, and not a requirement. - Agreement from GTW: Use 8 antenna elements as the assumption for defining minimum requirements - Check if there is any performance problem. If performance problem is identified and agreed, revisit the agreement. - The simulation assumptions for co-existence study can be used as baseline in the second round or future meetings for the performance analysis. 2 Handheld UE min peak EIRP - Agreement: Average handheld UE min peak EIRP in the range of [ 13.2 to 14.1 ] dBm which is the range of values arrived at by computing and average in dB domain and average in power domain. 3 Handheld UE REFSENS - Agreement: Min SENS for n263 400 MHz, based on averaging the proposals in the table is [ -73.0 dBm ] 7 FWA UE number of antenna panels for spatial coverage - Agreement: One panel as an assumption for FWA specification development. All companies that commented agreed with 1 panel. 10 Vehicular UE - Agreed WF: Output power, reference sensitivity and spherical coverage for vehicular UE was discussed but due to limited amount of inputs no agreements could be made. Companies are requested to provide their inputs in the next meeting. Requirements for Vehicular UE should be defined using similar approach as is done for Handheld UE. Based on the agreement in RAN4#101e Vehicular array size is equal to or larger than the handheld.

## FR2-1 background

Table 2.1.1 and Table 2.1.2 summarize the UE minimum peak EIRP and the spherical coverage’s EIRP which correspond to  power class 1, 2, 3, 4 and 5 in FR2-1.

Table 2.1.1: UE minimum peak EIRP for Power Class 1,2,3,4 and 5 in FR2-1

| Operating band | Frequency | Min peak EIRP (dBm) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PC1 | PC2 | PC3 | PC4 | PC5 |
| n257 | 28GHz(26500MHz – 29500MHz) | 40.0 | 29 | 22.4 | 34 | 30 |
| n258 | 24GHz(24250MHz – 27500MHz) | 40.0 | 29 | 22.4 | 34 | 30.4 |
| n259 | 39GHz(39500MHz – 43500MHz) |  |  | 18.7 |  | 27.7 |
| n260 | 39GHz(37000MHz – 40000MHz) | 38.0 |  | 20.6 | 31 |  |
| n261 | 28GHz(27500MHz – 28350MHz) | 40.0 | 29 | 22.4 | 34 |  |
| n262 | 47GHz(47200MHz – 48200MHz) | 34.2 | 22.9 | 16.0 | 28.3 |  |

Table 2.1.2: UE spherical coverage for Power Class 1,2,3,4 and 5 in FR2-1

| Operating band | Min EIRP at X %-tile CDF (dBm) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | PC1 | PC2 | PC3 | PC4 | PC5 |
|  | X = 85 | X = 60 | X = 50 | X = 20 | X = 85 |
| n257 | 32.0 | 18.0 | 11.5 | 25 | 22 |
| n258 | 32.0 | 18.0 | 11.5 | 25 | 22.4 |
| n259 |  |  | 5.8 |  | 19.7 |
| n260 | 30.0 |  | 8 | 19 |  |
| n261 | 32.0 | 18.0 | 11.5 | 25 |  |
| n262 | 26.0 | 11.0 | 2.9 | 16.2 |  |

Table 2.1.3 summarize the difference between the minimum peak EIRP and spherical coverage’s EIRP which correspond to each X%-tile for power class.

Table 2.1.3: Difference between the minimum peak EIRP and EIRP at X%-tile CDF for Power Class 1,2,3,4 and 5 in FR2-1

| Operating band | Difference between minimum peak EIRP and EIRP at X%-tile CDF (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | PC1 | PC2 | PC3 | PC4 | PC5 |
|  | X = 85 | X = 60 | X = 50 | X = 20 | X = 85 |
| n257 | 8.0 | 11.0 | 10.9 | 9.0 | 8.0 |
| n258 | 8.0 | 11.0 | 10.9 | 9.0 | 8.0 |
| n259 |  |  | 12.9 |  | 8.0 |
| n260 | 8.0 |  | 12.9 | 12.0 |  |
| n261 | 8.0 | 11.0 | 10.9 | 9.0 |  |
| n262 | 8.2 | 11.9 | 13.1 | 12.1 |  |

## Vehicular UE

For vehicular UE, based on the agreement in WF, 16 antenna elements (16Tx) is considered.

For minimum peak EIRP the analysis in Table 2.2.1 shows that level of  22.7 dBm can be achieved.

Table 2.2.1: UE minimum peak EIRP for vehicular UE in FR2-2

| Parameter | Unit | Value |
| --- | --- | --- |
| Antenna element number per polarization |  | 16 |
| Avg. element gain per polarization | dBi | 4.6 |
| Antenna roll-off loss vs frequency | dB | -2.5 |
| Polarization gain | dB | 2.5 |
| Array gain | dB | 12.04 |
| Total realized antenna gain | dBi | 16.6 |
| Pout(per element) | dBm | 2 |
| Array gain | dB | 12.04 |
| Total conducted Power (per polar) | dBm | 14.04 |
| Total IM Loss | dB | -8 |
| Minimum Peak EIRP | dBm | 22.7 |

Proposal 1: For vehicular UE, minimum peak EIRP of  22.7 dBm is proposed (based on 16 antenna elements)

Table 2.2.2: UE minimum peak EIRP for Power Class 2

| Operating band | Frequency | Min peak EIRP (dBm) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PC2 |  |  |  |
| n257 | 28GHz(26500MHz – 29500MHz) |  | 29 |  |  |  |
| n258 | 24GHz(24250MHz – 27500MHz) |  | 29 |  |  |  |
| n259 | 39GHz(39500MHz – 43500MHz) |  |  |  |  |  |
| n260 | 39GHz(37000MHz – 40000MHz) |  |  |  |  |  |
| n261 | 28GHz(27500MHz – 28350MHz) |  | 29 |  |  |  |
| n262 | 47GHz(47200MHz – 48200MHz) |  | 22.9 |  |  |  |
| n263 | 60GHz(5700MHz – 71000MHz) |  | 22.7 |  |  |  |

For spherical coverage, 16Tx (2x8) is analysed, which is based on measurment with 1 panel. Figure 2.2.1 shows the CDF of spherical coverage based on hemisphere.

Figure 2.2.1: Spherical Coverage of 1 panel based on hemisphere for vehicular UE with 16 antenna elements (2x8) in FR2-2

In general, 60%-tile CDF based on full sphere with 1 panel is almost equal to 20%-tile CDF based on hemisphere with 1 panel. And, 60%-tile CDF based on full sphere with back-to-back 2 panels is almost equal to 60%-tile CDF based on hemisphere with 1 panel.

Based on analysis above and the agreement to use one panel for defining the spherical coverage requirements for FWA UE following proposal is made.

Proposal 2: For vehicular UE, EIRP at 60%-tile CDF is 15.1 dB lower than minimum peak EIRP (based on 16 antenna elements).

Table 2.2.3: Difference between the minimum peak EIRP and EIRP at X%-tile CDF for Power Class 2

| Operating band | Difference between minimum peak EIRP and EIRP at X%-tile CDF (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | PC2 |  |  |  |
|  |  | X = 60 |  |  |  |
| n257 |  | 11.0 |  |  |  |
| n258 |  | 11.0 |  |  |  |
| n259 |  |  |  |  |  |
| n260 |  |  |  |  |  |
| n261 |  | 11.0 |  |  |  |
| n262 |  | 11.9 |  |  |  |
| n263 |  | 15.1 |  |  |  |

# Conclusion

In this contribution, we provided our views on MOP for 60GHz vehicular UE.

Proposal 1: For vehicular UE, minimum peak EIRP of  22.7 dBm is proposed (based on 16 antenna elements)

Proposal 2: For vehicular UE, EIRP at 60%-tile CDF is 15.1 dB lower than minimum peak EIRP (based on 16 antenna elements).

Draft CR that captures the proposal can be found in R4-2205210 [2].

# Reference

[1] R4-2202366, “	WF on 60GHz UE RF”, Qualcomm

[2] R4-2205210, “draft CR on vehicular UE Tx RF requirements  in FR2-2”, LG Electronics