3GPP TSG-RAN4 Meeting #102-e	R4-2205371

# Electronic Meeting, 21 February – 3 March, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on collision handling for concurrent MGs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-Core |  |  |  |  |  | Date: |  |  | 2022-01-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR is based on endorsed Big CR R4-2202753 from RAN#101-bis-e. - The concept of “dropped MG occasion” for collision handling is used in other requirements, e.g. those with CSSF and Kp, but it has not beenn defined - The requirements related to data Tx/Rx in case of collision are in [] and there are editor’s note. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the collision handling for concurrent MGs: - Define concept of “dropped MG occasion” - Update requirements for data Tx/Rx and remove [] and editor’s note. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for collision handling are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2B.3, 9.1.2B.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.1.2B.3	Collision between concurrent measurement gaps

When UE is configured with concurrent measurement gap patterns and all of them are per-UE measurement gap patterns, two measurement gap occasions are considered colliding if at least one of the following conditions is met:

-	the occasions are fully or partially overlapping in time domain, or

-	the minimal distance between the two occasions is equal to or smaller than X.

When UE is configured with concurrent measurement gap patterns and all of them are per-FR measurement gap patterns, two measurement gap occasions of different measurement gaps for the same FR are considered colliding if at least one of the following conditions is met:

-	the occasions are fully or partially overlapping in time domain, or

-	the minimal distance between the two occasions is equal to or smaller than X.

When UE is configured with concurrent measurement gap patterns and one of them is per-UE measurement gap pattern and the other(s) are per-FR measurement gap patterns, one measurement gap occasion of the per-FR measurement gap pattern and one measurement gap occasion of the per-UE measurement gap pattern are considered colliding if at least one of the following conditions is met:

-	the occasions are fully or partially overlapping in time domain, or

-	the minimal distance between the two occasions is equal to or smaller than X.

The minimal distance between one measurement gap i and another measurement gap j is defined as the minimum of

-	the time difference between the ending point of one occasion of measurement gap i and the starting point of the earliest occasion of measurement gap j after the concerned occasion of measurement gap i, and

-	the time difference between the ending point of one occasion of measurement gap j and the starting point of the earliest occasion of measurement gap i after the concerned occasion of measurement gap j.

At the occasions where two measurement gaps are colliding, UE shall perform measurement in the occasion of the measurement gap with higher priority, and the occasion of the measurement gap with lower priority is considered as a dropped occasion. UE shall be able to conduct reception/transmission from/to the corresponding NR serving cells in the slots in the dropped occasions that are not interrupted according to requirements in clause 9.1.2B.4.

Editor’s note: value of X will be specified based on further agreement in RAN4.

9.1.2B.4	Measurement gap related requirements of concurrent measurement gaps

A slot is considered as interrupted if it is interrupted by an occasion of any of the multiple measurement gaps following the measurement gap interruption requirements in clause 9.1.2, except when it is only interrupted by a dropped occasion.

<End of Change 1>