3GPP TSG-RAN WG4 Meeting # 102-e	R4-2205396

Electronic Meeting, February 21 – March 3, 2022

Source:	ZTE Corporation

Title:	UE Rx/Tx and gNB Rx/Tx timing delay mitigation

Agenda item:	10.21.2.1

Document for:	Discussion

# 1	Introduction

In RAN4 101-e-bis, there were initial discussions on TEG. Some of the WF [1] are copied below.

Issue 1-1-1 The values of timing error margins associated with Rx TEGs for UE/TRP? - Agreements in GTW (1.19) - A single timing error margin is associated with each Rx TEG - FFS if same or different margins are used for measurements with different time stamps - FFS: whether the timing error margin is the same or not for all Rx TEGs if UE/TRP has multiple TEGs Candidate options: - Option A: (Qualcomm, Nokia) - Step #1: RAN4 define multiple candidate values {TE1, TE2, …} in the spec. - Step #2: UE/TRP has multiple Rx TEGs (TEG#1, TEG#2, …) associated with multiple values (M1, M2, …), which means the timing error difference between the measurements within the TEG#i is within the margin Mi where i=1,2,…. - Mi is selected from {TE1, TE2, …} - Mi can be same as or different from each other - Step #3: UE/TRP reports the corresponding margin together with Rx TEG ID during the measurement report. - Step #4: The applicability of reported UE Rx TEG is limited to the measurements contained within the measurement report in which the Rx TEG information is provided, and only to measurements that are tagged with a Rx TEG ID. - Step #5: RRM accuracy requirements will be defined based on the different values {TE1, TE2, …}. - Option C: (Ericsson, OPPO, Intel, Nokia) - Step #1: RAN4 define multiple candidate values {TE1, TE2, …} in the spec. - Step #2: UE/TRP has multiple Rx TEGs (TEG#1, TEG#2, …) associated with the same value M, which means the timing error difference between the measurements within the same Rx TEG is within the margin M. - M is selected from {TE1, TE2, …} - Step #3: UE/TRP reports selected margin M before the measurement (e.g. after receiving the location request) and only report the Rx TEG ID during the measurement report. - Step #4: The applicability of reported UE Rx TEG is limited to the measurements contained within the measurement report in which the Rx TEG information is provided. - Step #5: RRM accuracy requirements will be defined based on the different values {TE1, TE2, …}. - Step #6: FFS whether NW can configure requested margin to UE/TRP based on positioning demand. - Option D (HW): - Step #1: RAN4 define multiple candidate values {TE1, TE2, …} in the spec. - Step #2: LMF selects one value M from {TE1, TE2, …} and indicate to UE/TRP - Step #3: UE/TRP has multiple Rx TEGs (TEG#1, TEG#2, …) associated with the same value M, which means the timing error difference between the measurements within the same Rx TEG is within the margin M. - Step #4: The applicability of reported UE Rx TEG is limited to the measurements contained within the measurement report in which the Rx TEG information is provided, and only to measurements that are tagged with a Rx TEG ID. - Step #5: RRM requirements will be defined based on the different values {TE1, TE2, …}. Issue 1-4-1 How to report transmissions/measurements which cannot be associated with any TEG Candidate options: - Option 1: (CATT, ZTE, Huawei, vivo, Ericsson) - Whether and how to report the measurement without TEG association should be within RAN1/2 scope. - Option 2: (Qualcomm, Ericsson, Nokia, OPPO) - Association of transmissions/measurements to TEGs is optional. - If a measurement or transmitted signal is not associated with a TEG, then no further assumption about relative timing between said measurement/signal and other measurement/signals can be made beyond what is already implied by measurement accuracy requirements in Rel-16.(Qualcomm)

- This paper discusses the issues listed above and also provide our view on the other related issues.

2	Discussions

One of the main issues discussed was the feasibility of TEG. In the RAN1 LS which triggered the work in RAN4, the definition of TE and TEG is given as follows.

Agreement: The following definitions are used for the purpose of discussion of internal timing errors (these terms are not agreed to be included in the specifications): - Tx timing error: From a signal transmission perspective, there will be a time delay from the time when the digital signal is generated at baseband to the time when the RF signal is transmitted from the Tx antenna. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Tx time delay for the transmission of the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Tx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Tx time delay after the calibration, or the uncalibrated Tx time delay is defined as Tx timing error. - Rx timing error: From a signal reception perspective, there will be a time delay from the time when the RF signal arrives at the Rx antenna to the time when the signal is digitized and time-stamped at the baseband. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Rx time delay before it reports the measurements that are obtained from the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Rx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Rx time delay after the calibration, or the uncalibrated Rx time delay is defined as Rx timing error. - UE Tx ‘timing error group’ (UE Tx TEG): A UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing errors within a certain margin. - TRP Tx ‘timing error group’ (TRP Tx TEG): A TRP Tx TEG is associated with the transmissions of one or more DL PRS resources, which have the Tx timing errors within a certain margin. - UE Rx ‘timing error group’ (UE Rx TEG): A UE Rx TEG is associated with one or more DL measurements, which have the Rx timing errors within a certain margin. - TRP Rx ‘timing error group’ (TRP Rx TEG): A TRP Rx TEG is associated with one or more UL measurements, which have the Rx timing errors within a margin. - UE RxTx ‘timing error group’ (UE RxTx TEG): A UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, and one or more UL SRS resources for the positioning purpose, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin. - TRP RxTx ‘timing error group’ (TRP RxTx TEG): A TRP RxTx TEG is associated with one or more gNB Rx-Tx time difference measurements and one or more DL PRS resources, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin.

According to the definition of TE and TEG, it seems that the relevant impacting factors are at least the RF chains which are actually used for radio transmissions and the antenna panels. Theoretically the TE can be measured and calibrated upon manufacturing and testing of the UE or TRP. However, it’s uncertain whether the TE property will be time-variant and how the drifts would be.

Issue 1-1-1 The values of timing error margins associated with Rx TEGs for UE/TRP?

Support Option C which allows the NW to configure requested margin to UE/TRP based on positioning demand. We think that for a certain UE, it is possible for it to perform more accurate positioning with respect to a smaller margin value of the TEG by consuming more power. Since in step 3, UE/TRP already reports selected margin M before the measurement, then the LMF should be aware of all possible TEGs. Selecting one from them under cases when positioning accuracy is critical should be supported.

Support Option C which allows the NW to configure requested margin to UE/TRP based on positioning demand.

Issue 1-4-1 How to report transmissions/measurements which cannot be associated with any TEG

The other issue to be considered is how to define reporting mechanism for signal transmissions or measurements which cannot be associated with any TEG. First of all this issue may not happen, or can be solved by the current approach if a TEG can comprise of a single measurement. Either way, this is clearly RAN2 scope.

How to report transmissions/measurements which cannot be associated with any TEG shall be discussed in RAN2.

3	Conclusion

- Proposal 1: Support Option C which allows the NW to configure requested margin to UE/TRP based on positioning demand.

- Proposal 2: How to report transmissions/measurements which cannot be associated with any TEG shall be discussed in RAN2.

References

- R4-2202684