3GPP TSG-RAN WG4 Meeting #102-e	R4-2205429

Electronic Meeting, 21 Feb. – 3rd Mar. 2022

Agenda Item:	10.19.4.3.1

Source:	Ericsson

Title:	Discussion on CSI reporting for Multi-TRP transmission

Document for:	Discussion

# 1	Introduction

During the last RAN4 meeting’s discussion, it was agreed to introduce CSI requirements for CSI reporting enhancement for m-TRP NCJT transmission. However, which exact requirement from CQI, PMI and RI for single-DCI based multi-TRP scheme has not been decided yet.

- FFS on CQI, PMI, and RI requirement for Single-DCI based Multi-TRP scheme - Option 1: CQI, PMI, RI - Option 2: CQI - Option 3: PMI only

Same time, it is still open whether to have CQI, PMI and RI requirement for Multi-DCI based Multi-TRP scheme.

- FFS on CQI, PMI, and RI requirement for Multi-DCI based Multi-TRP scheme - Option 1: CQI

In this contribution, we shared our views over these two open issues.

# 2	Discussion

## 2.1 CSI reporting for single-DCI based multi-TRP scheme

As we know, for single-DCI based multi-TRP schemes, the UE needs to report two pieces of PMI, RI but one piece of CQI for two different TRPs. From our understanding, the performance of multi-TRP transmission will benefit more from PMI reporting. Meanwhile, considering some prioritization is needed due to the huge workload in FeMIMO for Rel-17, we propose to only introduce requirements for PMI reporting for single-DCI based multi-TRP scheme.

Proposal 1: Introduce requirements for PMI reporting for single-DCI based multi-TRP scheme.

Proposal 2: Not to introduce requirement for CQI and RI reporting for single-DCI based multi-TRP scheme.

## 2.2 CSI reporting for multi-DCI based multi-TRP scheme

From our perspective, we don’t see the need for introducing the requirement for CSI reporting for multi-DCI based multi-TRP scheme, since RAN4 has introduced performance requirement for non-overlapping scheme for multi-DCI based PDSCH transmission. Furthermore, we don’t see the difference from UE receiver’s perspective between single-DCI based CSI reporting and multi-DCI based CSI reporting.

In this case, we don’t support introducing CSI reporting requirement for multi-DCI based multi-TRP scheme for testing the same UE process.

Proposal 3: Not to introduce CSI reporting requirement for multi-DCI based multi-TRP scheme.

# 3	Summary

In summary, we discussed the scope of CSI reporting for both single-DCI based and multi-DCI based multi-TRP transmission. Following proposals have been made:

Proposal 1: Introduce requirements for PMI reporting for single-DCI based multi-TRP scheme.

Proposal 2: Not to introduce requirement for CQI and RI reporting for single-DCI based multi-TRP scheme.

Proposal 3: Not to introduce CSI reporting requirement for multi-DCI based multi-TRP scheme.

# References

[1]	R4-2203090: WF on general and CSI requirement for Rel-17 FeMIMO, Samsung