3GPP TSG-RAN WG4 Meeting#102-e                               R4-2205459

E-meeting,21th Feb – 3rd Mar,2022

Agenda item:	10.16.3.1

Source: 	ZTE Corporation

Title: 	Further Discussion on spectral utilization requirements for FR2-2

Document for:	Approval

Introduction

In the previous RAN4 meetings, there were some discussions on spectral utilization requirements for FR2-2, however there were no much progress been made due to the uncertainty of RF requirement from both UE and BS perspective, in this contribution, we want to shared some further understanding on this issue.

Discussion

Firstly, based on the discussions and agreement for 60GHz co-existence study, it was agreed to reuse the 70GHz ACIR requirement from TR 38.803 and ACLR and ACS are rounded up to the nearest integers.

DL ACIR requirement for 57-71GHz

| DL ACIR | BS ACLR | UE ACS |
| --- | --- | --- |
| ~18.7 dB | 24 dB | 21 dB |

UL ACIR requirement for 57-71GHz

| UL ACIR | BS ACS | UE ACLR |
| --- | --- | --- |
| 13.8 dB | 22 dB | 15 dB |

From the BS Tx perspective, the BS OBUE requirement was also updated similar as FR2-1 with the expectation of lower output power at 52.6-71GHz, therefore 2dB power decreasing was assumed similar as as FR2-1.

From the UE Tx perspective, in the last RAN4 meeting, it seems that following table was agreeable with -5dBm/MHz within 10%*BW adjacent to carrier edge and -13dBm/MHz up to 2*BW from the carrier edge. In other words, SEM requirement for FR2-2 is also defined as FR2-1 without much difference observed.

| ΔfOOB (MHz) | 50 MHz | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ± 0-5 | -5 | -5 | -5 | -5 | -5 | -5 | 1 MHz |
| ± 5-10 | -13 | -5 | -5 | -5 | -5 | -5 | 1 MHz |
| ± 10-20 | -13 | -13 | -5 | -5 | -5 | -5 | 1 MHz |
| ± 20-40 | -13 | -13 | -5 | -5 | -5 | -5 | 1 MHz |
| ± 40-100 | -13 | -13 | -13 | -5 | -5 | -5 | 1 MHz |
| ± 100-120 |  | -13 | -13 | -13 | -5 | -5 | 1 MHz |
| ± 120-160 |  | -13 | -13 | -13 | -5 | -5 | 1 MHz |
| ± 160-200 |  | -13 | -13 | -13 | -13 | -5 | 1 MHz |
| ± 200-400 |  |  | -13 | -13 | -13 | -13 | 1 MHz |
| ± 400-800 |  |  | -13 | -13 | -13 | -13 | 1 MHz |
| ± 800-1600 |  |  |  | -13 | -13 | -13 | 1 MHz |
| ± 1600-2400 |  |  |  |  | -13 | -13 | 1 MHz |
| ± 2400-3200 |  |  |  |  | -13 | -13 | 1 MHz |
| ± 3200-4000 |  |  |  |  |  | -13 | 1 MHz |

Based on the above ACLR/ACS requirement of BS and UE, BS Tx OBUE and [UE SEM] requirement agreed so far, it seems that the existing spectral utilization requirements for FR2-1 could be reused at least for 100MHz and 400MHz with 120kHz SCS.

Meanwhile it should be also noted that spectral utilization requirement is quite important for lots of BS RF core requirement, like total dynamic range or, ACS requirement, Rx intermodulation requirement and ICS requirements. This meeting should be last RAN4 meeting for Rel-17 core part, therefore we need to finalize the spectral utilization requirement this meeting.

Proposal 1: the existing spectral utilization requirements for FR2-1 could be reused at least for 100MHz and 400MHz with 120kHz SCS.

As mentioned in the contribution [6], the main concern is from FFT utilization perspective at least. If too high FFT utilization was used at the end, then spectrum shaping would be impacted due to FFT truncation. In the following table, we will review FFT utilization for FR2-1 and try to figure out how it could be applied for FR2-2 with some scaling approach.

In the following Table 1 and Table 2, it should be noted that FFT utilization would be 75-77% with spectral utilization around 92-95%.

Table 1 : EVM window length for normal CP, FR2, 60 kHz SCS for FR2-1

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (Note) (%) | RB number | FFT utilization | SU |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 1024 | 72 | 36 | 50 | 66 | 0.77 | 0.9504 |
| 100 | 2048 | 144 | 72 | 50 | 132 | 0.77 | 0.9504 |
| 200 | 4096 | 288 | 144 | 50 | 264 | 0.77 | 0.9504 |

Table 2: EVM window length for normal CP, FR2, 120 kHz SCS for FR2-1

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (Note) (%) | RB number | FFT utilization | SU |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 512 | 36 | 18 | 50 | 32 | 0.75 | 0.9216 |
| 100 | 1024 | 72 | 36 | 50 | 66 | 0.77 | 0.9504 |
| 200 | 2048 | 144 | 72 | 50 | 132 | 0.77 | 0.9504 |
| 400 | 4096 | 288 | 144 | 50 | 264 | 0.77 | 0.9504 |

For FR2-2, to the same approach for FR2-1 with similar FFT utilization and spectral utilization, then we could hev following Table 3, 4, 5 for FR2-2 corresponding to 120kHz, 480kHz and 960kHz.

In the following table 5, it should be noted that FFT utilization maintained as 77% similar as FR2-1, then its spectral utilization would be compromised from 95% to 76% which is not reasonable especially considering the transmission bandwidth 1830.47MHz for of 802.11ad 2.16GHz with spectral utilization larger than 84%. In addition, as shown in Table 5, 2000MHz with FFT size 4096 and 165PRBs, then its FFT utilization would be down to 48% which would increase wasting some UE power due to low utilization of FFT size. If FFT size is maintained as 3072, then FFT utilization would be increased to 68% which is close to the 77% and seems reasonable assumptions.

Table 3 : EVM window length for normal CP, FR2, 120 kHz SCS for FR2-2 [the same as table 1 for FR2-1]

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (Note) (%) | RB number | FFT utilization | SU |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 1024 | 72 | 36 | 50 | 66 | 0.77 | 0.9504 |
| 400 | 4096 | 288 | 144 | 50 | 264 | 0.77 | 0.9504 |

Table 4 : EVM window length for normal CP, FR2, 480 kHz SCS for FR2-2

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (Note) (%) | RB number | FFT utilization | SU |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 400 | 1024 | 72 | 36 | 50 | 66 | 0.77 | 0.9504 |
| 800 | 2048 | 144 | 72 | 50 | 132 | 0.77 | 0.9504 |
| 1600 | 4096 | 288 | 144 | 50 | 264 | 0.77 | 0.9504 |

Table 5 : EVM window length for normal CP, FR2, 960 kHz SCS for FR2-2

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (Note) (%) | RB number | FFT utilization | SU |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 400 | 512 | 36 | 18 | 50 | 33/32 | 0.77 | 0.9504 |
| 800 | 1024 | 72 | 36 | 50 | 66 | 0.77 | 0.9504 |
| 1600 | 2048 | 144 | 72 | 50 | 132 | 0.77 | 0.9504 |
| 2000 | 2048 | 144 | 72 | 50 | 132 | 0.77 | 0.76032 |
| 2000 | 4096 | 288 | 144 | 50 | 165 | 0.48 | 0.9504 |
| 2000 | 3072 | 216 | 108 | 50 | 165 | 0.64 | 0.9504 |

Based on the above considerations and and analysis, we made the following proposals for FR2-2 spectral utilization:

Table 5.3.2-3: Transmission bandwidth configuration NRB for FR2-2

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 120 | 66 | 132 | N/A | N/A | N/A |
| 480 | N/A | 66 | 132 | 264 | N/A |
| 960 | N/A | 33/32 | 66 | 132 | 165 |

Conclusions

In this contribution, we want to further considerations on each BS Tx requirements for 60GHz and some proposals and observations are made as following:

Proposal 1: the existing spectral utilization requirements for FR2-1 could be reused at least for 100MHz and 400MHz with 120kHz SCS..

Proposal 2: to propose the following transmission bandwidth configuration for FR2-2:

Table 5.3.2-3: Transmission bandwidth configuration NRB for FR2-2

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 120 | 66 | 132 | N/A | N/A | N/A |
| 480 | N/A | 66 | 132 | 264 | N/A |
| 960 | N/A | 33/32 | 66 | 132 | 165 |

References

- R4-2106113,WF on BS RF TX requirements for 52.6 – 71 GHz, approved.

- R4-2117315,Discussion of spectrum utilization for 52.6-71 GHz, CATT

- R4-2118737,System parameters for a NR band in the range 52.6GHz – 71GHz, Nokia

- R4-2118737,60GHz UE TX, Qualcomm

- R4-2119188,Further discussion on system parameters for 52.6-71GHz, ZTE

- R4-2201598, System parameters for a NR band in the range 52.6GHz – 71GHz, Noted

- R4-2202367, WF on the decision of DL/UL ACIR and BS/UE ACLR/ACS for FR2-2, approved.

- R4-2202366 WF on 60 GHz UE RF, approved.

[9] R4-2200238, 60GHz UE TX, Qualcomm, Noted.