3GPP TSG-RAN WG4 Meeting#102-e                               R4-2205461

E-meeting,21th Feb – 3rd Mar,2022

Agenda item:	10.16.4.2

Source: 	ZTE Corporation

Title: 	Further discussion on BS Rx requirements for 52.6-71GHz

Document for:	Approval

Introduction

In the previous RAN4 meetings, some further progress on BS Rx requirement on 52.6-71GHz was made, however there are still some remaining issue left for further discussions. In this contribution, we want to share some further considerations on this topic.

Discussion

## 2.1. Refsens FRC

In the previous RAN4 meeting, REFSENS FRC was not discussed yet since spectral utilization is finalized yet, however since RAN4#102 meeting would be last RAN4 meeting to complete the core part of Rel-17, in order to speed up the discussion, we want to share some initial suggestions based on the proposed spectral utilization in our companion contribution [6].

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 120 | 66 | 132 | N/A | N/A | N/A |
| 480 | N/A | 66 | 132 | 264 | N/A |
| 960 | N/A | 33/32 | 66 | 132 | 165 |

Based on the above spectral utilization values for candidate channel bandwidth for FR2-2, then RAN4 FRC could defined in the following way:

One minor difference between the following proposed FRC and agreed draft CR [4] in last meeting is additional dedicated FRC might be proposed. For 960kHz, if transmission bandwidth configuration for 400MHz with 960KHz is agreed as 33PRB, then other transmission bandwidth of other CBW with 960kHz is multiple of that one, then single FRC would be enough, however if we have the same situation as FR2-1 for 120KHz where transmission bandwidth configuration NRB is 32 for 50MHz with 120kHz, then additional FRC defined would be better since this would reduce the test time in case of multiple FRC within the CBW are tested simultaneously.

| Frequency Range | BS channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | RB number |
| --- | --- | --- | --- | --- |
| FR2-2 | 100,400 | 120 | G-FR2-A1-3 | 66 |
|  | 400, 800, 1600 | 480 | G-FR2-A1-6 | 66 |
|  | 400 | 960 | G-FR2-A1-7 | 32 |
|  | 800, 1600, 2000 | 960 | G-FR2-A1-8 | 66 |

Table 10.9.3-2: FRC table for REFSENS and ICS requirement

|  | RB number | SCS (KHz) | TBS (Bits) |
| --- | --- | --- | --- |
| G-FR2-A1-2 | 32 | 120 | 2792 |
| G-FR2-A1-3 | 66 | 120 | 5632 |
| G-FR2-A1-6 | 66 | 480 | 5632 |
| G-FR2-A1-7 | 32 | 960 | 2792 |
| G-FR2-A1-8 | 66 | 960 | 5632 |

Proposal 1: the tentative PRB configuration for FR2-2 FRC could be defined as following：

| Frequency Range | BS channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | RB number |
| --- | --- | --- | --- | --- |
| FR2-2 | 100,400 | 120 | G-FR2-A1-3 | 66 |
|  | 400, 800, 1600 | 480 | G-FR2-A1-6 | 66 |
|  | 400 | 960 | G-FR2-A1-7 | 32 |
|  | 800, 1600, 2000 | 960 | G-FR2-A1-8 | 66 |

## 2.2. ACS

For ACS requirements, the only remaining issue for further checking is frequency offset for interfering signal, the pending issue for this problem is transmission bandwidth configuration for 100MHz in FR2-2. As analyzed in the companion contribution [6], it is proposed to specify the following transmission bandwidth for each CBW and SCS combination.

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 120 | 66 | 132 | N/A | N/A | N/A |
| 480 | N/A | 66 | 132 | 264 | N/A |
| 960 | N/A | 33/32 | 66 | 132 | 165 |

If 100MHz with 66 PRB is agreeable, to follow the legacy principle for interfering signal frequency offset to avoid the orthogonality between wanted signal and interfering signal, the required increment in frequency offset can be calculated by applying the following equation:

where

and   is the nominal frequency offset (which is equal to half of the sum of the channel bandwidth of the wanted signal and the channel bandwidth of the interfering signal) and  is the calculated offset increment. In addition, the frequency offset need to be further reduced by 1440 kHz (1 RBs) considering the DFT-s-OFDM interfering signal type.

Based on the above considerations, we could have the following frequency offset for FR2-2.

Proposal 1: to specify the tentative frequency offset for ACS interfering signal for FR2-2 as following:

Table 1: OTA ACS interferer frequency offset for 60GHz

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 100 | 48.58 | 100 MHz DFT-s-OFDM NR signal,120 kHz SCS, 64 RBs |
| 400 | 48.58 |  |
| 800 | 48.62 |  |
| 1600 | 48.58 |  |
| 2000 | 48.62 |  |

## 2.3. RX IMD requirements

For the RX IMD requirement, we have reached on the following agreement for power levels for RX IMD interfering signal, however for frequency offset for interfering signal is still not defined due to the pending spectral utilization for FR2-2.

The interferer levels for general receiver intermodulation (25 dB higher than the OTA REFSENS level) of BS type 2-O should be used for NR operation in 52.6 – 71 GHz range.

In order to speed up the discussion for this requirement and try to finalize the core part in this RAN4 meeting, based on the proposed spectral utilization in the companion contribution [6], similar as ACS requirement, for Rx IMD requirements to follow the legacy principle defined in TR 38.817-02 to ensure that resultant intermodulation resultant intermodulation products fall on the wanted signal edge resource block at least with the lowest SCS defined for the wanted signal channel bandwidth, then we could get the following table for FR2-2.

During the calculation for frequency offset of RX IMD for FR2-2, we want to highlight the following observations:

- For 100MHz and 400MHz, lowest SCS is 120kHz defined and SCS of interfering signal is the same as that of wanted signal, meanwhile for 400MHz with 120kHz, since frequency offset of the CW interfering signal is too close to the lower/upper Base Station RF Bandwidth edge, therefore the modulated interfering signal centre frequency is increased by 5 MHz;

- For 800MHz and 1600MHz, lowest SCS is 480kHz, in addition SCS of interfering signal should be the same as that of wanted signal, therefore interfering signal for these two cases should be updated from 100MHz to 400MHz since it’s the minimum channel bandwidth supported for 480kHz. In addition, because the CW frequency offset is too close to the RF bandwidth edge or within frequency range of wanted signal for 1600MHz, therefore for he modulated interfering signal centre frequency is increased by 10MHz for 800MHz case and 30MHz for 1600MHz case;

- For 2000MHz, lowest SCS is 960kHz,in addition SCS of interfering signal should be the same as that of wanted signal, therefore interfering signal for this should be updated from 100MHz to 400MHz since it’s the minimum channel bandwidth supported for 960kHz similar above two cases. In addition, modulated interfering signal centre frequency is increased by 30MHz for 2000MHz case for the same reason. Here it should be noted that 400MHz with 960kHz, 32 PRB is assumed which is for the simplification of FRC design for FR2-2, we are also open for 33 PRBs with another FRC table defined.

Proposal 2: to specify the tentative frequency offset for Rx IMD interfering signal for FR2-2 as following:

|  | CBW | SCS [KHz] | PRB | GB for wanted signal | interfering signal CBW | GB for interfering signal | Intefering signal offset | CW offset |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2-1 |  |  |  |  |  |  |  |  |
|  | 50 | 60 | 66 | 1240 | 50 | 1240 | 40 | 7.5 |
|  | 100 | 60 | 132 | 2480 | 50 | 1240 | 40 | 6.88 |
|  | 200 | 60 | 264 | 4960 | 50 | 1240 | 40 | 5.64 |
|  | 400 | 120 | 264 | 9920 | 50 | 1960 | 45 | 6.02 |
| FR2-2 | 100 | 120 | 66 | 2480 | 100 | 2480 | 65 | 7.5 |
|  | 400 | 120 | 264 | 9920 | 100 | 2480 | 70 | 6.28 |
|  | 800 | 480 | 132 | 19840 | 400 | 9920 | 225 | 7.54 |
|  | 1600 | 480 | 264 | 39680 | 400 | 9920 | 245 | 7.62 |
|  | 2000 | 960 | 165 | 49600 | 400 | 15680 | 245 | 5.54 |

## 2.4. RX ICS requirements

In the previous RAN4 meeting, there were extensive discussions on FR2-2 ICS requirement, the high level principle was agreed, however the the exact FRC design is still not decided due to the pending spectral utilization. In the following section, we want to share some background how this FRC requirement is designed for Rel-15 NR FR2-1 for the reference of FR2-2 ICS FRC design.

For FR2-1, similar as did in LTE BS, it is proposed to define the UL signal for just 2 users, one being the “wanted” signal and the other one being the “interfering” signal at elevated power. The following wanted/interfering RB allocations are agreed:

Table 2 : PRB allocation of in-channel selectivity for FR2-1 NR

| NR BS channel bandwidth [MHz] | subcarrier spacing[KHz] | Transmission bandwidth configuration NRB | Reference measurement channel | Wanted signal PRB | Interfering signal PRB |
| --- | --- | --- | --- | --- | --- |
| 50 | 60 | 66 | G-FR2-A1-4 | 33PRB | 32PRB |
| 100,200 | 60 | 132,264 | G-FR2-A1-1 | 66PRB | 64PRB |
| 50 | 120 | 32 | G-FR2-A1-5 | 16PRB | 16PRB |
| 100,200,400 | 120 | 66, 132, 264 | G-FR2-A1-2 | 32PRB | 32PRB |

For 50MHz with 60kHz, transmission bandwidth is 66, therefore wanted signal and interfering signal is half of that transmission bandwidth configuration, PRB number for interfering is adjusted due to the DFT-s-OFDM from 33 to 32;

For 100MHz,200MHz with 60kHz, wanted signal and interfering signal 66 is sufficient to test ICS requirements.

For 50MHz with 120kHz,transmission bandwidth is 32, therefore wanted signal and interfering signal is half of that transmission bandwidth configuration;

For 100,200,400MHz with 120kHz, wanted signal and interfering signal 32. is sufficient to test ICS requirements.

Table 3: FRC table for REFSENS and ICS requirement for FR2-1

|  | RB number | SCS (KHz) | TBS (Bits) |
| --- | --- | --- | --- |
| G-FR2-A1-1 | 66 | 60 | 5632 |
| G-FR2-A1-2 | 32 | 120 | 2792 |
| G-FR2-A1-3 | 66 | 120 | 5632 |
| G-FR2-A1-4 | 33 | 60 | 2856 |
| G-FR2-A1-5 | 16 | 120 | 1416 |

Based on the proposed spectral utilization in the following table, we could have the following proposals:

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 120 | 66 | 132 | N/A | N/A | N/A |
| 480 | N/A | 66 | 132 | 264 | N/A |
| 960 | N/A | 33/32 | 66 | 132 | 165 |

For 100,400MHz with 120kHz, wanted signal and interfering signal 32. is sufficient to test ICS requirements which is same as FR2-1.

For 400MHz with 480kHz, transmission bandwidth is 66, therefore wanted signal and interfering signal is half of that transmission bandwidth configuration, PRB number for interfering is adjusted due to the DFT-s-OFDM from 33 to 32;

For 800MHz, 1600MHz with 480kHz, wanted signal and interfering signal 66 is sufficient to test ICS requirements.

For 400MHz with 960kHz,transmission bandwidth is 32, therefore wanted signal and interfering signal is half of that transmission bandwidth configuration e.g. 16PRB;

For 800,1600,2000MHz with 960kHz, wanted signal and interfering signal 32. is sufficient to test ICS requirements.

The above proposals are summarized in the following table 4 and new FRC for ICS requirement is proposed in Table 5 correspondingly.

Table 4 : PRB allocation of in-channel selectivity for FR2-2 NR

| NR BS channel bandwidth [MHz] | subcarrier spacing[KHz] | Transmission bandwidth configuration NRB | Reference measurement channel | Wanted signal PRB | Interfering signal PRB |
| --- | --- | --- | --- | --- | --- |
| 100, 400 | 120 | 132,264 | G-FR2-A1-2 | 32PRB | 32PRB |
| 400 | 480 | 66 | G-FR2-A1-9 | 33PRB | 32PRB |
| 800,1600 | 480 | 132,264 | G-FR2-A1-6 | 66PRB | 64PRB |
| 400 | 960 | 32 | G-FR2-A1-10 | 16PRB | 16PRB |
| 800,1600,2000 | 960 | 66,132,165 | G-FR2-A1-7 | 32PRB | 32PRB |

Table 5: FRC table for REFSENS and ICS requirement for FR2-2

|  | RB number | SCS (KHz) | TBS (Bits) |
| --- | --- | --- | --- |
| G-FR2-A1-2 | 32 | 120 | 2792 |
| G-FR2-A1-3 | 66 | 120 | 5632 |
| G-FR2-A1-5 | 16 | 120 | 1416 |
| G-FR2-A1-6 | 66 | 480 | 5632 |
| G-FR2-A1-7 | 32 | 960 | 2792 |
| G-FR2-A1-8 | 66 | 960 | 5632 |
| G-FR2-A1-9 | 33 | 480 | 2856 |
| G-FR2-A1-10 | 16 | 960 | 1416 |

In addition, in the last RAN4 meeting, there were some discussions on how to define FR2-2 ICS requirement, it was agreed to reuse existing FR2-1 requirements, with scaling according to CHBWs i.e., in the following section, for CBW, the ICS requirement is derived in details. It should be noted that the value should updated to the integer similar as FR2-1.

For 100,400MHz with 120kHz:

-	The wanted signal is specified based on G-FR2-A1-2 FRC which is 32 PRBs wide and 120 kHz SCS, so its value should be:

EISREFSENS_50M + 3 + 10 * log10(32*12*120*1000/(32*12*120*1000)) = EISREFSENS_50M + 3;

-	The interferer signal is specified based on 32 PRBs and 120 kHz SCS, so its value should be:

EISREFSENS_50M + 13 + 10 * log10(32*12*120*1000/(32*12*60*1000)) = EISREFSENS_50M + 13;

For 400MHz with 480kHz:

-	The wanted signal is specified based on G-FR2-A1-9 FRC which is 33 PRBs wide and 480 kHz SCS, so its value should be:

EISREFSENS_50M + 10*log10(400/50)+3 + 10 * log10(33*12*480*1000/(66*12*480*1000)) = EISREFSENS_50M + 9;

-	The interferer signal is specified based on 32 PRBs and 480 kHz SCS, so its value should be:

EISREFSENS_50M +10*log10(400/50)+ 13 + 10 * log10(32*12*480*1000/(66*12*480*1000)) = EISREFSENS_50M + 19;

For 800MHz, 1600MHz with 480kHz:

-	The wanted signal is specified based on G-FR2-A1-6 FRC which is 66 PRBs wide and 480 kHz SCS, so its value should be:

EISREFSENS_50M + 10*log10(400/50)+3 + 10 * log10(66*12*480*1000/(66*12*480*1000)) = EISREFSENS_50M + 12;

-	The interferer signal is specified based on 64 PRBs and 480 kHz SCS, so its value should be:

EISREFSENS_50M +10*log10(400/50)+ 13 + 10 * log10(64*12*480*1000/(66*12*480*1000)) = EISREFSENS_50M + 22;

For 400MHz with 960kHz:

-	The wanted signal is specified based on G-FR2-A1-10 FRC which is 16 PRBs wide and 960 kHz SCS, so its value should be:

EISREFSENS_50M + 10*log10(400/50)+3 + 10 * log10(16*12*960*1000/(32*12*960*1000)) = EISREFSENS_50M + 9;

-	The interferer signal is specified based on 32 PRBs and 480 kHz SCS, so its value should be:

EISREFSENS_50M +10*log10(400/50)+ 13 + 10 * log10(16*12*960*1000/(32*12*960*1000)) = EISREFSENS_50M + 19;

For 800MHz, 1600MHz, 2000MHz with 960kHz:

-	The wanted signal is specified based on G-FR2-A1-7 FRC which is 32 PRBs wide and 960 kHz SCS, so its value should be:

EISREFSENS_50M + 10*log10(400/50)+3 + 10 * log10(32*12*960*1000/(32*12*960*1000)) = EISREFSENS_50M + 12;

-	The interferer signal is specified based on 32 PRBs and 480 kHz SCS, so its value should be:

EISREFSENS_50M +10*log10(400/50)+ 13 + 10 * log10(32*12*960*1000/(32*12*960*1000)) = EISREFSENS_50M + 22;

Based on the above considerations, the proposed requirement for each CBW and SCS combination was summarized as following:

| Frequency Range | BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) (Note 2) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
| FR2-2 | 100,400 | 120 | G-FR2-A1-2 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 32 RB |
|  | 400 | 480 | G-FR2-A1-9 | EISREFSENS_50M + 9 + ΔFR2_REFSENS | EISREFSENS_50M + 19+ ΔFR2_REFSENS | DFT-s-OFDM NR signal, 480 kHz SCS, 32 RB |
|  | 800, 1600 | 480 | G-FR2-A1-6 | EISREFSENS_50M + 12 + ΔFR2_REFSENS | EISREFSENS_50M + 22 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 480 kHz SCS, 64 RB |
|  | 400 | 960 | G-FR2-A1-10 | EISREFSENS_50M + 9 + ΔFR2_REFSENS | EISREFSENS_50M + 19+ ΔFR2_REFSENS | DFT-s-OFDM NR signal, 960 kHz SCS, 16 RB |
|  | 800, 1600, 2000 | 960 | G-FR2-A1-7 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 960 kHz SCS, 32 RB |
| NOTE 1: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. NOTE 2: EISREFSENS_50M is defined in clause 10.3.3. |  |  |  |  |  |  |

Conclusions

In this contribution, further considerations on each BS Rx requirements for 52.6-71GHz and some proposals and observations are made as following:

Proposal 1: to specify the frequency offset for ACS interfering signal for FR2-2 as following:

Table 1: OTA ACS interferer frequency offset for 60GHz

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 100 | 48.58 | 100 MHz DFT-s-OFDM NR signal,120 kHz SCS, 64 RBs |
| 400 | 48.58 |  |
| 800 | 48.62 |  |
| 1600 | 48.58 |  |
| 2000 | 48.62 |  |

Proposal 2: to specify the frequency offset for Rx IMD interfering signal for FR2-2 as following:

|  | CBW | SCS [KHz] | PRB | GB for wanted signal | interfering signal CBW | GB for interfering signal | Intefering signal offset | CW offset |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2-1 |  |  |  |  |  |  |  |  |
|  | 50 | 60 | 66 | 1240 | 50 | 1240 | 40 | 7.5 |
|  | 100 | 60 | 132 | 2480 | 50 | 1240 | 40 | 6.88 |
|  | 200 | 60 | 264 | 4960 | 50 | 1240 | 40 | 5.64 |
|  | 400 | 120 | 264 | 9920 | 50 | 1960 | 45 | 6.02 |
| FR2-2 | 100 | 120 | 66 | 2480 | 100 | 2480 | 65 | 7.5 |
|  | 400 | 120 | 264 | 9920 | 100 | 2480 | 70 | 6.28 |
|  | 800 | 480 | 132 | 19840 | 400 | 9920 | 225 | 7.54 |
|  | 1600 | 480 | 264 | 39680 | 400 | 9920 | 245 | 7.62 |
|  | 2000 | 960 | 165 | 49600 | 400 | 15680 | 245 | 5.54 |

Proposal 3: to specify the tentative ICS requirement in FR2-2 as following:

| Frequency Range | BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) (Note 2) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
| FR2-2 | 100,400 | 120 | G-FR2-A1-2 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 32 RB |
|  | 400 | 480 | G-FR2-A1-9 | EISREFSENS_50M + 9 + ΔFR2_REFSENS | EISREFSENS_50M + 19+ ΔFR2_REFSENS | DFT-s-OFDM NR signal, 480 kHz SCS, 32 RB |
|  | 800, 1600 | 480 | G-FR2-A1-6 | EISREFSENS_50M + 12 + ΔFR2_REFSENS | EISREFSENS_50M + 22 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 480 kHz SCS, 64 RB |
|  | 400 | 960 | G-FR2-A1-10 | EISREFSENS_50M + 9 + ΔFR2_REFSENS | EISREFSENS_50M + 19+ ΔFR2_REFSENS | DFT-s-OFDM NR signal, 960 kHz SCS, 16 RB |
|  | 800, 1600, 2000 | 960 | G-FR2-A1-7 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 960 kHz SCS, 32 RB |
| NOTE 1: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. NOTE 2: EISREFSENS_50M is defined in clause 10.3.3. |  |  |  |  |  |  |

Table 4 : PRB allocation of in-channel selectivity for FR2-2 NR

| NR BS channel bandwidth [MHz] | subcarrier spacing[KHz] | Transmission bandwidth configuration NRB | Reference measurement channel | Wanted signal PRB | Interfering signal PRB |
| --- | --- | --- | --- | --- | --- |
| 100, 400 | 120 | 132,264 | G-FR2-A1-2 | 32PRB | 32PRB |
| 400 | 480 | 66 | G-FR2-A1-9 | 33PRB | 32PRB |
| 800,1600 | 480 | 132,264 | G-FR2-A1-6 | 66PRB | 64PRB |
| 400 | 960 | 32 | G-FR2-A1-10 | 16PRB | 16PRB |
| 800,1600,2000 | 960 | 66,132,165 | G-FR2-A1-7 | 32PRB | 32PRB |

Table 5: FRC table for REFSENS and ICS requirement for FR2-2

|  | RB number | SCS (KHz) | TBS (Bits) |
| --- | --- | --- | --- |
| G-FR2-A1-2 | 32 | 120 | 2792 |
| G-FR2-A1-3 | 66 | 120 | 5632 |
| G-FR2-A1-5 | 16 | 120 | 1416 |
| G-FR2-A1-6 | 66 | 480 | 5632 |
| G-FR2-A1-7 | 32 | 960 | 2792 |
| G-FR2-A1-8 | 66 | 960 | 5632 |
| G-FR2-A1-9 | 33 | 480 | 2856 |
| G-FR2-A1-10 | 16 | 960 | 1416 |

References

- R4-2106114, WF on BS RF RX requirements for 52.6 – 71 GHz, approved.

- TR 38.808

- R4-2203017,WF on BS RF Rx requirements, approved.

- R4-2203015,Draft CR for TS 38.104 on introduction of BS RF Rx requirements for 57-71GHz in section 10.1 – 10.5, CATT, approved.

- R4-2120061,WF on NR extension to 71 GHz (Part 1) - System Parameters, approved.

- R4-22xxxx, Further discussion spectral utilization requirements for FR2-2, ZTE

Annex

| Reference channel | G-FR2-A1-1 | G-FR2-A1-2 | G-FR2-A1-3 | G-FR2-A1-4 | G-FR2-A1-5 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 120 | 120 | 60 | 120 |
| Allocated resource blocks | 66 | 32 | 66 | 33 | 16 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Payload size (bits) | 5632 | 2792 | 5632 | 2856 | 1416 |
| Transport block CRC (bits) | 24 | 16 | 24 | 16 | 16 |
| Code block CRC size (bits) | - | - | - | - | - |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Code block size including CRC (bits) (Note 3) | 5656 | 2808 | 5656 | 2872 | 1432 |
| Total number of bits per slot | 19008 | 9216 | 19008 | 9504 | 4608 |
| Total symbols per slot | 9504 | 4608 | 9504 | 4752 | 2304 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS, additional DM-RS position = pos1 with l0 = 2, l = 11 as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size. NOTE 3: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |