3GPP TSG-RAN WG4 Meeting#102-e                               R4-2205470

E-meeting,21th Feb – 3rd Mar,2022

Agenda item:	10.3.4.2

Source: 	ZTE Corporation

Title: 	Discussion on NTN UE Rx RF requirements

Document for:	Approval

Introduction

In the previous RAN4 meetings, there were extensive discussions on how to define NTN UE RF requirement, in this contribution, we want to share further inputs of remaining issues.

Discussion

  - Rx requirements

One remaining issue left for further discussions is how to define the REFSENS requirement for n256, by reusing the band 65 for reusing the hardware of 65 design as much as possible or with new dedicated design for n256 similar as n24. Both approach could be understandable from the implementation perspective indeed. From our perspective, it seems that with new dedicated band as n24, its performance is 0.5dB better than that of reusing n65, therefore we prefer to define the NTN UE REFSENS requirement by option 2 with new dedicated design.

Proposal 1: to define NTN UE REFSENS requirement for n256 with option 2.

Issue 2-2-5: REFSENS for n256

- Proposals

  - Option 1:  to specify the REFSENS for band n256 as below table

REFSENSE of n256

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n256 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
|  | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |

| n65 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  | -89.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  | -89.3 |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  | -89.4 |

  - Option 2: to specify the REFSENS for band n256 as below table

REFSENSE of n256

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n256 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |

| n24 | 15 | -100.0 | -96.8 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 30 |  | -97.1 |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 |  |  |  |  |  |  |  |  |

Conclusions

In this contribution, we want to share some further views on NTN UE RF requirements and proposals/observations are made as following:

Proposal 1: to define NTN UE REFSENS requirement for n256 with option 2.

References

- ] R4-2120676	WF on NTN UE RF requirements, approved.

- R4-2203036	WF on UE RF requirement for NTN UE, approved.

- R4-2203113	Email discussion summary for [101-bis-e][308] NTN_Solutions_Part3, Noted.