# 3GPP TSG-RAN WG4 Meeting #102-e                                                                        R4-2205494

# Electronic Meeting, 21st Feb – 3rd March, 2022

Title:	draft LS on UE capability and network assistant signalling for CRS interference mitigation in scenarios with overlapping spectrum for LTE and NR

Release:	Rel-17

Work Item:	NR_perf_enh2_Demod

Source:	RAN WG4

To:	RAN WG2

Cc:

Contact Person:

Name:	Jingzhou Wu

E-mail Address:	wujingzhou@chinatelecom.com

Attachments:

1. Overall Description:

Within the Release 17 work item on further enhancement on NR demodulation performance (NR_perf_enh2_Demod), RAN4 has discussed UE capability and network assistant signalling for CRS interference mitigation (CRS-IM) in the following two scenarios with overlapping spectrum for LTE and NR as illustrated in TR 38.833:

- Scenario 1: Serving and neighbouring cells are both operating with dynamic spectrum sharing (DSS) of NR and LTE. The NR UE in the serving cell is configured with lte-CRS-ToMatchAround, and is suffering interference from the LTE CRS of neighbouring cells.

- Scenario 2: Serving cell is operating in NR, and neighbouring cells are operating in LTE. The NR UE in the serving cell is suffering interference from the LTE CRS of neighbouring cells.

As an outcome, following agreements are reached by RAN4:

- From the RAN4 perspective, the following new NR UE capabilities are agreed to be defined for CRS-IM:

- Capability #1: NR UE capable of performing CRS-IM in scenario 1 with 15 kHz SCS, with the granularity TBA.

- Capability #2: NR UE capable of performing CRS-IM in scenario 2 with 15 kHz SCS without new network assistant signalling, with the granularity TBA.

- Capability #3: NR UE capable of performing CRS-IM in scenario 2 with 15 kHz SCS with new network assistant signalling to be introduced in Rel-17, with the granularity TBA.

- Capability #4: NR UE capable of performing CRS-IM in scenario 2 with 30kHz SCS, with the granularity TBA.

- For UE supporting Capability #2, it can also support Capability #3.

- The above capabilities are optional for UE to report.

- To perform CRS-IM, RAN4 agree that the UE should have the following default network configuration assumptions for each neighbour LTE cell:

- For scenario 1, the CRS port number is same with that of the serving cell which is indicated in the existing IE RateMatchPatternLTE-CRS. For scenario 2, the CRS port number is 4.

- Network-based CRS interference mitigation (i.e., CRS muting) is not enabled by crs-IntfMitigConfig in TS 36.331 for scenario 1 and 2.

- For scenario 1, channel bandwidth and centre frequency is same with that of the serving cell, which is indicated in the existing IE RateMatchPatternLTE-CRS.

- For scenario 1, MBSFN configuration is same as that of serving cell if it is indicated in the existing IE RateMatchPatternLTE-CRS. For scenario 2, MBSFN is not configured.

- With the above default network configuration assumptions, RAN4 agree that for UE supporting Capability #1 and Capability #2, the UE can perform CRS-IM without Rel-17 new RRC network assistant signalling in scenario 1 and 2, respectively. Meanwhile, new RRC based network assistant signalling is agreed to be introduced to assist CRS-IM in Rel-17, and the Rel-17 new RRC signalling are optional to be indicated to UE supporting Capability #1 and Capability #2.

- Regarding the content of the Rel-17 new RRC network assistant signalling, for each neighbour LTE cell, RAN4 reach following agreements:

- Centre frequency

  - For scenario 1, the centre frequency can be optionally signalled to the UE, if the above default centre frequency assumption is not valid.

  - For scenario 2, the centre frequency can be optionally signalled to UE.

- Channel bandwidth

  - For scenario 1, the channel bandwidth can be optionally signalled to the UE, if the above default channel bandwidth assumption is not valid.

  - For scenario 2, the LTE channel bandwidth can be optionally signalled to UE supporting Capability #2, and should be signalled to UE supporting Capability #3 but not supporting Capability #2.

- CRS port number

  - For scenario 1 and scenario 2, the CRS port number can be optionally signalled to the UE, if the above default CRS port number configuration assumption is not valid

- Cell ID

  - For scenario 1 and scenario 2, the physical Cell ID can be optionally signalled to UE.

- CRS muting

  - For scenario 1 and scenario 2, the configuration of CRS muting can be optionally signalled to the UE if the above default CRS muting assumption is not valid.

- MBSFN configuration

  - For scenario 1 and scenario 2, the MBSFN subframe configuration can be optionally signalled to the UE if the above default MBSFN configuration assumption is not valid.

- The above information is indicated [per serving cell], and information for maximum of [8] LTE neighbour cells [for each serving cell] can be indicated.

2. To RAN WG2 group.

ACTION: RAN4 respectfully request RAN2 to take the above information into account and design the corresponding UE capability and network assistance signalling for CRS-IM in scenarios with overlapping spectrum for LTE and NR.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #103-e 	15 May - 27 May 2022	                                  Online

TSG-RAN WG4 Meeting #104 	22 Aug - 26 Aug 2022