3GPP TSG-RAN4 Meeting # 102-e	R4-2205498

Electronic Meeting, 21st February – 4th March

Source:	China Telecom

Title:	On CSI requirements for UE MMSE-IRC receiver for inter-cell interference suppression

Agenda Item:	10.12.2.1.2

Document for:	Discussion

- Introduction

In the RAN4#101-bis-e meeting, we discussed the test parameters and test requirement definition for CQI reporting requirements for UEs supporting MMSE-IRC receiver for suppressing inter-cell interference, and the WF was agreed in [1].

In this paper, our views on the remaining open issues are given.

- Discussion

Test setup methodology for signal power

Status in [1]:

  - Option 1: Define test based on SINR

  - Option 2: Define test based on SNR

In our understanding, when executing the CQI test with MMSE-IRC under ICI scenario, the test is mainly divided in 2 parts:

Part#1: Test the UE throughput under both ICI and white noise added, with SINR = x dB, in which case the SNR is smaller than x dB.

Part#2: Test the UE throughput with only white noise added, with SINR = SNR = x dB, which means the white noise power will be increased for test part#2.

We observe that the SNR will be changed while executing the CQI test with MMSE-IRC under ICI scenario, therefore, we think SINR based requirement definition is more preferable from our point of view.

Observation 1: The SNR will be changed while executing the CQI test with MMSE-IRC under ICI scenario.

Proposal 1: SINR based requirement definition is more preferable.

- Conclusions

The following proposals were given for CQI reporting requirements:

Observation 1: The SNR will be changed while executing the CQI test with MMSE-IRC under ICI scenario.

Proposal 1: SINR based requirement definition is more preferable.

- References

R4-2203009, WF on CQI reporting requirements for inter-cell interference MMSE-IRC, Ericsson, RAN4 #101-bis-e, Jan 2022.